3GPP TSG RAN WG2#128	R2-2410124

Orlando, US, Nov 18th – 22nd, 2024

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on AIoT functionalities

Document for:	Discussion, Agreement

Introduction

This paper will discuss some of the issues of stage-2 AIoT functionalities as follows:

- Segmentation;

- energy status reporting;

- AS ID;

- L2 repetition.

Discussion

Issue-1: segmentation

In RAN2#127bis meeting, RAN2 has agreed to study segmentation. The related agreements are given below.

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

Based on above agreements, the segmentation functionality is in MAC, and buffering for A-IoT segmentation functionality is in the upper layer. Sequence number for A-IoT segmentation is not supported since all the transmission will be received in order due to no HARQ and ARQ mechanism. To support the segmentation functionality, it has been agreed to introduce indication on whether the data is segmented and whether it is last segment, but segment number and the number of segments is not supported. Then an open issue is how receiver/transmitter knows if a part of segments is lost. For D2R transmission, as the scheduling information including time-frequency resource is determined by reader, based on whether reader successfully receives the transmission on the given time-frequency resource, reader can send a feedback with ACK or NACK value to device, so that device could decide whether to retransmit previous segment or transmit a new segment for next  transmission. If reader still does not receive the segment successfully after multiple retransmission, MAC can send a failure indication to upper layer so that upper layer could be aware of the lost segment. For R2D transmission, it is also expected that device could provide the ACK/NACK information to facilitate reader’s scheduling. As the transmission resource is determined by transmitter (i.e. reader), there may be a case that reader performs a R2D transmission but device doesn’t receive it or even doesn’t know reader has performed a R2D transmission, in which case device would not provide feedback information to the reader. From reader’s perspective, if it does not receive any feedback for a time duration, it could consider that device has failed to receive the segment, then it can retransmit previous segment rather than transmit the next new segment by its implementation. In our view, whether segmentation functionality is needed for both or either of R2D and R2D transmission may depend on the packet size, link coverage, etc. We can make conclusion on this after receiving reply from SA2. From RAN2’s perspective, segmentation functionality can work for both directions.

Regarding on the indication on whether the data is segmented and whether it is last segment, we think 2bits are needed, i.e. one bit for segmented flag and one bit for last segment flag.

RAN2 discusses on the need of segmentation functionality after more progress is made in SA2.

Introduce ACK/NACK feedback for both R2D and R2D directions.

Use two bits for segmented indication and last segment indication, respectively.

Issue-2: energy status reporting

In last RAN2 meeting, the follow agreements were achieved regarding energy status reporting.

Agreements on energy bit indication

- Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled.

Based on above, RAN2 has agreed to support 1bit energy status indication. The main use case for energy status is that device can indicate to reader whether it can support the follow-up messages due to energy issue, so that the reader may need such information to decide whether to further schedule the devices or hold on the current transmission. Given that the transmission/reception of follow-up messages in different cases would require different device energy, e.g. in some cases the following-up message would last for a relative long time which would use more device energy, while in other cases the following-up message will be quickly completed, and such information is known by reader, it can be up to reader to decide whether energy status reporting is needed.

Energy status reporting can be enabled in an on-demand way, e.g., requested by the reader.

Issue-3: AS ID

In RAN2#127bis meeting, the following agreements were made regarding AS ID.

Agreements on AS ID - RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. - RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. - From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. - RAN2 will aim to define one common design for all RA procedures (if technically possible)

Based on above, RAN2 understands that the AS ID can be used at least for purpose of D2R scheduling and R2D reception. Regarding how to determine the AS ID, for CBRA, if option1 is adopted, the random ID transmitted in msg1 is used as the AS ID after the reader echoes the ID by sending msg2. However, since the random ID is generated by device, post-collision between devices whose contention is resolved already is possible. there is no such issue in option 2. For CFRA, as random ID is not used, only option 2 is feasible.Based on above, we prefer to adopt option 2 for both CBRA and CFRA.

For both CBRA and CFRA, reader assigns the “AS ID” to the device via Msg2.

Issue-4: L2 repetitions

In RAN2#127 meeting, some companies  proposed to support a L2 repetition mechanism, in order to handle the case when the message is not successfully received by the device/reader. While some other companies think such repletion mechanism can be handled by implementation, e.g., for the R2D case, when the reader is aware that the message is not successfully received by the AIoT device, it can transmit the same message again by implementation.

In our view, we think we don’t need to support a mechanism of L2 repetition. For R2D case, the reader can handle the case of transmission when reader is aware of message is not successfully received by the AIoT device, e.g., if reader does not receive any response or feedback for a time duration, thus it can trigger another transmission of the same message again by implementation. For the D2R case, the reader can also trigger the paging procedure when the expected information is not received by the reader during the inventory procedure or the command procedure.

No need to support L2 repetition mechanism, as it can be handled by reader implementation.

- Conclusion

We have the following proposals:

- RAN2 discusses on the need of segmentation functionality after more progress is made in SA2.

Introduce ACK/NACK feedback for both R2D and R2D directions.

Use two bits for segmented indication and last segment indication, respectively.

Energy status reporting can be enabled in an on-demand way, e.g., requested by the reader.

For both CBRA and CFRA, reader assigns the “AS ID” to the device via Msg2.

No need to support L2 repetition mechanism, as it can be handled by reader implementation.