3GPP TSG-RAN WG2#128	R2-2410145

Orlando, USA, Nov. 18th – 22nd, 2024

Source: 	Fujitsu

Agenda item:	8.5.2

Title: 	Discussion on on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127bis, RAN2 has discussed the on-demand SSB SCell operation, and the following agreements were made [1]:

Agreements on OD-SSB SCell - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we would like to discuss and provide our views on the on-demand SSB SCell operation from RAN2 perspective.

# Discussion

## On-demand SSB SCell

To reduce network energy consumption, on-demand SSB SCell should not be used as a normal cell. We assume the SCell is activated to handle user burst traffics in connected mode, and when the NW is expected the traffic is low, the NW should stop on-demand SSB transmissions and deactivate/release the SCell. Given the above use-case, on-demand SSB SCell should not be used as a PCell. Otherwise, the UEs may camp on this cell during on-demand SSB transmission periods and consequently the UE suddenly loses DL synchronisations due to stopping of on-demand SSB. It may impact on idle/inactive UE behaviours.

Observation 1:	Even if the UE can receive SSB in the cell (always-on SSB or on-demand SSB), the cell should not be allowed to access from idle/inactive mode UEs as a PCell.

Therefore, for on-demand SSB SCell, some barring mechanism for idle/inactive UEs is used. In our understanding, because the UE in idle/inactive mode behaviours are the same, the Rel-19 NES UEs also should not camp on the on-demand SSB SCell as well as legacy UEs. Thus, an existing barring mechanism (e.g., MIB barring) can work for this purpose.

Proposal 1:	An existing mechanism (e.g., MIB barring) indicates the on-demand SSB SCell is not used as a PCell for idle/inactive mode UEs. No specification is changed.

### On-demand SSB SCell configuration

Identification of on-demand SSB SCell:

From the RAN1 agreements, the SCell with on-demand SSB transmission would have different SSB transmission mode from the legacy SCell, i.e., the SCell with on-demand SSB transmission may have no SSB transmission or have the always-on SSB transmission (e.g., with a long periodicity than usual) before the on-demand SSB transmission. In order for the UE not to measure the SCell with no always-on SSB to save the UE’s power, or for the UE to measure the SCell with always-on SSB with the relaxed measurement requirements, the UE may need to know which SCell is the SCell with on-demand SSB. Therefore, it is proposed that the gNB should indicate to the UE if the configured SCell is an SCell with on-demand SSB.

Proposal 2: 	The gNB indicates to the Rel-19 NES UE if the configured SCell is an SCell with on-demand SSB transmission.

NCD SSB configuration:

As agreed in RAN1#116-bis, it is up to gNB implementation whether always-on SSB on the cell is cell-defining SSB (CD SSB) or non-cell-defining SSB (NCD SSB). It means the gNB can transmit always-on NCD SSBs in the cell.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

However, NCD SSB is defined that it has the same properties as the corresponding CD SSB as shown in TS 38.331.

nonCellDefiningSSB

If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB.

The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

So far, RAN1 has not yet concluded whether on-demand SSB is limited to NCD or not. Depending on the RAN1 conclusion, it would be some further RAN2 impacts on the current NCD SSB definitions in RRC due to the above relationship between CD and NCD SSB. Table.1 shows potential RAN2 impacts on the possible SSB type combinations between always-on SSB and on-demand SSB.

Table.1 - Possible SSB type combinations between always-on SSB and on-demand SSB

| Combination no. | Always-on SSB | On-demand SSB | Can NCD SSB use the same properties of corresponding CD SSB? |
| --- | --- | --- | --- |
| 1 | N/A | CD | - |
| 2 |  | NCD | No |
| 3 | CD | CD | - |
| 4 |  | NCD | Yes |
| 5 | NCD | CD | Maybe Yes |
| 6 |  | NCD | No |

From Table.1, at least for the combinations 2 and 6, the on-demand SSB cannot refer the same properties of corresponding CD SSB, because there is no corresponding CD SSB in the cell. For the combination 5, it is not straightforward way, but it is possible that the on-demand SSB (CD-SSB) is used as a corresponding SSB. On the other hand, considering network energy saving efficiency, it might be problematic because the periodicity of NCD SSB shall not be less than the periodicity of the serving cell’s CD SSB.

Observation 2:	If the on-demand SSB is NCD SSB in Case #1 (no always-on SSB in the cell), the UE may not identify the NCD on-demand SSB properties.

Observation 3:	If both the always-on SSB and the on-demand SSB in the cell are NCD SSB, the UE may not identify the NCD on-demand SSB properties.

In order to resolve the issues, two options are considered:

- Option1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option2: NCD SSB configuration will be enhanced to include all SSB properties.

If RAN1 agrees the on-demand SSB is limited to NCD SSB and existing NCD SSB configuration is reused, the always-on SSB may be limited to CD SSB only (combination 4). It seems contradiction with the previous RAN1 agreement “whether it is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not”. Option 2 can support any combinations in Table.1 but specification impact is larger than Option 1.

Proposal 3:	RAN2 to discuss and select one of the following options, in case there is no corresponding CD SSB in the cell.

- Option 1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option 2: NCD SSB configuration will be enhanced to include all SSB properties.

### L3 measurements

Framework of L3 measurements:

In RAN2#127bis meeting, the following two options were discussed for L3 measurements framework but RAN2 have not agreed because it is not clear for RAN4 impact.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

For option 1, it is less impact from RAN4 perspective, because L3 measurement requirement in TS 38.133 is defined based on the SMTC configuration, not for the SSB configuration. From RAN2 perspective, although on-demand SSB has different (typically short) periodicity from that of always-on SSB, the same measurement mechanism can be applied by configuring SMTC window for on-demand SSB periodicity. Since RAN2 already supports multiple SMTC lists (e.g., smtc3list, smtc4list), it would be small impact on the specifications in RAN2.

For option2, it may be beneficial if on-demand SSB pattern needs full flexibility. For example, at least applicable value for on-demand SSB periodicity can be indicated by a new MAC CE for on-demand SSB transmission activation. As the SSB periodicity is set by one of {ms5, ms10, ms20, ms40, ms80, ms160}, then in case of option 1, at worst the NES UE should be pre-configured with 6 SMTCs for on-demand SSB measurements. However, even if on-demand SSB needs some flexibility for quick DL synchronisations and/or L1/L3 measurements, in practical, at most 2 additional measurement configurations would be sufficient (i.e., for scenarios 2/2A).

Observation 4:	In practical, it is enough to support at most 2 additional measurement configurations for on-demand SSB (i.e., for scenarios 2/2A).

On the other hand, RAN4 introduced “shortMeasInterval-r18” indicates the UE supports to use SSB periodicity instead of SMTC periodicity in Rel-18 RRM enhancement [2]. But, in our understanding, it is applied only for L1-RSRP measurements on unknown SCell activation hence it is not just reused for on-demand SSB L3 measurements. RAN4 still uses SMTC-based L3 measurements. In addition, if this UE capability is reused for on-demand SSB L3 measurements, it imposes the UE on supporting this feature as a prerequisite of on-demand SSB.

| shortMeasInterval-r18 Indicates whether the UE supports using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement and performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation. UE is required to meet the shortened SCell activation delay requirement in TS 38.133 [5] if the feature is supported. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Considering the above, there is no clear motivation to support such a flexibility for L3 measurements, and option 1 is less impact on other WGs with reasonable RAN2 impact. So, we prefer to support option 1 for L3 measurements framework of on-demand SSB. The limited number of SMTC window for on-demand SSB can be pre-configured by RRC at the SCell addition. The UE can apply the SMTC window for on-demand SSB indicated by MAC CE or RRC when the on-demand SSB transmission is activated.

Proposal 4:	RAN2 to support “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 5:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

L3 measurements for Case#2:

In RAN4#112bis meeting, RAN4 agreed OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered [3]. However, there are still many options how the UE measures the always-on SSB and/or OD-SSB when both SSB types are transmitted in Case#2.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In RAN1#118bis meeting, time/frequency domain relation between always-on SSB and on-demand SSB was discussed. But it concluded that it needs more study in RAN1.

Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

If always-on SSB and on-demand SSB are transmitted on the same frequency location, it could be combined both measurement results, whereas it is difficult to combine if these are transmitted on the different frequency location each other. Therefore, it is premature to discuss how to obtain L3 measurement results for Case#2 because it depends on time/frequency domain relation between always-on SSB and on-demand SSB.

Observation 5:	RAN1/4 are still discussing the always-on SSB and on-demand SSB time/frequency domain relation for Case#2.

Proposal 6:	For Case#2, RAN2 to wait for RAN1/4 progress for L3 measurements.

L3 measurements for neighbour cell:

In RAN4#112bis meeting, RAN4 also agreed the legacy measurement framework is applied for neighbour cell measurement [3].

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. This agreement can be revisited if it is not aligned with further agreement in other WGs.

From RAN2 perspective, as the UE does not know the SSB transmission status (on-demand or broadcasted) of neighbour cells, it is difficult to measure the on-demand SSB per neighbour cell. Hence, it is reasonable to follow the above RAN4 agreement as is.

Proposal 7:	For L3 measurements, the legacy measurement framework is applied for neighbour cell measurements.

# Conclusion

In this contribution, we have some discussions about on-demand SSB SCell operations, and the following proposals are made:

General aspect:

Proposal 1:	An existing mechanism (e.g., MIB barring) indicates the on-demand SSB SCell is not used as a PCell for idle/inactive mode UEs. No specification is changed.

Identification of on-demand SSB SCell:

Proposal 2: 	The gNB indicates to the Rel-19 NES UE if the configured SCell is an SCell with on-demand SSB transmission.

NCD SSB configuration:

Proposal 3:	RAN2 to discuss and select one of the following options, in case there is no corresponding CD SSB in the cell.

- Option 1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option 2: NCD SSB configuration will be enhanced to include all SSB properties.

Framework of L3 measurements:

Proposal 4:	RAN2 to support “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 5:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

L3 measurements for Case#2:

Proposal 6:	For Case#2, RAN2 to wait for RAN1/4 progress for L3 measurements.

L3 measurements for neighbour cell:

Proposal 7:	For L3 measurements, the legacy measurement framework is applied for neighbour cell measurements.

# References

- R2-2409211, Report from session on V2X/SL, R19 NES and MOB

- TS 38.306 v18.3.0

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1