3GPP TSG-RAN2 Meeting # 128	R2-2410160

# Orlando, USA,  18 – 22 November, 2024

Agenda item:	8.15

Source: 	Huawei, HiSilicon

Title: 	Discussion on the remaining issues for BDS B2b

Document for:	Discussion and Decision

1. Introduction

Regarding the introduction of BDS B2b signal, in the previous RAN2 meeting, the following agreements have been made:

Agreements:

Introduce new parameters for B2b signal in the following affected IEs:

AlmanacReducedKeplerianSet: Add one bit to indicate health state of B2b_I signal.

AlmanacMidiAlmanacSet: Add one bit to indicate health state of B2b_I signal.

BDS-ClockModel2: Add one bit to indicate group delay differential between BDS B2b and B3I signal.

GNSS-SignalID: Use one value to indicate the B2b I signal.

GNSS-SignalIDs: Use one value to indicate the B2b I signal.

FFS: Iod: Add one bit B2b signal in ‘GNSS to iod Bit String(11) relation’ table.

In this contribution, we continue the discussion on the remaining issues.

2. Discussion

According to the ICD, within the nav message of BDS B2b, the following fields have been added for B2b. Basically, for the support of BDS B2b in A-GNSS, what we need to do is to carry these information elements defined in the nav message within the LPP message. We examine the spec impacts of each parameter within the nav message one by one

| Nav message elements | Description for the element | LPP spec impact |
| --- | --- | --- |
| PRN | identity for the satellite | Corresponds to the IE SV-ID in the current LPP spec. legacy IE can be reused but reference needs to be added |
| Message Types | MesType is used to identify the message types of the B-CNAV3 frames | No change needed. Message type is for the format delivered by the satellite link directly. While for GNSS assistance data delivery by LPP transport, it is not needed |
| SOW | Seconds of week | Can be supported with the GNSS-SystemTime field. However, needs to be converted to gnss-TimeOfDay and gnss-DayNumber with the counting starting from January 1st 2006 00:00:00 UTC (NTSC). |
| WN | Week number | Can be supported with the GNSS-SystemTime field. However, needs to be converted to gnss-TimeOfDay and gnss-DayNumber with the counting starting from January 1st 2006 00:00:00 UTC (NTSC). |
| Clock Correction parameters | Includes toc, a0, a1, a2 | Can reuse the existing fields in BDS-ClockModel2 with the range of different parameters to be very different. Better to create a new IE to hold these parameters with new ranges |
| Group delay differential parameters | T_GDB2bI | Can be added to the BDS-ClockModel2 clock model |
| Ephemeris parameters |  | Can reuse the existing fields in NavModel-BDS-KeplerianSet2. But description needs to be updated that when the signal type is BDS B2b, the field bdsIODE is ignored |
| Ionospheric Delay Correction Model Parameters | The BeiDou Global Ionospheric delay correction Model (BDGIM) contains nine parameters which are used to correct the effect of ionospheric delay for the single frequency user | Can reuse klobucharModel2Parameter in the current LPP |
| Midi Almanac Parameters | Contains 14 parameters | Can reuse the existing field AlmanacMidiAlmanacSet but description needs to be updated. Same discussion on the Health field. This is aligned with the agreement in the last R2 meeting |
| Reduced Almanac parameters |  | Can reuse the existing field AlmanacReducedKeplerianSet For the indication of the SV health, previous BDS system reuse the bits for the GPS system redAlmL1/L2/L5health. However, with the introduction of BDS B2b, the number of bits is not enough anymore. We need to add a new bit corresponding to the 5th bit in the Health field. This is aligned with the agreement in the last R2 meeting |
| Earth Orientation Parameters |  | Can reuse the IE GNSS-EarthOrientationParameters |
| BDT-UTC Time Offset Parameters |  | Can reuse the IE UTC-ModelSet2 but the DN field is no longer 4 bits long. Clarify in the field description that on the 3 bits from LSB are used. Legacy BDS may also have the same issue. |
| BDT-GNSS Time Offset Parameters |  | Can reuse the IE GNSS-TimeModelElement |
| Satellite Health Status | Satellite Health Status (HS) is an unsigned integer with a length of 2 bits, which indicates the health status of the transmitting satellite. | Can reuse the existing IE svHealth. But clarify that bit 1 and bit 2 are used for B2b signal |
| Satellite Integrity Status Flag | The specific definitions of the signal integrity status flag parameters will be published in a future update of this ICD. | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |
| Signal In Space Accuracy Index | The signal in space accuracy describes the predictive accuracy of the orbital parameters and clock correction parameters broadcast in the navigation message. It contains the along track and cross-track accuracy of the satellite orbit (SISAoe) and the satellite orbit radial and satellite clock correction accuracy (SISAoc). The specific definitions of the signal in space accuracy index parameters will be published in a future update of this ICD | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |
| Signal In Space Monitoring Accuracy Index | The estimated error of the signal in space accuracy is described by the zero-mean Gaussian distribution model. The signal in space monitoring accuracy (SISMA) is the variance of the Gaussian distribution, which is indicated by the signal in space monitoring accuracy index (SISMAI). The specific definitions of the signal in space monitoring accuracy index parameters will be published in a future update of this ICD | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |

Based on the discussion above, we propose the following:

Proposal1: For different parameters in the navigation message defined in clause 6 and 7 of ICD, support them by the following:

- Ephemeris parameters: Reuse the existing IE NavModel-BDS-KeplerianSet2 but the field bdsIODE is ignored

- BDT-UTC Time Offset Parameters: reuse the IE UTC-ModelSet2. Clarify in the field description that on the 3 bits from LSB are used. Legacy BDS may also have the same issue.

- Satellite Health Status: reuse the existing IE svHealth. But clarify that bit 1 and bit 2 are used for B2b signal

- No change is needed for all the parameters related to integrity in the nav message.

3. Conclusion

In this paper, we discuss the current issues within the LPP spec for the introduction of BDS B2b signal. We propose the following:

Proposal1: For different parameters in the navigation message defined in clause 6 and 7 of ICD, support them by the following:

- Ephemeris parameters: Reuse the existing IE NavModel-BDS-KeplerianSet2 but the field bdsIODE is ignored

- BDT-UTC Time Offset Parameters: reuse the IE UTC-ModelSet2. Clarify in the field description that on the 3 bits from LSB are used. Legacy BDS may also have the same issue.

- Satellite Health Status: reuse the existing IE svHealth. But clarify that bit 1 and bit 2 are used for B2b signal

- No change is needed for all the parameters related to integrity in the nav message.

4. References

[1] BDS B2b signal ICD, http://www.beidou.gov.cn/xt/gfxz/202008/P020230516558683155109.pdf