3GPP TSG-RAN WG2 Meeting #128											R2-2410163

Orlando, USA, Nov.18th–22nd, 2024

Agenda item:		8.5.2

Title: 	On-demand SSB SCell operation in connected mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The following agreements have been achieved in the past meetings [1] [2]:

RAN2#127 Agreements

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

RAN2#127bis Agreements

Agreements on OD-SSB SCell

1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this paper, we share some understanding on the remaining open issues.

- Discussion

  - Configuration of on-demand SSB

Understanding on the existing SCell Addition/Modification and activation/deactivation:

SCell can be added and modified via SCell addition/modification with the initial state set to set as “deactivated”. When a SCell is added but deactivated, UE will still measure SSB, if configured, but would not monitor PDCCH.

Figure 1. Understanding on case#1

As we agreed last meeting that For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated, the SCell addition/modification with the initial state set to “deactivated” cannot be directly apply for on demand SSB case#1. Additional indication is needed to inform UE that there is no need to perform SSB measurements.

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Figure 2: Understanding on case#2

For case#2, we understand the always on SSB can be SSB with larger periodicity for energy saving purpose while the SSB after on demand triggering would be more frequently transmitted. In this case, if UE does not measure the always on SSB, the case#2 would be the same as case#1 and NW would not be aware of the radio quality of the SCell and there would be no gain compared to case#1 while more power consumption is required for the always on SSB transmission.

However, if UE measure the always on SSB before on demand SSB triggering, NW can get the measurement report in advance and decide whether to trigger on demand SSB transmission and activate the SCell.

Thus, we see benefits if UE measure the always on SSB before on demand SSB transmission is activated for case#2.

Proposal 1: For case#2, UE should measure the always on SSB before on demand SSB transmission is activated.

Then in case#2, the SCell addition/modification with the initial state set to “deactivated” can be applied, with which UE would measure the always on SSB but not monitor the PDCCH. And the SSB transmission update is expected upon on demand SSB transmission triggering.

Observation 2: For case#2, the SCell addition/modification with initial state set to “deactivated” for the corresponding SCell can be reuse while UE needs to be aware of the SSB transmission update upon SSB transmission triggering.

Proposal 2a: For on demand SSB configuration, the existing SCell addition/modification is reused with the initial state set to “deactivated” for the corresponding SCell for both case#1 and case#2.

Proposal 2b: For case#1, an additional indication should be introduced to indicate that UE does not need to measure SSB for the corresponding SCell.

  - L3 Measurement

An LS [3] have been sent from RAN4 to RAN2 for L3 measurements:

---------------------------------------------------------------LS from RAN4---------------------------------------------------------------

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion:

Agreements:

- RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption:

  - L1-RSRP for serving cell

  - L1-SINR

  - BFD

  - CBD

  - SCell activation

  - TCI state switching

  - NR intra-frequency L3 measurement

  - NR inter-frequency L3 measurement

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

---------------------------------------------------------------LS from RAN4---------------------------------------------------------------

As shown in the following figure, four options can be considered for the handling of L3 measurements for case#2:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Introduce additional SMTC for OD-SSB, UE measures OD-SSB in the additional SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

Figure 3: L3 measurements for case#2

Regarding the questions from RAN4, it is highly related to the solution we select while option 2/3/4 would all have RAN4 impact while option 1 would be the simplest way to go.

For option 2 and 3, RAN4 has to define L3 measurement requirement based on SSB pattern. For option 4, additional SMTC would be introduced and RAN4 needs to consider the impact.

Proposal 3: RAN2 to discuss the following options for L3 measurement handling in case#2 and inform RAN1/4:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Introduce additional SMTC for OD-SSB, UE measures OD-SSB in the additional SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

  - On-demand SSB procedure

The on demand SSB procedure can be triggered when UL or DL traffic arrives and more activate SCells are needed. Thus we understand both UE and NW trigger can be considered.

Proposal 4: The on demand SSB procedure can be triggered by NW or UE.

    - On demand SSB procedure triggered by NW

Issue1: On demand SSB activation MAC CE design

As we agreed last meeting that New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE, we understand a MAC-CE as follows can be introduced with each bit indicating the transmission of SSB for each SCell.

Figure 4: A example of on demand SSB activation MAC CE of four octets

For case#1 with Scenario#2, the above example of SSB transmission indication MAC CE would be sufficient and UE would start to measure the SSB if the bit for the corresponding SCell is set to 1.

Proposal 5a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

For case#2 with Scenario#2, two options can be considered:

- Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

Option 1: Two sets of SSB configuration via RRC with 1 bit per SCell in MAC CE to switch

- Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 5b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

--Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

--Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Issue 2: On demand SSB deactivation

There has been discussion on how to indicate the transmission of on demand SSB while we have not mentioned whether we can deactivate the on demand SSB transmission. Per our understanding, the deactivation via MAC CE should be also supported.

For case#1, UE would stop measurements on the SSB upon reception of the deactivation. For case#2, UE is informed of the SSB transmission update, e.g. periodicity update, based on the deactivation indication.

Proposal 6: On demand SSB deactivation via MAC CE should be supported:

--For case#1, UE stop SSB measurements upon reception of the deactivation.

--For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

    - On demand SSB procedure triggered by UE

From RAN2’s perspective, UE can send on demand SSB request to network via UE assistance information if we go for pure RAN2 solution without much impact in RAN1. Upon reception of the request, it is still up to network to decide whether to activate the on demand SSB transmission while the activation/deactivation procedure for NW triggered procedure can be reused.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Configuration of on-demand SSB

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Proposal 1: For case#2, UE should measure the always on SSB before on demand SSB transmission is activated.

Observation 2: For case#2, the SCell addition/modification with initial state set to “deactivated” for the corresponding SCell can be reuse while UE needs to be aware of the SSB transmission update upon SSB transmission triggering.

Proposal 2a: For on demand SSB configuration, the existing SCell addition/modification is reused with the initial state set to “deactivated” for the corresponding SCell for both case#1 and case#2.

Proposal 2b: For case#1, an additional indication should be introduced to indicate that UE does not need to measure SSB for the corresponding SCell.

L3 measurement

Proposal 3: RAN2 to discuss the following options for L3 measurement handling in case#2 and inform RAN1/4:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Introduce additional SMTC for OD-SSB, UE measures OD-SSB in the additional SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

On-demand SSB procedure

Proposal 4: The on demand SSB procedure can be triggered by NW or UE.

Proposal 5a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

Proposal 5b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

--Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

--Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 6: On demand SSB deactivation via MAC CE should be supported:

--For case#1, UE stop SSB measurements upon reception of the deactivation.

--For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Reference

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v2

- 3GPP, R4-2416911, LS on SSB adaption, RAN4#112bis, Oct, 2024