3GPP TSG-RAN WG2 Meeting #128										    R2-2410167

Orlando, USA, 14 - 18 Nov, 2024                                                  Revision of R2-2408008

Agenda item:		8.4.3

Title: 	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN #102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved in [1].

In RAN2#126 meeting, the following agreements have been made for RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode [3]:

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In RAN2#127 meeting, the following agreements have been made for RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode [5]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In RAN2#127bis meeting, the following assumption has been made for RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode [6]:

Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In this contribution, we will discuss the RAN2 related issue on RRM measurement relaxation and offloading for IDLE/INACTIVE modes and give our proposals.

- Discussion

Based on the RAN2#126 agreement that “RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise”, it can be understood that LP-WUR can only be used for serving cell measurement.

Considering that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and the main radio can be turn off for UE power saving, which has also been agreed in RAN4#111, it can be understood that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement (e.g. both the serving sell measurement based on MR and neighbor cell measurement are stopped).

Proposal 1: RAN2 confirms that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement for both serving cell and neighbor cell.

Since RAN4 has agreed to support the RRM measurement relaxation, e.g. the following case#3, RAN2 has agreed that RRM measurement of the neighbouring cell can only be performed by MR, it can be understood that MR serving cell measurement can be relaxed(e.g. use larger measurement cycle than legacy) when LP-WUR measurement is started. And with the LP-WUR measurement aided, same measurement accuracy can be obtained although the MR serving cell measurement is relaxed.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: RAN2 confirms that serving cell measurement relaxation means that UE performs serving cell measurement based on MR with larger measurement cycle than legacy, while perform serving cell measurement based on LP-WUR simultaneously.

  - Serving cell measurement relaxation

Based on the current agreement in RAN2 that the RRM measurement of the neighbour cell can only be performed by MR, it can be assumed that RRM measurement relaxation is to relax the serving cell measurement when the LP-SS based measurement is performed. In NB-IoTand eMTC, serving cell measurement relaxation is supported, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the UE supporting WUS shall relax serving cell measurement requirement defined with the DRX cycle from the DRX cycle length of DRX cycle to the DRX cycle length of min (DRX cycle * numDRX-CyclesRelaxed, 10.24s), the similar mechanism can be used for NR serving cell measurement relaxation.

Proposal 3: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX cycle to min( M1*N1*DRX cycle * numDRX-CyclesRelaxed, 10.24s).

In NB-IoT and eMTC, the coverage of WUS is the same as the coverage of cell. But in the NR, the coverage of LP-WUS may be less than or equal to the cell coverage, and the NR serving cell measurement relaxation can only be performed only in the coverage of the LP-WUS. Thus, cell edge condition (e.g. whether in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation.

In general, the RSRP and RSRQ are used as the metric for determining the cell coverage. Considering the different measurement accuracy of MR, OFDM-based WUR and OOK-based WUR, the RSRP and RSRQ measured by MR, OFDM-based WUR and OOK-based WUR needs to be distinguished.

- Specially, for UE with OOK-based WUR, if both of RSRP measured by MR and OOK-based WUR of the serving cell are larger than an RSRP threshold, optionally both of RSRQ measured by MR and OOK-based WUR of the serving cell are larger than an RSRQ threshold, the NR serving cell measurement relaxation can be performed.

- For UE with OFDM-based WUR, if both of RSRP measured by MR and OFDM-based WUR of the serving cell are larger than an RSRP threshold, optionally both of RSRQ measured by MR and OFDM-based WUR of the serving cell are larger than an RSRQ threshold, the NR serving cell measurement relaxation can be performed.

Proposal 4: Cell edge condition (i.e. whether the UE is in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation, e.g. the (LP-) RSRP of the serving cell is larger than an RSRP threshold and optionally the (LP-) RSRQ of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- For UE with OOK-based WUR, both of the RSRP measured by MR and OOK-based WUR of the serving cell are larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OOK-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- for UE with OFDM-based WUR, both of the RSRP measured by MR and OFDM-based WUR of the serving cell is larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OFDM-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

  - Neighbor cell measurement relaxation

Since RAN2 understands that the RRM measurement of the neighboring cell can only be performed by MR, the neighboring cell measurement accuracy has not changed when introducing LP-WUS, thus the neighbor cell measurement relaxation should not be changed, e.g. the legacy Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ can reused for neighbour cell measurement relaxation.

Proposal 5a: The legacy Rel-16 criteria for ‘low mobility’ is reused for neighbour cell measurement relaxation.

Since the RRM measurement of the neighboring cell can only be performed by MR, when UE perform neighboring cell measurement relaxation, the MR is still in “on” state, at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered. In which case, UE can always obtain the serving cell’s MR measurement results. Whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

Proposal 5b: RAN2 confirms that when UE performs neighboring cell measurement relaxation, the MR is still in “on” state, e.g. at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered.

Proposal 5c: When UE performs neighboring cell measurement relaxation, whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

  - Serving cell measurement offloading

In order to achieve the UE power saving gain by LP-WUS/WUR, the RRM measurement on serving cell and neighbouring cell via MR may be skipped when UE is using LP-WUS or MR is in ultra-deep sleep. However, UE still needs to ensure that IDLE/INACTIVE mode measurements are possible during this state. In order to avoid or minimize the performance degradation of mobility handling in RRC_IDLE or RRC_INACTIVE when MR is stopped, RAN2 has already agreed in SI stage [2] that: ultra-deep sleep state could be entered when the serving cell quality of both LP-SS and MR SSB is larger than a threshold, e.g. UE is not at cell edge. As legacy, the relaxed measurement criterion for UE not at cell edge [3] can be used to decide whether the UE is at cell edge. Whether a threshold is separately defined for LP-SS and MR SS can be further considered, e.g. depending on the measurement performance of LP-WUS defined by RAN4.

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

The relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

Proposal 7a: RAN2 confirms that the relaxed measurement criterion for UE not at cell edge (e.g. based on (LP-) RSRP value, and optional (LP-) RSRQ value) is used for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE modes.

Furthermore, some UEs may move quickly, and the resumption of Main Radio may require longer duration (e.g. several seconds), the serving cell measurement relaxation of UE MR based on LP-WUR should consider UE mobility, e.g. only the UEs with low mobility can apply the serving cell measurement relaxation of UE MR. As legacy, the Relaxed measurement criterion for UE with low mobility [3] can be used to decide whether the UE is in low mobility.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

Proposal 7b: The relaxed measurement criterion for UE with low mobility (e.g. based on (LP-) RSRP change) is reused for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

- Case 1: UE has not perform MR serving cell measurement offloading and may perform MR serving cell measurement on MR SSB and LP-SS

When the UE is not performing MR serving cell measurement offloading, the UE may perform RRM measurements based on MR SSS and LP-SS, the UE can decide whether it is in low mobility state and not at cell edge based on MR SSS and/or LP-SS change (e.g. based on (LP) RSRP change, (LP) RSRP value, optionally LP-RSRQ value).

Proposal 8a: When the UE is not performing MR serving cell measurement offloading, serving cell quality change and quality based on MR SSB and LP-SS measurement can be used to determine whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

- Case 2: UE has performed MR serving cell measurement offloading and perform MR serving cell measurement only on LP-SS

While the UE performing MR serving cell measurement offloading, UE perform serving cell measurement based on LP-SS only. If the UE determines that it is neither in stationary state nor is at the cell edge based on the LP-SS measurement, the UE starts to perform serving cell measurements based on main radio (e.g. MR SSB).

Proposal 8b: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

As legacy, the Relaxed measurement criterion for UE with low mobility (e.g. the LP-SS RSRP change value is less than a threshold) can be used to decide whether the UE is in low mobility, and relaxed measurement criterion for UE not at cell edge (e.g. the LP-SS RSRP value is lower than an RSRP threshold and the LP-SS RSRQ value is lower than an RSRQ threshold, if configured) can be used to decide whether the UE is at the cell edge.

Proposal 8c: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and optionally the LP-RSRQ value is lower than an RSRQ threshold, if configured.

However, if the entry conditions for serving cell measurement offloading is satisfied, e.g. in low mobility state or not in the cell age, under some special case, UE couldn’t perform serving cell measurement offloading, e.g. the serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

Proposal 9: The serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

- Conclusion

Proposal 1: RAN2 confirms that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement for both serving cell and neighbor cell.

Proposal 2: RAN2 confirms that serving cell measurement relaxation means that UE performs serving cell measurement based on MR with larger measurement cycle than legacy, while perform serving cell measurement based on LP-WUR simultaneously.

Serving cell measurement relaxation

Proposal 3: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX cycle to min( M1*N1*DRX cycle * numDRX-CyclesRelaxed, 10.24s).

Proposal 4: Cell edge condition (i.e. whether the UE is in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation, e.g. the (LP-) RSRP of the serving cell is larger than an RSRP threshold and optionally the (LP-) RSRQ of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- For UE with OOK-based WUR, both of the RSRP measured by MR and OOK-based WUR of the serving cell are larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OOK-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- for UE with OFDM-based WUR, both of the RSRP measured by MR and OFDM-based WUR of the serving cell is larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OFDM-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

Neighbor cell measurement relaxation

Proposal 5a: The legacy Rel-16 criteria for ‘low mobility’ is reused for neighbor cell measurement relaxation.

Proposal 5b: RAN2 confirms that when UE performs neighboring cell measurement relaxation, the MR is still in “on” state, e.g, at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered.

Proposal 5c: When UE performs neighboring cell measurement relaxation, whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

Serving cell measurement offloading

Proposal 7a: RAN2 confirms that the relaxed measurement criterion for UE not at cell edge (e.g. based on (LP-) RSRP value, and optional (LP-) RSRQ value) is used for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE modes.

Proposal 7b: The relaxed measurement criterion for UE with low mobility (e.g. based on (LP-) RSRP change) is reused for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

Proposal 8a: When the UE is not performing MR serving cell measurement offloading, serving cell quality change and quality based on MR SSB and LP-SS measurement can be used to determine whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

Proposal 8b: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

Proposal 8c: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and optionally the LP-RSRQ value is lower than an RSRQ threshold, if configured.

Proposal 9: The serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

- References

- RP-240801 Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR) , RAN#103

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- 3GPP TR 38.869 V18.0.0 (2023-12), Study on low-power wake-up signal and receiver for NR

- 3GPP TR 38.304 V18.1.0 (2024-03), NR;User Equipment (UE) procedures in Idle mode and RRC Inactive state

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Annex

RAN4#111 agreement

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

RAN4#112 agreements

Agreement:

- Support case #3

  - For serving cell measurement, further discuss:

    - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR

    - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR

    - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

    - Other options are not precluded.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

  - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.