3GPP TSG-RAN WG2 Meeting #128	R2-2410187

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.3.2.1

Source:	OPPO

Title:	Discussion on simulation result of RRM measurement prediction

Document for:	Discussion, Decision

Introduction

In RAN2#126, the following cases are identified for RRM measurement prediction with 3 cases of high priority for evaluation.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In RAN2#127bis, we have reached agreements on some general observations based on performance results. How to capture those performance results into TR needs some discussion. This paper tends to provide a possible way. Additionally, we would also like to get more observations from simulation results.

Discussion

Ways to capture performance results

In TR 38.843 [1], companies’ simulation results under different scenarios are captured in different sub-sections. The ways to capture performance results for beam management are illustrated in the following structure:

Figure 1. Illustration of the hierarchy of beam management section in TR 38.843

We can see from Figure 1 that the basic principle is to give key cases a separate sub-section to record the detailed results and leave the rest to the “performance under different assumptions/scenarios…” sub-section.

In RRM, the current simulation results focus on high- and medium- priority scenarios, i.e., scenarios 2, 3, 4 and 6. If we follow the same principle as TR 38.843, we can give those higher-priority scenarios separate sub-sections to capture their extensive simulation results. Therefore, it is proposed that

Proposal 1: RAN2 can capture the performance results in the following structure:

RRM prediction

- Basic performance for scenario 2

- Basic performance for scenario 3

- Basic performance for scenario 4

- Basic performance for scenario 6

- Generalization performance

- Summary of performance results

For each high-priority scenario, we have simulated the performance under different sub-use cases. The input and output of different sub-use cases vary and may have different specification impacts. Therefore, it is better to place them into parallel sections secondary to the basic performance of scenarios.

Proposal 2: Under basic performance for scenarios 2, 3, 4 and 6, using separate sub-sections for sub cases 1, 2, and 3, e.g.,

- Basic performance for scenario 2

  - Sub case 1

  - Sub case 2

  - Sub case 3

Again, let us look at the structure of TR 38.843,

(A)With measurements of fixed Set B of beams that of 1/4 of Set A of beams - Top-1 DL Tx beam prediction accuracy: - evaluation results from xx sources indicate that, AI/ML can achieve about xx%~xx% beam prediction accuracy… - Top-1 DL Tx beam with 1dB margin… - Top-K(=2) DL Tx beam prediction accuracy… - (B) With measurements of fixed Set B of beams that of 1/8 of Set A of beams - Top-1 DL Tx beam prediction accuracy…

The assumptions that may impact the performance results are displayed in items (A) and (B). RAN2 can adopt a similar approach and identify which factors matter most.

For the four higher-priority scenarios, the factors that impact the performance results vary and should be treated case by case. In

[POST127bis][016][AI Mob] Simulation results (Mediatek)

Intended outcome: finalize the table (one week email deadline)

- Informational email discussion on logistics of storing simulation results.

items in the “Variable setting” could play the role of those important factors. Table 1 shows those listed factors.

Table 1. Variables in “Variable setting” of different scenario spreadsheets.

| Scenario 2 | MRRT, OW, PW, UE speed |
| --- | --- |
| Scenario 3 | OW, channel correlation coefficient, UE speed |
| Scenario 4 | OW, PW, UE speed |
| Scenario 6 | MRRS, UE speed |

In Scenario 2, since the MRRT partially reflects the impact of both the observation and prediction windows, discussing them together is redundant. In Scenario 4, the observation and prediction windows are typically intertwined, allowing us to consolidate their effects into a single factor. For Scenario 3, we've found that UE speed has a minimal effect on prediction accuracy, and the length of the observation window can be company implementation. The channel correlation coefficient is a variable statistic that differs among companies and cannot be set to a specific value. In Scenario 6, the findings are somewhat limited due to the limited input from companies. It appears that there is no necessity to introduce additional factors for scenarios 3 and 6.

Proposal 3: Within each sub case, capture the following sub-bullets on performance results:

- For scenario 2: (A) MRRT, (B) UE speed

- For scenario 3: No sub-bullet

- For scenario 4: (A) observation window : prediction window, (B) UE speed

- For scenario 6: No sub-bullet

For easy of understanding, we have given an example of the proposed structure of the TR in the annex.

Sample-and-hold vs. AI for scenarios 2

In RAN2#127bis, two companies [QC and MTK] presented their non-AI results on scenario 4 and showed that AI can provide better performance than sample-and-hold, especially when PW is large. Compared to scenario 4, AI’s benefits in scenario 2 seem to be a little controversial. In this section, we would like to look into it. As in RAN2#127bis, we use example 2 as the skipping pattern. The difference between non-sliding and sliding filtering is the length of two consecutive measurement instances.

Figure 2. Illustration of used skipping patterns

For the sample-and-hold baseline, we simply use the last instance within OW to be the sample held for PW. The prediction accuracy of sample-and-hold and AI is shown in Table 2.

Table 2. Temporal domain case B results of AI and sample-and-hold

| Filtering option |  | Non-sliding |  |  |  | sliding |
| --- | --- | --- | --- | --- | --- | --- |
| UE speed |  | 30km/h |  |  | 60km/h | 30km/h |
| MRRT |  | 50% | 67% | 75% | 50% | 50% |
| OW |  | 600ms | 800ms | 1000ms | 600ms | 120ms |
| PW |  | 200ms | 400ms | 600ms | 200ms | 40ms |
| Average L3 cell-level RSRP difference (dB) | AI | 0.26 | 0.41 | 0.64 | 0.51 | 0.10 |
|  | Non-AI: sample-and-hold | 0.41 | 0.61 | 0.81 | 0.73 | 0.10 |
| AI gain (= 1 – AI / non-AI) |  | 37% | 33% | 21% | 30% | 0% |

Observation 1: For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold. The observed AI gain ranges from 21% to 37%.

For the sliding filtering approach, PW is short, which means the RSRP would not vary much from the last instance within OW. Therefore, we can see that sample-and-hold can achieve a performance comparable to AI. However, when PW is larger (i.e., in non-sliding cases), the gains of AI become pronounced as the channel state changes more significantly.

As MRRT increases, we can observe that

Observation 2: For FR1 intra-frequency temporal domain case B, the increase of MRRT results in a decrease in AI gain.

One of the reasons that explain this is that the increase of MRRT leads to worse performance of AI prediction, The negative impact of MRRT on AI is more significant than on sample-and-hold, making the gain of AI less significant. A similar phenomenon can also be found in UE speed impact.

Observation 3: For FR1 intra-frequency temporal domain case B, the increase of UE speed results in a decrease in AI gain.

MRRT for scenarios 2

In RAN2#127bis, the majority of companies [OPPO, vivo, CMCC, CATT, Xiaomi, Samsung, Huawei] found that the prediction accuracy decreases when MRRT increases. ZTE holds a different view, as they find that 66.7% MRRT with skipping pattern example 2 performs better than 60% MRRT with skipping pattern example 1 as shown in the highlighted part in the following figure.

Figure 3. Simulation results of scenario 2 from [2]

The difference is because example 2 achieves better accuracy than example 1 as observed in [3]. In fact, when the same skipping pattern is adopted as shown in Figure 3 lower panel (with MRRT 66.7% and 50%), ZTE’s results are consistent with what was found by other companies, i.e., an increase in MRRT leads to a decrease in prediction accuracy. Additionally, according to [3], the filtering option adopted would also influence the final L3-RSRP difference results. Therefore we can conclude that:

Observation 4: the increase of MRRT deteriorates the prediction accuracy of FR1 intra-frequency temporal domain case B when the same filtering option and the same skipping pattern are adopted.

It is proposed that

Proposal 4: Capture the following observation into TR: the increase of MRRT deteriorates the prediction accuracy of FR1 intra-frequency temporal domain case B when the same filtering option and the same skipping pattern are adopted.

Conclusion

In this contribution, we have shared our simulation results on RRM measurement prediction and have the following observations:

Observation 1: For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold. The observed AI gain ranges from 21% to 37%.

Observation 2: For FR1 intra-frequency temporal domain case B, the increase of MRRT results in a decrease in AI gain.

Observation 3: For FR1 intra-frequency temporal domain case B, the increase of UE speed results in a decrease in AI gain.

Observation 4: the increase of MRRT deteriorates the prediction accuracy of FR1 intra-frequency temporal domain case B when the same filtering option and the same skipping pattern are adopted.

It is proposed that

Proposal 1: RAN2 can capture the performance results in the following structure:

RRM prediction

- Basic performance for scenario 2

- Basic performance for scenario 3

- Basic performance for scenario 4

- Basic performance for scenario 6

- Generalization performance

- Summary of performance results

Proposal 2: Under basic performance for scenarios 2, 3, 4 and 6, using separate sub-sections for sub cases 1, 2, and 3, e.g.,

- Basic performance for scenario 2

  - Sub case 1

  - Sub case 2

  - Sub case 3

Proposal 3: Within each sub case, capture the following sub-bullets on performance results:

- For scenario 2: (A) MRRT, (B) UE speed

- For scenario 3: No sub-bullet

- For scenario 4: (A) observation window : prediction window, (B) UE speed

- For scenario 6: No sub-bullet

Proposal 4: Capture the following observation into TR: the increase of MRRT deteriorates the prediction accuracy of FR1 intra-frequency temporal domain case B when the same filtering option and the same skipping pattern are adopted.

Reference

TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML), Release 18

R2-2409207, Evaluation on RRM measurement prediction, ZTE

R2-2407977, Discussion on simulation result of RRM measurement, OPPO

Annex

Proposed structure for RRM evaluation results

- 5.2.2 		Evaluation results

- 5.2.2.1 		Basic performance for scenario 2

- 5.2.2.1.1 		Performance of sub case 1

- Table 5.1 example for capturing performance results

| - UE speed \ MRRT | - 50% | - 67% | - … |
| --- | --- | --- | --- |
| - 30km/h | - Average L3 RSRP cell difference - [0.1, 1] dB: 9 sources - [xx, xx] dB: xx sources - … | - … |  |
| - 60km/h | - … | - … |  |
| - … |  |  |  |

- 5.2.2.1.2			Performance of sub case 2

- 5.2.2.1.3			Performance of sub case 3

- 5.2.2.2 		Basic performance for scenario 3

- 5.2.2.2.1 		Performance of sub case 1

- 5.2.2.2.2 		Performance of sub case 2

- Average L3 RSRP cell difference

              - [0.1, 1] dB: 9 sources

              - [xx, xx] dB: xx sources

          - …

- 5.2.2.2.3 		Performance of sub case 3

- 5.2.2.3 		Basic performance for scenario 4

- 5.2.2.3.1 		Performance of sub case 1

- 5.2.2.3.2 		Performance of sub case 2

- Table 5.2 example for capturing performance results

| - UE speed \ OW: PW | - 800ms:400ms | - 1200ms: 400ms | - … |
| --- | --- | --- | --- |
| - 30km/h | - Average L3 RSRP cell difference - [0.1, 1] dB: 9 sources - [xx, xx] dB: xx sources - … | - … |  |
| - 60km/h | - … | - … |  |
| - … |  |  |  |

- 5.2.2.3.3 		Performance of sub case 3

- 5.2.2.4 		Basic performance for scenario 6

- 5.2.2.4.1 		Performance of sub case 1

- 5.2.2.4.2 		Performance of sub case 3

- MRRS = xx, UE speed = xx:

  - Average L3 RSRP cell difference

                - [0.1, 1] dB: 9 sources

                - [xx, xx] dB: xx sources

  - …

- MRRS = xx, UE speed = xx:

- …

- 5.2.2.5 		Generalization performance

- 5.2.2.6 		Summary of performance results