3GPP TSG-RAN WG2 #128	R2-2410208

Orlando, USA, 18th – 22nd Nov. 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on LCP enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4.1		LCP enhancements

- Introduction

In RAN #105, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

And the following agreements related to LCP enhancements were reached in RAN2#127bis meeting.

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

In this contribution, we provide the discussion on LCP enhancements for UL scheduling.

- Discussion

For delay-aware LCP enhancement, RAN2 agreed to support LCH priority adjustment based on delay status in Rel-19. A new remaining time threshold can be introduced to define “LCH priority-adjusted data”, and if there is the LCH priority-adjusted data for a LCH, an additional priority can be used for the LCH instead of the legacy priority. The adjustment to the higher priority level can provide more chances to transmit data with low remaining time.

Proposal 1:	If there is LCH priority-adjusted data for a LCH, an additional priority can be used for the LCH instead of the legacy priority. Otherwise, UE uses the legacy priority.

One issue would be how to configure the additional priority. For example, it is necessary to discuss whether the additional priority can be configured per LCH or in common for all LCHs. Considering the legacy priority configuration, it would be better to configure the additional one per LCH rather than across all LCHs. Furthermore, we need to consider how many additional priorities can be configured per LCH. If multiple additional priorities are introduced, the UE needs to determine which priority to use based on multiple delay status. For example, multiple remaining time thresholds may be needed for LCH priority adjustment, which complicates UE operation. For simplicity, only one additional priority per LCH would be sufficient.

Proposal 2:	Up to one additional priority can be configured for each LCH.

We can also consider the adjustment of the other LCP-related parameters (e.g., PBR, BSD). These parameters calculate Bj, which limits the resource size allocated in the 1st round of resource allocation procedure. Even if a LCH has a large amount of LCH priority-adjusted data, the UE can allocate resources to the LCH only up to its Bj in the 1st round. In this case, some of the data may remain buffered even with LCH priority adjustment. Therefore, it may be beneficial to consider adjusting PBR and/or BSD as well as LCH priority so that the UE can maximize the transmission of LCH priority-adjusted data.

Observation 1:	PBR and BSD are configured for a LCH and limit the size of resources allocated to the LCH in the 1st round of resource allocation procedure.

Proposal 3:	RAN2 to consider whether the adjustment of PBR and/or BSD can be supported in order to maximize the transmission of LCH priority-adjusted data.

In the previous meeting, it was agreed that the UE determine if a LCH has LCH priority-adjusted data when starting the 1st round to determine its priority for both 1st and 2nd rounds. However, there is the FFS on if the UE can still change the priority for the 2nd round. For example, if all LCH priority-adjusted data is exhausted in the 1st round, the UE can fall back to the legacy priority for the 2nd round. While this operation is beneficial to solve the fairness issue, it can increase the UE complexity and the processing time of the LCP procedure. Furthermore, the fairness issue can also be avoided with NW configuration. For example, with proper PBR configuration, resources can be fairly allocated in the 1st round, and there may be only a few resources left in the 2nd round. Therefore, re-adjusting the LCH priority for the 2nd round may not provide much benefit.

Proposal 4:	UE can determine if a LCH has LCH priority-adjusted data only when starting the 1st round to determine its priority for both the 1st and the 2nd rounds. The UE doesn’t need to re-adjust the LCH priority for the 2nd round.

Another issue would be when is the reference time for the UE to determine if a LCH has the LCH priority-adjusted data. One possible option is at the time of the LCP procedure. In other words, the UE determines if the remaining time at the time of the LCP procedure is below the new threshold. However, the LCP procedure is performed prior to the actual transmission of the corresponding data. Therefore, there may be the cases where the data is LCH priority-adjusted data at the time of transmission, but not at the time of the LCP procedure. In this case, LCH priority adjustment may not be applied even though the data is actually delayed. From the above, it would be more suitable to use the point of transmission as the reference time at which the UE determines if the remaining time is below the threshold for delay-aware LCP.

Proposal 5:	RAN2 to consider the following options as a reference time for UE to determine if a LCH has LCH priority-adjusted data for delay-aware LCP.

- Option1: At the time of LCP procedure

- Option2: At the time of transmission of the corresponding data

Proposal 6:	For the reference time, UE can determine if a LCH has LCH priority-adjusted data based on the remaining time at the transmission of the corresponding data.

LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization). Therefore, it needs to be discussed whether the additional LCH priority can be used for those purposes as well. For BSR trigger, there seems to be no advantage to using the additional priority. BSR should always be triggered based on the legacy priority to avoid complexity.

Observation 2:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Observation 3:	For BSR trigger, there seems to be no advantage to using the additional priority.

Proposal 7:	BSR should always be triggered based on the legacy priority even if the additional priority is configured.

On the other hand, for intra-UE prioritization, it may be useful to adjust the LCH priority based on the delay status. In intra-UE prioritization, if there are overlapping UL grants, only the UL grant with the highest priority is considered as a prioritized UL grant and can be used for transmission. The priority of each UL grant is determined by the highest priority among priorities of the LCHs that are multiplexed or have data available that can be multiplexed in the MAC PDU. This can lead to the same problem as in the LCP procedures. In other words, an UL grant for which delay-critical data from LCHs with a lower priority are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed. Therefore, it needs to be considered to apply LCH priority adjustment to intra-UE prioritization as well.

Observation 4:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed.

Proposal 8:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- Conclusion

In this contribution, we provided the following observations and proposals.

Proposal 1:	If there is LCH priority-adjusted data for a LCH, an additional priority can be used for the LCH instead of the legacy priority. Otherwise, UE uses the legacy priority.

Proposal 2:	Up to one additional priority can be configured for each LCH.

Observation 1:	PBR and BSD are configured for a LCH and limit the size of resources allocated to the LCH in the 1st round of resource allocation procedure.

Proposal 3:	RAN2 to consider whether the adjustment of PBR and/or BSD can be supported in order to maximize the transmission of LCH priority-adjusted data.

Proposal 4:	UE can determine if a LCH has LCH priority-adjusted data only when starting the 1st round to determine its priority for both the 1st and the 2nd rounds. The UE doesn’t need to re-adjust the LCH priority for the 2nd round.

Proposal 5:	RAN2 to consider the following options as a reference time for UE to determine if a LCH has LCH priority-adjusted data for delay-aware LCP.

- Option1: At the time of LCP procedure

- Option2: At the time of transmission of the corresponding data

Proposal 6:	For the reference time, UE can determine if a LCH has LCH priority-adjusted data based on the remaining time at the transmission of the corresponding data.

Observation 2:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Observation 3:	For BSR trigger, there seems to be no advantage to using the additional priority.

Proposal 7:	BSR should always be triggered based on the legacy priority even if the additional priority is configured.

Observation 4:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed.

Proposal 8:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- References

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3