3GPP TSG-RAN WG2 #128	R2-2410243

Orlando, USA, Nov 18th-22nd, 2024

Agenda item: 		8.15

Title:		A-GNSS support for NavIC L1 SPS

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The Objective of the WID [1] for NavIC L1 band SPS is formulated as follows:

- Specify signalling support for NavIC L1 band SPS in NR specifications by introducing necessary assistance data information (e.g., Clock Model for NavIC, Navigation Model for NavIC, and Almanac for NavIC L1 SPS) based on the existing GNSS framework, including enhancements to Ionospheric model, NavIC system time and Ephemeris. [RAN2] - Specify signalling support for NavIC L1 band SPS in LTE specifications similar to NR [RAN2]

In this contribution, we provide our initial consideration on NavIC L1 band SPS in A-GNSS.

# 2. Discussion

The first generation NavIC L5 band SPS in A-GNSS is supported in TS 37.355. The current procedure and framework of TS 37.355 can be reused for NavIC L1 band SPS, but some NavIC L1 specific information and related description should be introduced.

For ionosphere model, based on the ICD of NavIC L1 band SPS [2], we observed that the ionospheric delay correction coefficients include the Klobuchar like model and NeQuick-N model.

For Klobuchar like model of ionosphere for NavIC L1 band SPS, compare to current KlobucharModelParameter IE which can be used for NavIC L5 SPS, the value of some parameters are different and new information/fields are needed for NavIC L1 band. Therefore, instead of reusing the current KlobucharModelParameter IE, a new Klobuchar Model Parameter IE (e.g., KlobucharModel3Parameter) can be introduced to support NavIC L1 band SPS.

The ionospheric correction based on NeQuick model for single frequency users of NavIC (NeQuick-N) will be computed through a set of 3 coefficients given for 3 regions defined by maximum and minimum modified dip latitude (MODIP) coverage, maximum and minimum longitude coverage, the ionosphere disturbance flag (IDF) for each set and the issue of data NeQuick-N (IODN). Therefore, in order to support NeQuick model based ionospheric correction for NavIC L1 band SPS, a new NeQuick-N model parameter IE should be introduced, which including the parameters of Table 17 of section 6.1.3.3 in reference [2].

Proposal 1: The new Klobuchar Model parameter IE and NeQuick-N Model parameter IE is introduced for ionospheric model.

For GNSS Navigation Model, the svID, svHealth, gnss-ClockModel and gnss-OrbitModel information is included.

- GNSS-NavigationModel

-- ASN1START

GNSS-NavigationModel ::= SEQUENCE {

nonBroadcastIndFlag		INTEGER (0..1),

gnss-SatelliteList		GNSS-NavModelSatelliteList,

...

}

GNSS-NavModelSatelliteList ::= SEQUENCE (SIZE(1..64)) OF GNSS-NavModelSatelliteElement

GNSS-NavModelSatelliteElement ::= SEQUENCE {

svID				SV-ID,

svHealth			BIT STRING (SIZE(8)),

iod					BIT STRING (SIZE(11)),

gnss-ClockModel		GNSS-ClockModel,

gnss-OrbitModel		GNSS-OrbitModel,

...,

[[	svHealthExt-v1240 BIT STRING (SIZE(4))			OPTIONAL		-- Need ON

]]

}

... ...

In order to support the NavIC L1 band SPS, we further discuss the impact on GNSS navigation model:

- the L1 health indication for NavIC should be introduced for svHealth information.

              - GNSS to svHealth Bit String(8) relation

| GNSS | svHealth Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| GPS L1/CA(1) | SV Health [4] |  |  |  |  |  | '0' (reserved) | '0' (reserved) |
| Omitted |  |  |  |  |  |  |  |  |
| BDS(7) B1C [39]/B2a [49] | Sat Clock Health [39], [49] | B1C Health [39], [49] | B2a Health [39],[49] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| NavIC | L5 health | 'L1health (reserved) | (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |

- the iod of NavIC L1 SPS is indicated by 11 bits of toec (i.e., time of ephemeris and clock) in reference [2].

For NavIC L5 band, we observed that the 11 MSB bits of toe (i.e., time of ephemeris) is used to indicate the iod information, with the scaling factor of 512 (i.e., 24+5).

Similarly, 11 bits of toec (i.e., time of ephemeris and clock) [2] for NavIC L1 band can be used to indicate the iod information, with the scaling factor of 300.

              - GNSS to iod Bit String(11) relation

| GNSS | iod Bit String(11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 | Bit 11 (LSB) |
| GPS L1/CA | '0' | Issue of Data, Clock [4] |  |  |  |  |  |  |  |  |  |
| Modernized GPS | toe (seconds, scale factor 300, range 0 – 604500) [4,5,6] |  |  |  |  |  |  |  |  |  |  |
| SBAS | '0' | '0' | '0' | Issue of Data ([10], Message Type 9) |  |  |  |  |  |  |  |
| QZSS QZS-L1 | '0' | Issue of Data, Clock [7] |  |  |  |  |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | toe (seconds, scale factor 300, range 0 – 604500) [7] |  |  |  |  |  |  |  |  |  |  |
| GLONASS | '0' | '0' | '0' | '0' | tb (minutes, scale factor 15) [9] |  |  |  |  |  |  |
| Galileo I/NAV | '0' | IODnav [8] |  |  |  |  |  |  |  |  |  |
| BDS B1I/B3I | 11 MSB bits of toe (seconds, scale factor 512, range 0 – 604672) [23], [50] |  |  |  |  |  |  |  |  |  |  |
| BDS B1C/B2a | '0' | Issue of Data, Clock [39], [49] |  |  |  |  |  |  |  |  |  |
| NavIC | 11 MSB bits of toe (seconds, scale factor 512) [38] |  |  |  |  |  |  |  |  |  |  |
| NavIC L1 | 11 bits of toec (seconds, scale factor 300) [xx] |  |  |  |  |  |  |  |  |  |  |

- a new NavIC clock Model is introduced to support L1 band.

Compare the parameters of NavIC-ClockModel IE in TS 37.355 [3] with the clock model parameters of NavIC L1 band SPS [2], we observed that the value/bits of parameters are different, and some new information/fields should be introduced for NavIC L1 band. Therefore, instead of reusing the existing NavIC-ClockModel IE, we suggest to introduce a new clock Model for NavIC L1 band.

- a new NavIC orbit Model is introduced to support L1 band.

Similar to clock model, we think that a new orbit model should be introduced for NavIC L1 band SPS.

Based on the above analysis, we propose:

Proposal 2: For GNSS-navigationModel IE, the following information is introduced to support NavIC L1 band SPS:

- the NavIC L1 health indication should be introduced

- the iod of NavIC L1 SPS is indicated by 11 bits of toec (seconds, scale factor 300) in reference [2]

- a new NavIC clock Model is introduced

- a new NavIC orbit Model is introduced

For GNSS-Almanac, in current TS 37.355, the AlmanacNavIC-AlmanacSet is used for NavIC to indicate the almanac related parameters. For NavIC L1 band SPS, based on the 6.1.3.2 of [2], we find that the current AlmanacNavIC-AlmanacSet IE can be reused, but the value of some parameters/fields in it needs to be extended to applicable for the NavIC L1, e.g., navic-AlmE, navic-AlmOMEGADOT, navic-Almf0.

Proposal 3: For almanac model, extend the value of some parameters/fields in AlmanacNavIC-AlmanacSet IE, e.g., navic-AlmE, navic-AlmOMEGADOT, navic-Almf0, to support the NavIC L1 band SPS.

The GNSS-UTC-Model is used by the location server to provide several sets of parameters needed to relate GNSS system time to Universal Time Coordinate (UTC). For NavIC L1 band SPS, the UTC-ModelSet2 can be reused.

Proposal 4: Reuse the UTC-ModelSet2 for NavIC L1 band SPS.

In current TS 37.355, the IE NavIC-GridModelParameter is used by the location server to provide Ionospheric Grid Information to the target device for NavIC L5 band SPS, wherein the transformation timing of navigation message provided by the time of week count (TOWC) corresponding to the given set of grid ionospheric parameters. The Time of Week (TOW) in seconds is obtained by multiplying TOWC with 12 and the TOW count value ranges from 1 to 50400 to cover one entire week.

However, for NavIC L1 band SPS, there have different transformation indication method. Specifically, it provided by interval time of week count (ITOWC) and time of interval count (TOIC). The range of the ITOWC is from 0 to 83 2-hour epochs, and the TOIC range from 0 to 399 is the count of the number of 18 sec message intervals in each 2 hours.

Proposal 5: For NavIC L1 band SPS, the transmission timing of the navigation message provided by the ITOWC and TOIC corresponding to the given set of grid ionospheric parameters. The range of the ITOWC is from 0 to 83 and the range of TOIC is from 0 to 399.

# 3. Conclusion

In this contribution we discussed impact of NavIC L1 band SPS and made the following proposals.

Proposal 1: The new Klobuchar Model parameter IE and NeQuick-N Model parameter IE is introduced for ionospheric model.

Proposal 2: For GNSS-navigationModel IE, the following information is introduced to support NavIC L1 band SPS:

- the NavIC L1 health indication should be introduced

- the iod of NavIC L1 SPS is indicated by 11 bits of toec (seconds, scale factor 300) in reference [2]

- a new NavIC clock Model is introduced

- a new NavIC orbit Model is introduced

Proposal 3: For almanac model, extend the value of some parameters/fields in AlmanacNavIC-AlmanacSet IE, e.g., navic-AlmE, navic-AlmOMEGADOT, navic-Almf0, to support the NavIC L1 band SPS.

Proposal 4: Reuse the UTC-ModelSet2 for NavIC L1 band SPS.

Proposal 5: For NavIC L1 band SPS, the transmission timing of the navigation message provided by the ITOWC and TOIC corresponding to the given set of grid ionospheric parameters. The range of the ITOWC is from 0 to 83 and the range of TOIC is from 0 to 399.

# 4. References

- RP-242414, WID NavIC L1 SPS AGNSS in 5G NR & LTE

- NAVIC SIGNAL IN SPACE ICD FOR STANDARD POSITIONING SERVICEIN in L1 FREQUENCY, version 1.0, August, 2023

- 3GPP TS 37.355: “Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)”, V 18.3.0