3GPP TSG-RAN WG2 Meeting #128	R2-2410253

Orlando, USA, 18 – 22 November 2024

Agenda item:	6.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Maximum number of SI for on demand SI

WID/SID:	TEI17 - Release 17

Document for:	Discussion and Decision

# 1	Introduction

We discussed in RAN2#127bis:

Max nrof SI messages

R2-2408023	Clarification on the maximum number of SI for Msg3-based on-demand SI request	Xiaomi	discussion	Rel-17	TEI17

Proposal: RAN2 is kindly requested to discuss the following options for the Msg3-based on-demand SI request:

	Option 1: The network ensures that the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is 32.

	Option 2: If the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is more than 32, the UE chooses the first 32 entries in the list of concatenated SI messages used by requested-SI-List.

R2-2408024	Option 1 for clarifying the maximum number of SI for Msg3-based on-demand SI request (Rel-17)	Xiaomi	CR	Rel-17	38.331	17.10.0	4979	-	F	TEI17

R2-2408025	Option 1 for clarifying the maximum number of SI for Msg3-based on-demand SI request (Rel-18)	Xiaomi	CR	Rel-18	38.331	18.3.0	4980	-	A	TEI18

R2-2408026	Option 2 for clarifying the maximum number of SI for Msg3-based on-demand SI request (Rel-17)	Xiaomi	CR	Rel-17	38.331	17.10.0	4981	-	F	TEI17

R2-2408027	Option 2 for clarifying the maximum number of SI for Msg3-based on-demand SI request (Rel-18)	Xiaomi	CR	Rel-18	38.331	18.3.0	4982	-	A	TEI18

-	Ericsson think that in the RRC spec its clear that the max nrof SI messages is 32, so think that this is sufficient. Xiaomi think that if we concatenate two lists its unclear. ZTE think that this is not a real issue, but think the spec strictly allows 64 SI messages. LG think that the limit is 32 according to current spec and this is already clear. Vivo think that the spec allows for more than 32. Xiaomi thinks that the 32 SI message limitation is only for msg1-based SI request. Ericsson think that the SI-broadcast status field description.

Postponed, can come back if there are real issues.

We provide in this paper our view that there is no need for actions in RAN2

# 2	Discussion

R2-2408023 claimed following observation:

The UE behaviours on how to set the bits in requested-SI-List would be unclear when the concatenated SI message list used by requested-SI-List has more than 32 entries.

It is true that in theory it is possible to have more than 32 Sis in concatenated si-SchedulingInfos. but it should be noted that there limitation how many SIs may have status of “not broadcasted” as written in field descriptions:

Observation 1: It is not possible to have more than maxSI-Message SI messages having si-BroadcastStatus as “notBroadcasting”

If we study the

So we can see that there is no way of ever requesting more SI messages to be broadcast than maxSI-Message

Observation: UE cannot request in RRCSystemInfoRequest more than maxSI-Message amount of Sis

Proposal 1: RAN2 does not need to consider issues in amount of concatenated Sis in r15 and r17 SchedulingInfoLists

# 3	Conclusion

Proposal 1: RAN2 does not need to consider issues in amount of concatenated Sis in r15 and r17 SchedulingInfoLists