3GPP TSG-RAN WG2 Meeting#128	R2-2410274

Orlando, USA, Nov.18–22, 2024

Agenda item:	 8.10.2

Source:		Lenovo

Title:		Discussion on MRO for R18 mobility

Document for:		Discussion and Decision

1	Introduction

In this paper, we would discuss the details of MRO for R18 mobility features, including LTM and CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for LTM

We would focus on MCG LTM to mainly discuss RLF report, SHR and RA report.

To reduce interruption time during PSCell change procedure, SCG LTM is introduced in R18 with same priority as MCG LTM. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance, we can further discuss MRO for SCG LTM when the discussion of MRO for MCG LTM is stable.

    - 2.1.1 MRO for connection failure in a LTM procedure

RAN2#127 meeting agreed to include the specific access type in the RLF report, i.e. whether it is RACH-based or RACH-less cell switch. FFS details, e.g. if explicit or implicitly signalled. We can also find that RAN2#127bis meeting agreed to include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. It is better to align the failure case and near failure case, i.e. include explicit access type in the RLF report whether it is RACH-based or RACH-less LTM cell switch.

Proposal 1: Include explicit access type in the RLF report whether it is RACH-based or RACH-less LTM cell switch.

RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). Since it is network that configures the UE to perform UE based TA measurement or PDCCH ordered early TA acquisition, network can identify how the UE got the TA value based on the stored UE context, it is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 2: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

    - 2.1.2 MRO for near failure in a LTM procedure

RAN2#127 meeting agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Considering the case that network does not have UE context related with LTM configurations (e.g. the list of LTM candidate cells) when receiving the SHR, and to align with RAN2#127bis meeting’s agreement for RLF report (i.e. Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell), an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not may be included in the SHR.

Proposal 3: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR.

    - 2.1.3 RA report for a LTM procedure

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before cell switch command, early TA acquisition/UL synchronization procedure is triggered by PDCCH order to an LTM candidate cell, or the UE performs RACH for early UL synchronization or RACH for early TA acquisition;

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

To optimize RA resources for LTM procedure, the UE may report L1 measurement results (of source cell, or LTM candidate cell, or target cell) during LTM procedure in the RA report. Additionally, for optimization of early UL synchronization or early TA acquisition, a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early UL synchronization or early TA acquisition may be included in the RA report.

Proposal 4: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

  - 2.2 MRO for CHO with candidate SCG(s)

    - 2.2.1 MRO for connection failure case

RAN3 summarized the failure cases as below table in [1]:

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

It was agreed in RAN3#125 meeting that MRO for case 4/5/6 is in the scope and it was FFS whether there is any specification impact for case 4/5/6. In case of 4/5/6, configuration of CHO only is configured to the UE together with configuration of CHO with candidate SCG(s) configuration, the UE performs CHO only execution due to CHO with candidate SCG(s) can’t be triggered (e.g. CHO execution condition of CHO with candidate SCG(s) is fulfilled but CPAC execution condition of CHO with candidate SCG(s) is not fulfilled, CPAC execution condition of CHO with candidate SCG(s) is fulfilled but CHO execution condition of CHO with candidate SCG(s) is not fulfilled, neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled). To improve mobility performance, it is beneficial for network to know the reason about why CHO with candidate SCG(s) together with CHO only is configured but CHO only is performed instead of CHO with candidate SCG(s), e.g. only CHO execution condition or only CPAC execution condition of CHO with candidate SCG(s) is fulfilled, or neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled. RAN3#125 meeting agreed the RLF report includes the type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. This agreement applies to the case 4/5, for case 6, SCG failure information message needs to include the type of the first fulfilled execution condition of CHO with candidate SCG(s) if any e.g. CPAC or CHO.

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

RAN3#125bis meeting further agreed to include the identifier of candidate PCell(s) and/or PSCell(s) that fulfilled execution conditions when/before the RLF is encountered in RLF report. For case 6, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message.

Proposal 6: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

RAN2#127bis meeting agreed that the UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. For case 7b and case 8b, SCG failure happens after CHO with candidate SCG(s) is triggered. To analyse whether the failure is caused due to inappropriate CHO execution condition and/or CPAC execution condition, it is beneficial to store and report time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition as well as the elapsed time between the point in time of the first fulfilled execution condition and SCG failure in the SCG failure information message.

Proposal 7: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition as well as the elapsed time between the point in time of the first fulfilled execution condition and SCG failure can be stored and reported by the UE in SCG failure information message.

    - 2.2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen.

For legacy SHR or SPR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR or SPR. It seems that there is no issue to reuse legacy SHR or SPR trigger threshold for the CHO with candidate SCG(s) procedure.

When CHO with candidate SCG(s) is performed, both CHO execution condition of the target PCell and corresponding CPAC execution condition of the associated target PSCell are fulfilled, there is a possibility that the time duration between the fulfilment of CHO execution condition and the fulfilment of CPAC execution condition is higher than a threshold, which may cause a near-failure successful CHO with candidate SCG(s). Therefore, besides legacy T310/T312/T304 related trigger threshold, RAN2 can consider new trigger condition(s) for SHR or SPR for the CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell. When the time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell is higher than the threshold, SHR and SPR may be generated by the UE.

Proposal 8: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

No matter what is the trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), for sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, R17 SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 9: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, R18 SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

3	Conclusion

In this paper, the details of MRO for R18 mobility features are discussed. We have the following proposals:

MRO for LTM

Proposal 1: Include explicit access type in the RLF report whether it is RACH-based or RACH-less LTM cell switch.

Proposal 2: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 3: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR.

Proposal 4: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

MRO for CHO with candidate SCG(s)

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Proposal 6: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

Proposal 7: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition as well as the elapsed time between the point in time of the first fulfilled execution condition and SCG failure can be stored and reported by the UE in SCG failure information message.

Proposal 8: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 9: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

Reference

[1] R3-244223, Failure scenarios on MRO for CHO with candidate SCGs, Samsung, Cybercore, ZTE