3GPP TSG-RAN WG2 #128	R2-2410294

# Orlando, USA, 18th – 22th November 2024

Agenda Item:	8.12.2

Source:	Ericsson

Title:	Asymmetric DL/UL mTRP user plane impact from MIMO ph. 5

Release:	19

Document for:	Discussion

# 1	Introduction

Release 19 MIMO with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5 [1].

The subtopic objectives relevant to this WI are:

- UE initiated/event-triggered beam reporting,

- increased number of ports (up to 128) for CSI measurement and reporting,

- inter-TRP calibration for CJT,

- 3Tx for UL,

- asymmetric DL sTRP/UL mTRP.

This contribution discusses user plane impacts of asymmetric DL sTRP/UL mTRP that has clear impact to RAN2 specification, also considering some later RAN1 related agreements.

# 2	Discussion

2.1 Background Asymmetric DL sTRP UL mTRP

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment. In existing NR multi-TRP operation, it is assumed that each TRP can be used for both DL transmission and UL reception. Uplink power control can thus be based on the pathloss measurement of DL PL-RS from the same TRP that provides the DL service. Different pathloss for uplink and downlink arises when the power for uplink transmission towards UL-only TRP is based on, however, pathloss measurement of DL PL-RS from the Anchor TRP.

Apart from power control, an important aspect with asymmetric mTRP deployment is how to set the UL transmission timing towards the Anchor TRP and UL-only TRP respectively. In a practical deployment the propagation delay differences and synchronization timing error between the Anchor TRP and the non-collocated UL-only TRP can be significantly large that will impact the performance of the asymmetric TRP deployment.

## 2.2	User Plane Impacts

### 2.2.1		MAC-CE update of PL Offset value for Asymmetric DL sTRP UL mTRP

In RAN1 a PL Offset has been introduced to be configured per TCI State. This PL Offset is associated with one of the indicated joint/UL TCI State for the UL TRP in the unified TCI framework and is applied PDCCH order PRACH transmission. Here, details of the design is still to be defined.

As stated in the LS in the previous meeting, MAC-CE is assumed to be used to update the PL Offset value for a UL TCI state (or PL offsets for TCI-states/joint TCI-states). Note that the RAN1 agreement also assumes that the asymmetric DL sTRP/UL mTRP deployment scenario reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

To incorporate the associating PL offset with a TCI state for the asymmetric DL sTRP/ UL mTRP deployment RAN2 need to note what can be applied to FR1 and FR2. From the above agreement we see that when in Rel-18 and unified TCI is configured, for FR2, {one DL TCI state + up to two UL TCI states} can be applied.  As the agreement also is based on the reuse of the (Rel-17 and) Rel-18 unified TCI framework, and as the Rel-18 TCI framework currently only supports pairs of UL/DL TCI states (38.214, section 5.1.5), we assume here that for this case (FR2, when Rel-18 unified TCI is configured) and related to asymmetric DL sTRP/UL mTRP, where " {one DL TCI state + up to two UL TCI states} can be applied “, this then means that two separate DL/UL TCI state pairs can be configured and indicated, but a TCI codepoint associated with one DL TCI state is only used updating one of the two indicated DL TCI states for mTRP operation. As this interpretation may impact the MAC CE design, it can be useful though to have this understanding confirmed by RAN1.

RAN2 to confirm with RAN1 if the current agreement entails that two separate DL/UL TCI state pairs can be configured and indicated, for where a TCI codepoint associated with one DL TCI state is only used updating one of the two indicated DL TCI states for mTRP operation in Rel-19

Currently, Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate or Unified TCI States are used for indicating Activation/Deactivation of TCI state(s) for PDCCH reception for a CORESET of a Serving Cell or a set of Serving Cells [38.321, 6.1.3.71]. The PL offset in Asymmetric DL/UL can be associated to an UL TCI state, and the UE should determine the output power for transmitted UL signals/channels associated with the UL TCI state by applying this PL offset to the open loop power control. The impact to existing legacy PHR trigger from this is discussed in 2.2.3 below.

As a result of TCI State association, for the PL Offset signalling, upon activation/deactivation of TCI States, the legacy TCI States Activation/Deactivation MAC CE CEs could be reused by incorporating also an associated PL Offset.

The legacy Enhanced Unified TCI States Activation/Deactivation MAC CE could be extended for signalling of PL Offset (and PL RS)

A resulting MAC CE example using The Enhanced Unified TCI States Activation/Deactivation MAC CE CE for Separate TCI States can be found in fig. 2. The Ti in this example would indicate the PL Offset for a TCI state with TCI-StateId “i” (c.f. 38.331) if needed. If the PL Offset are statically mapped to the TCI state, those may not be needed. Additionally, the number of bits for the PL Offset needs to be considered, i.e the granularity needed for the PL Offset signaling. The corresponding examples on additions using other legacy MAC-CE formats would apply.

Fig. 2

In the above, adding the PL Offset and additional fields to the TCI Activation/deactivation MAC CE results however in several new MAC CE formats for the UE to support along with large MAC CE sizes. Considering also that the PL Offset is initially RRC configured, it seems natural to agree to a baseline that defines the PL Offset outside of the MAC CEs used for activation/deactivation of TCI States (Note that in the figure, the actual bits used for the PL Offset does not represent the current assumption in RAN1, rather the principle signalling format).

A PL Offset update MAC CE would have lower impact on MAC CE signalling.

Example of MAC CE to update PL offset for activated TCI states for mTRP operation:

Fig 3

The Ti in this example indicate if the UL or Joint TCI state with TCI-StateId “i” (c.f. 38.331) should have an updated pathloss offset or not. The format can of course be optimized, e.g. using R-bits, although a baseline could be to represent a number N equal to the maximum number of Joint/UL TCI states configured. This also since it is yet not clear currently, what limitation exist in how many PL Offsets the UE will be able to maintain etc. For example, if the number of PL Offset is signalling is two (2), a MAC CE resulting example could be as in Fig x+3. Additionally, if the required bits for the PL Offset is lower, the TCI-StateId fields can be included in the PL Offset directly (not in example).

PL Offset update signalling MAC CE(s) is defined that includes a TCI-State Id association to activated joint/TCI States.

Fig 4

In the Unified TCI state framework, a path loss reference (PL-RS) can be associated with an UL TCI state through RRC configuration. The UE use this PL-RS associated with an UL TCI state when transmitting UL signals/channels associated with that UL TCI state. In an asymmetric deployment it is however assumed that uplink power control is based on the pathloss measurement using a DL PL-RS that may be the same as for a reference TRP; for instance, the joint/UL TCI State of the TRP that provides the DL service.

Considering mobility also in asymmetric Rel-19 deployments, the DL-TCI state towards the current serving TRP may thus need to be updated frequently. Similarly, the pathloss reference signal and PL Offset that the UE uses may also need to be updated frequently. Since the pathloss reference signal for the UL TCI state targeting the UL-only TRP is currently assumed to be RRC configured, this may require RRC signalling to frequently update the pathloss reference signal towards the serving TRP.

As a result, the MAC CE used for updating the PL Offset may need to also include an association to a PL-RS.  It is however not clear yet in RAN1 what the details are w.r.t PL-RS, i.e. to what combination of e.g. PL offset, the PL offset association to an UL TCI state (UL) and joint DL TCI state (serving TRP) while the PL-RS is related to the joint TCI state of a serving TRP. Here, we suggest waiting for more progress in RAN1. The addition to MAC CE signalling can be rather straightforward and like that of PL Offsets, see example in fig 5.

Fig 5 MAC CE to update two pathloss offsets, and one single DL PL RS (common to the pathloss offsets)

Note that since a PL offset is typically derived based on SRS received at anchor TRP and UL-only TRP, it will be associated to a joint TCI state (anchor) and an UL TCI state (UL-only). I.e

- PL offset 1 can be associated to activated pair of {joint TCI 1, UL TCI 1}

- PL offset 2 can be associated to activated pair of {joint TCI 3, UL TCI 1}

Fig 6 …

As in legacy, UE can have 8 activated TCI state pairs for mTRP, {DL/UL  DL/UL} , i.e., each TRP contains a DL/UL pair. For the Asymmetric case (UL-only), UE may then also have 8 activated TCI state pairs, each pair as {DL/UL TCI, UL TCI}, where the PL offset should be associated to a pair of activated {DL/UL,  UL} TCI states. If in total only 8 activated pairs, then the MAC CE should update 8 PL offsets:

Fig 7 MAC CE to update pathloss offsets, 8 activated pairs , each pair as {DL/UL TCI, UL TCI}

The MAC CE used for updating the PL Offset, may need to include an association to (a) PL-RS as mobility may result in that the pathloss reference signal and PL Offset that the UE uses need to be update frequently

RAN2 to study MAC CE signalling options for PL Offset update while waiting with detailed design until RAN1 progress related topics sufficiently.

### 2.2.2	PHR

In asymmetric DL sTRP/ UL mTRP deployment an UL TRP is receive only and hence does not transmit any DL signals. Without a DL PL-RS (reference) transmission, the pathloss to the UL-only cannot be estimated to determine the UL transmit power for the transmission towards the UL-only TRP. However, it is possible for a UE to determine the pathloss based on the pathloss of the serving (DL) TRP using the pathloss offset (PL∆ [dB]) indicated by the NW.

UL/DL TRP

UL only TRP

UE

PUSCH/PUCCH/SRS

DL-RS for path loss calculations

PL0

PL1

The current formula is thus updated to incorporate the determination for pathloss so that:

P=P0 +  α*(PLDL_RS+PL∆)+ PCL

PL∆=PL1-PL0

Fig. 6

Potential changes/impact to 38.321:

Concerning impact in 38.321, the PHR trigger may have impact from the addition of a PL Offset. In 38.321 the current trigger includes a change to pathloss more than PowerFactorChange DB.

The question is then if the legacy PHR triggering condition should consider the per-TCI state PL offset value.

However, the UE should determine the output power for transmitted UL signals/channels associated with the UL TCI state by applying this PL offset to the open loop power control.  This as the UL Tx power the UE should use changes with the PL offset. Thus, the UE should trigger a PHR when the UL Tx power changes more than PowerFactorChange +/- PL Offset.

If the above is the case, for Rel-19, either a new threshold configuration may be needed, e.g.  “phr-Tx-PowerFactorChange-Asym” should there be a need for different threshold values, or if not, a possibility is to use a conditional presence of the PL Offset {plOffset} as different PL offset values may be configured to TCI-states acc. to the RAN1 agreement for asymmetric mTRP deployment, and due to that the triggering calculation is based on the use on the serving DL/UL TCI State and the corresponding reference PL-RS. That is, if plOffset is configured for an associated PL-RS, the path loss calculation follows the updated formula that includes a PL Offset (according to the configuration, or as updated with MAC CE signalling) and this path loss calculation result is then measured toward a updated “PowerFactorChange” threshold.

As RAN1 have not yet decided if other trigger then for PHR Type 1 is to be supported for UL Only, as well as details w.r.t the configurable combinations that include DL sTRP or UL mTRP scenarios (e.g. twoPHRMode, multipanelSchemeSDM, multipanelSchemeSFN etc), RAN2 should wait with the detailed actual procedure impacts. However, it may be beneficial to know to what extent or if the current trigger and conditions can be maintained from a RAN2 p.o.v. Below find some initial examples how the above could be specified.

RAN2 to discuss impact on legacy triggers and thresholds if a PL Offset trigger is needed for asymmetric mTRP.

Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The legacy Enhanced Unified TCI States Activation/Deactivation MAC CE could be extended for signalling of PL Offset (and PL RS)

Observation 2	A PL Offset update MAC CE would have lower impact on MAC CE signalling.

Observation 3	The MAC CE used for updating the PL Offset, may need to include an association to (a) PL-RS as mobility may result in that the pathloss reference signal and PL Offset that the UE uses need to be update frequently

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm with RAN1 if the current agreement entails that two separate DL/UL TCI state pairs can be configured and indicated, for where a TCI codepoint associated with one DL TCI state is only used updating one of the two indicated DL TCI states for mTRP operation in Rel-19

Proposal 2	PL Offset update signalling MAC CE(s) is defined that includes a TCI-State Id association to activated joint/TCI States.

Proposal 3	RAN2 to study MAC CE signalling options for PL Offset update while waiting with detailed design until RAN1 progress related topics sufficiently.

Proposal 4	RAN2 to discuss impact on legacy triggers and thresholds if a PL Offset trigger is needed for asymmetric mTRP.

Proposal 5	Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023