3GPP TSG-RAN2 Meeting #128	R2-2410302

# Orlando, US, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4882 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of offsetThresholdTA-r18 for NR ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT, Nokia, Nokia Shanghai Bell, LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The unit of symbols is used for ATG in offsetThresholdTA-r18. But the duration of symbol changes with the change of the SCS. It is unclear in the current RRC spec whether the configured offsetThresholdTA-r18 changes, if the SCS changes with the BWP switching. Technicaly, the configured threshold should remain unchanged unless reconfigured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the field description of offsetThresholdTA-r18 that for ATG the offset is based on the SCS of the active BWP when the TAR-Config is applied and doesn't change even if the SCS changes with BWP switching. The proposed wording means that in the handover case the SCS is based on the target cell’s BWP, where the configuration is applied. Impact analysis Impacted functionality: NR ATG Inter-operability: If the UE is implemented according to this CR but the network is not, there is a possible misalignment in the NW behaviour whether the offset threshold changes with BWP. If the network is implemented according to this CR but the UE is not, there is a possible misalignment in the UE behaviour whether the offset threshold changes with BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear on which SCS the TAR-Config offset is based in ATG when BWP switching happens. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Wording changed to “when the TAR-Config is applied” to clarify that in the HO case the SCS is based on the target cell’s BWP. Rev 2: Update based on option 1b from offline discussion [AT127bis][013][ATG] CRs (Samsung). |  |  |  |  |  |  |  |  |

Start of changes

### 6.3.2	Radio resource control information elements

Unchanged Text is omitted

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. Network only configures offsetThresholdTA-r18 for ATG, which is in unit of symbols based on the SCS of the active BWP of the SpCell when TAR-Config is applied. Value of offsetThresholdTA-r18 does not change with BWP switching and the UE assumes the BWP switching does not occur when TAR-Config is applied. During handover, offsetThresholdTA-r18 is based on SCS of the BWP indicated by firstActiveDownlinkBWP-Id.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

End of changes