3GPP TSG-RAN WG2 #128	R2-2410313

Orlando, US, 18th – 22nd, November 2024

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on study of AIoT random access

Document for:	Discussion and decision

Agenda Item:	8.2.4

- Introduction

The SI: FS_ Ambient_IoT_Solutions_RAN is planned to be closed in this meeting [1]. This contribution provides our views on remaining issues to conclude the study on A-IoT random access.

- Discussion

  - Down selection for RA types (2-step, 3-step, CFRA)

RAN2 has agreed to study three types of A-IoT random access, 2-step CBRA, 3-step CBRA and CFRA. In this section, we provide our observations about these RA types and our view on which RA type to prioritize in WI phase.

- The 3-step CBRA is robust and resource-efficient random access. Comparing to 2-step CBRA, the device transmits the random ID and upper layer data including permanent device ID in multiple D2R messages as Figure 1 depicts, that means the size of each message is smaller than Msg1 for 2-step CBRA. Moreover, since the message size of Msg1 for 2-step CBRA is larger than that of 3-step CBRA, the reader to allocate larger size of resources for every access occasion for Msg1 transmission, which consumes much more resources in total, than 3-step CBRA.

Figure 1. Comparison between 3-step CBRA and 2-step CBRA.

Observation 1.	The 3-step CBRA is more robust and more resource-efficient than 2-step CBRA.

- The CFRA has the least flexibility on targeting devices. In the CFRA, the reader has to explicitly allocate a resource of access occasion for D2R transmission to each targeted device respectively. We understand that the group ID cannot be used in CFRA paging since it may be difficult to associate the indicated resources with devices in the group, and moreover, the reader has no idea which devices in the group are in the proximity of the reader itself. Therefore, the reader has to indicate targeted device(s) using corresponding ID(s) of targeted device(s). Considering that the transport block size for A-IoT air interface is limited approximately to 1,000 bytes, even the number of targeted devices will be limited in CFRA.

Figure 2. Comparison of A-IoT paging for CBRA and CFRA.

Observation 2.	The indication of CFRA is less flexible than 3-step CBRA and 2-step CBRA since the reader has to indicate targeted devices only by using corresponding IDs of targeted devices.

Therefore, we suggest deprioritizing CFRA and focus on 3-step CBRA at the beginning of the normative phase. It would be beneficial to specify 2-step CBRA if there remains enough time until Rel-19 completion.

Proposal 1.	RAN2 prioritizes 3-step CBRA for normative work in WI phase. 2-step CBRA is specified if time allows.

  - Failure/success indication for D2R message

In RAN2#127-bis, we have concluded to support explicit R2D failure/success feedback for Msg3. There is a still FFS on whether to support similar indication for D2R data transmission after successful random access.

RAN2#127-bis [2] 2 Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data.

Considering that we agreed to study segmentation in the MAC for D2R message, there is a case where the reader is waiting for reception of another segment when the segmentation is indicated in the previous D2R message. For instance, when the first segment is successfully received by the reader, but the second segment is not received, the failure detection of D2R data and re-access to fetch the missed second segment will be beneficial in order to utilize the first segment buffered in the reader side without restating the reception from the first segment.

Proposal 2.	Support optional explicit failure/success feedback indication for D2R data after successful random access completion.

- Summary and proposal

Observation 1.	The 3-step CBRA is more robust and more resource-efficient than 2-step CBRA.

Observation 2.	The indication of CFRA is less flexible than 3-step CBRA and 2-step CBRA since the reader has to indicate targeted devices only by using corresponding IDs of targeted devices.

Proposal 1.	RAN2 prioritizes 3-step CBRA for normative work in WI phase. 2-step CBRA is specified if time allows.

Proposal 2.	Support optional explicit failure/success feedback indication for D2R data after successful random access completion.

- References

[1]	RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN#103.

[2]	Chair’s notes, RAN2#127-bis.