3GPP TSG-RAN2 Meeting #128			R2-2410316

Orlando, USA, Nov. 18th-22nd, 2024

Agenda item:	8.7.4.1

Title:	Consideration on fairness in delay-aware LCP for XR

Source:	CMCC, Qualcomm Incorporated

Document for:	Discussion and decision

Introduction

In RAN2#126 [1], RAN2 agrees to use additional priority for delay-aware LCP and leaves an FFS for whether additional priority can be applied to non-delay-critical data.

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

In RAN2#127, LCP restriction is excluded, but there is no further discussion on whether the additional priority can be applied to non-delay-critical data. Later in RAN2#127bis [2], concerns raise regarding fairness of prioritizing non-delay-critical data.

FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

This contribution will discuss the fairness concerns caused by applying additional priority to non-delay-critical data and try to explore some potential enhancement to ensure fairness in delay-aware LCP.

Discussion

To our understanding, this WI aims to avoid too late PDUs being discarding for XR, while the impacts on other LCHs should be mitigated.

  - Fairness issue in delay-aware LCP

In a delay-critical LCH, the distribution of delay-critical and non-delay-critical data in buffer queue is uncertain, i.e., non-delay-critical data can be positioned ahead, behand, or interleaved with delay-critical data, given that the data in the LCH may have different discard timers and queueing time.

Considering the previous agreement that LCP prioritization within a logical channel will not be considered, all data, delay-critical or not, can use the same additional data and therefore become LCH priority-adjusted data. Hence, the concerns about fairness for delay-aware LCP mainly arise from the application of additional priority to non-delay-critical data.

Regarding to current LCP in TS38.321, in the resource allocation regardless of Bj (or 2nd round of resource allocation), MAC entity shall allocate all resource before either data from LCH or UL grant is exhausted in a decreasing priority order. Therefore, in cases where radio resources are insufficient, e.g., RAN is about to overload, applying additional priority to non-delay-critical would have significant impacts on any other LCHs whose priority is between the original and additional priority of delay- critical LCH.

For example, when resources are not enough, UE MAC entity may only allocate resources no more than Bj to non-delay-critical LCH j. The reason that there is no resource for LCH j in the remaining resource allocation is either UE MAC entity allocated all of remaining resource to delay-critical LCH with non-delay-critical data or there is just no remaining resource.

If the additional priority is high enough, delay-aware LCP can even affect GBR flow and SRB. Because in order to send delay-critical data as soon as possible, the additional priority could be configured no lower than any DRB. From our perspective, it will cause serious fairness issue.

Observation 1: If the additional priority is applied to non-delay-critical data, it may have significant negative impacts, e.g., discarding and starvation on other LCHs, even GBR flow and SRB.

The affected flows can be audio, pose or haptic data. They are also very important to XR service. And it cannot be guaranteed that these LCHs will be configured with additional priority.

Proposal 1: RAN2 agrees the negative impact of applying additional priority to non-delay-critical data on other LCHs and delay-critical data should be avoided for fairness.

With that, it would be beneficial to mitigated the impact on other LCH while avoiding PDU being discarded as much as possible.

  - Enhancement for fairness in delay-aware LCP

With the emerging concerns about fairness in delay-aware LCP, several solutions have been proposed to ensure fairness.

For example, in [3], it is proposed to re-assign delay-critical data to a new LCH with additional priority while the original LCH retains its original priority. Therefore, additional priority will not be applied to non-delay-critical data.

A noteworthy issue is that both of the aforementioned methods seem to assume that UE has the capability to identify and separate delay-critical data and non-delay-critical data in LCH. Only then UE is able to either map delay-critical data to new LCH or multiplex them into certain MAC PDU. This issue can be left to UE implementation since the buffering and queuing strategies may vary among different UE vendors.

As an example, a maybe feasible approach is UE to determine whether an SDU will become delay-critical during MAC PDU multiplexing based on its remaining time (with an enlarged remaining time threshold to compensate queuing). The data within an LCH can be divided into potential delay-critical and potential non-delay-critical data. Two separate buffer queues are then introduced for these two kinds of data. Data can subsequently be dequeued from these two queues and mapped to different LCHs or multiplexed into specific MAC PDUs accordingly, as shown in the following figure.

Figure 1: An example of separating delay-critical data and non-delay-critical data in LCH

UE implementation can handle it without much specification impact.

Proposal 2: Delay-critical data and non-delay-critical data should be separately handled during LCP priority adjustment. The exact method for the identification and separation of delay-critical data and non-delay-critical data is up to UE implementation

Conclusion

This contribution discusses the fairness issue in delay-aware LCP and provides the following for further discussion in RAN2:

Fairness issue aspect:

Observation 1: If the additional priority is applied to non-delay-critical data, it may have significant negative impacts, e.g., discarding and starvation on other LCHs, even GBR flow and SRB.

Proposal 1: RAN2 agrees the negative impact of applying additional priority to non-delay-critical data on other LCHs and delay-critical data should be avoided for fairness.

Proposal 2: Delay-critical data and non-delay-critical data should be separately handled during LCP priority adjustment. The exact method for the identification and separation of delay-critical data and non-delay-critical data is up to UE implementation

References

- R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC, Fukuoka, Japan

- R2_127bis R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM

- R2-2408130, Discussion on LCP enhancements, Qualcomm Incorporated