3GPP TSG-RAN WG2 Meeting #128	R2-2410338

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	8.7.3

Source:	CMCC, Qualcomm, Google

Title:	RRC-based MG Skipping Solution

Document for:	Discussion

# 1 Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved. According to the WID, RRM measurement gaps/restrictions related enhancements for XR was approved [1] with the following objectives:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

Last RAN2#127bis meeting is the first meeting for the discussion on RRM measurement gaps/restrictions enhancements for XR in RAN2 and achieved the following agreements [2].

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this contribution, we would provide our view on the additional MG skipping solution (e.g., RRC-based solution) for enabling TX/RX for XR during RRM measurements.

# 2 Discussion

According to the conclusion in RAN1#118 meeting [3], scheduling DCI solution is agreed for enabling Tx/Rx in particular gap/restriction that are caused by RRM measurements.

For solutions based on triggering/enabling by network signalling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

DCI-based solution exhibits the most flexibility to skip a particular gap/restriction that are caused by RRM measurements. It is suitable to be used for dynamic data scheduling, especially for the uncertainty handling of the size of XR data bursts and/or the arrival time of XR traffic with intense jitter, which could lead to the unfixed overlapping between XR data and gaps/restrictions.

Observation 1: DCI-based solution is suitable to be used for dynamic data scheduling, especially for the uncertainty handling of the size of XR data bursts and/or the arrival time of XR traffic with intense jitter.

However, DCI-based signaling as dynamic indication brings flexibility at the expense of signaling overhead and UE implementation complexity. Since the network has to send a scheduling DCI before the gap/restriction occasion coming and accordingly UE has to monitor PDCCH carrying the DCI format indicating whether to skip the coming gap/restriction. This situation may get worse when semi-static scheduling (i.e. CG and SPS) is used for XR service.

In Rel-18, CG used for UL XR traffic and SPS used for DL XR traffic were considered as important scheduling mechanisms for XR. When CG/SPS is used for XR traffic, the network has to send a scheduling DCI every time before CG/SPS occasion coming and UE has to continuously monitor PDCCH. Continuous PDCCH monitoring will negatively impact UE implementation and energy cost.

Observation 2: In the case where the arrival time of XR traffic is periodic or CG/SPS scheduling is used, DCI-based solution will negatively impact signaling overhead, UE implementation and energy cost.

Therefore, we think the semi-static solution (e.g., RRC signaling) to enable TX/RX in gaps/restrictions need to be considered as an additional MG skipping solution for the case where the arrival time of XR traffic is pre-determined periodic or CG/SPS scheduling is used. The following options of the semi-static solution could be considered:

- Option 1: Positions of the MGs/SMTC windows that are used for measurement, positions of the MGs/SMTC windows that are used for data scheduling, or a use pattern of MGs/SMTC windows in a time period T

- Option 2: A new periodicity P1, which is larger than the periodicity P of MGs/SMTC windows

For Option 1, which MGs/SMTC windows in the time period T could be used for measurement/data scheduling, or the use pattern of MGs/SMTC windows in the time period T is configured to the UE via RRC signaling. In general, the time period T can be a fixed time, such as 200ms, 600ms, etc, or it can be K multiples of the periodicity of MGs/SMTC windows, where K is a nonnegative integer.

Option 2 provides a larger periodicity P1, which means that the periodicity of MGs/SMTC windows that are actually used for measurement is larger than that originally configured. For example, P1=KP is provided to UE such that only one of the K MGs/SMTC windows is used for measurement. Compared to Option 2, Option 1 has higher flexibility of deactivating MGs/SMTC windows but larger signaling overhead.

Figure 1 and Figure 2 provide examples for Option 1 and Option 2, respectively. As shown in Figure 1, there are 3 MGs within the time period T, and the UE is instructed to measure in MG#1 and receive the DL XR packets in MGs#2 and #3. As depicted in Figure 2, the periodicity of the MGs that are actually used for measurement, i.e., P1=60ms is informed to UE. So, UE will not perform measurements in MG#2 and MG#3.

Figure 1. An example of Option 1

Figure 2. An example of Option 2

For the arrival time of XR traffic is periodic or CG/SPS scheduling is used, based on the semi-static solution (e.g., RRC signaling), the unnecessary signaling (scheduling DCI) and continuous PDCCH monitoring could be avoided. It is helpful to reduce signaling overhead and UE energy cost.

Observation 3: For the arrival time of XR traffic is periodic or CG/SPS scheduling is used, the semi-static solution (e.g., RRC signaling) is helpful to avoid the unnecessary signaling overhead and continuous PDCCH monitoring.

Proposal 1: The semi-static solution (e.g., RRC signaling) could be considered as an additional MG skipping solution to enable TX/RX in gaps/restrictions.

DCI-based and RRC-based solution could be used in different scenarios, e.g., DCI-based solution is used for dynamic data scheduling and RRC-based solution is used for the periodic arrival time of XR traffic or CG/SPS scheduling. When there is collision between DCI signaling and RRC signaling, DCI signaling has a higher priority. That means if the pattern of MGs (e.g., MG#1 can be used for measurement) is pre-configured to UE by RRC signaling, but UE monitors the DCI signaling to indicate to skip MG#1, then UE perform TX/RX during MG#1 based on DCI signaling.

Proposal 2: DCI signaling and RRC signaling are co-existing MGs skipping solutions to enable TX/RX in gaps/restrictions and DCI signaling has a higher priority.

- DCI-based solution could be used for dynamic data scheduling;

- RRC-based solution could be used for the periodic arrival time of XR traffic or CG/SPS scheduling.

Proposal 3: Consider the following information to be indicated to UE by the semi-static solution (e.g., RRC signaling):

- Alt 1: Positions of the MGs/SMTC windows that are used for measurement in a time period;

- Alt 2: Positions of the MGs/SMTC windows that are used for data scheduling in a time period;

- Alt 3: A use pattern of MGs/SMTC windows in a time period;

- Alt 4: A single periodicity parameter of MGs/SMTC windows that are actually used for measurement.

# 3 Conclusion

In this paper, we provide our views on the additional MG skipping solution (e.g., RRC-based solution) for enabling TX/RX for XR during RRM measurements, we have the following observations and proposals:

Observation 1: DCI-based solution is suitable to be used for dynamic data scheduling, especially for the uncertainty handling of the size of XR data bursts and/or the arrival time of XR traffic with intense jitter.

Observation 2: In the case where the arrival time of XR traffic is periodic or CG/SPS scheduling is used, DCI-based solution will negatively impact signaling overhead, UE implementation and energy cost.

Observation 3: For the arrival time of XR traffic is periodic or CG/SPS scheduling is used, the semi-static solution (e.g., RRC signaling) is helpful to avoid the unnecessary signaling overhead and continuous PDCCH monitoring.

Proposal 1: The semi-static solution (e.g., RRC signaling) could be considered as an additional MG skipping solution to enable TX/RX in gaps/restrictions.

Proposal 2: DCI signaling and RRC signaling are co-existing MGs skipping solutions to enable TX/RX in gaps/restrictions and DCI signaling has a higher priority.

- DCI-based solution could be used for dynamic data scheduling;

- RRC-based solution could be used for the periodic arrival time of XR traffic or CG/SPS scheduling.

Proposal 3: Consider the following information to be indicated to UE by the semi-static solution (e.g., RRC signaling):

- Alt 1: Positions of the MGs/SMTC windows that are used for measurement in a time period;

- Alt 2: Positions of the MGs/SMTC windows that are used for data scheduling in a time period;

- Alt 3: A use pattern of MGs/SMTC windows in a time period;

- Alt 4: A single periodicity parameter of MGs/SMTC windows that are actually used for measurement.

# Reference

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm (Rapporteurs).

- R2_127bis R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM, Oct 2024.

- Draft_Minutes_report_RAN1#118_v030, Aug 2024.