3GPP TSG-RAN WG2 Meeting #128	R2-2410339

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	8.3.2.1

Source:	CMCC

Title:	Simulation results for RRM measurement prediction

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In the previous RAN2 meeting, there were discussion on Simulation assumptions and evaluation methodology for RRM measurement prediction. The agreements about simulation assumptions for RRM measurement prediction were achieved [1][2][3] with the following agreed evaluation scenarios to start the relevant simulations.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In last RAN2#127bis meeting [4], the following agreements on intra-frequency temporal domain prediction (Case 2/4) and inter-frequency prediction (Case 3) were achieved. For intra-frequency temporal domain prediction, companies should provide the last value at the end of the PW.

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.  The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

In this contribution, we provide our updated simulation results of intra-frequency temporal domain prediction (Case 2/4) with the last value at the end of the PW. And we also provide our initial consideration on potential specification impact for RRM measurement prediction.

# 2 Discussion

2.1 Simulation assumptions of the evaluation scenario

2.1.1 RRM sub case

In our simulation, we use RRM sub case 2 as starting point. The measurements of L3 cell level RSRP are used as AI/ML model input to predict L3 cell level RSRP results.

Figure 1: RRM sub case 2

2.1.2 L1/L3 filtering

During RAN2#127 meeting, RAN2 concluded that there are two L1/L3 filtering options as shown below that could be used in the simulations of RRM measurement prediction and up to company to report. In our simulation, we use non-sliding L1/L3 filtering option in Case 2-FR1 to FR1 intra-frequency temporal domain case B (measurement result are generated every 200ms), and use sliding L1/L3 filtering option in Case 4-FR2 to FR2 intra-frequency temporal domain case A (measurement result are generated every 80ms).

Figure 2: Sliding L1/L3 filtering [5]

Figure 3: Non-sliding L1/L3 filtering [5]

2.2 Simulation results and analysis

2.2.1 Case 2-FR1 to FR1 intra-frequency temporal domain case B

In our evaluation of case B, we use temporal domain case example 2 and simulate to compare the RSRP difference at different MRRT with the same UE speed (90km/h).

Table 1: The key simulation assumptions in Case 2

| Data Size (Number of Samples) | Training/validity | 13000,1625 |
| --- | --- | --- |
|  | Testing | 1625 |
| AI/ML model input/output | Model input | Cell level RSRP of all cells |
|  | Model output | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |
|  | Model complexity in a number of parameters(M) | 24181 |
|  | Model complexity in model size (e.g. Mbyte) | 0.093MB |
|  | Computational complexity [FLOPs] | 292.66K |

The simulation results of Sub-case 2 in Case 2-FR1 to FR1 intra-frequency temporal domain case B is shown in Table 2.

Table 2: The simulation results of Sub-case 2 in Case B

| Set No. | UE Speed | OW vs PW | MRRT | Average RSRP-Differ (dB) | The last RSRP-Differ(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 90 km/h | N:N | 1/2 | 0.877 | 0.877 |
| 2 | 90 km/h | N:2N | 2/3 | 1.344 | 1.698 |
| 3 | 90 km/h | N:3N | 3/4 | 1.754 | 2.448 |
| 4 | 90 km/h | N:4N | 4/5 | 2.133 | 3.060 |

Based on the above simulation results, it could be observed that the last measurement result at the end of the prediction window has the maximum RSRP difference which is above the Average difference. And with the increase of prediction window from N to 4N, the last value of RSRP difference increase from 0.877dB to 3.060dB.

Observation 1: In our simulation, in the case that UE speed is 90 km/h, the average L3 cell level RSRP difference increases from 0.877dB to 2.133dB with the increase of MRRT from 1/2 to 4/5.

Observation 2: With the same UE speed, the prediction accuracy for intra-frequency temporal domain case B reduces as the increase of MRRT.

Observation 3: When the MRRT is up to 2/3 and above, the average RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5.

Observation 4: For intra-frequency temporal domain case B, the last measurement result at the end of the prediction window has the maximum RSRP difference.

Observation 5: With the increase of prediction window, the prediction accuracy of the last value at the end of the prediction window deteriorates seriously.

Proposal 1: RAN2 to define the threshold of the prediction window (e.g., 3N) and MRRT (e.g., 3/4) for intra-frequency temporal domain case B to avoid the prediction accuracy deteriorate seriously.

2.2.2 Case 4-FR2 to FR2 intra-frequency temporal domain case A

In our evaluation, we simulate to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

Table 3: The key simulation assumptions in Case 4

| Data Size (Number of Samples) | Training/validity | 208656,23184 |
| --- | --- | --- |
|  | Testing | 33810 |
| AI/ML model input/output | Model input | Cell level RSRP of all cells |
|  | Model output | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 868,560 |
|  | Model complexity in model size (e.g. Mbyte) | 10.5MB |
|  | Computational complexity [FLOPs] | 1300.08K |

The simulation results of Sub-case 2 in Case 4-FR2 to FR2 intra-frequency temporal domain case A is shown in Table 4.

Table 4: The simulation results of Sub-case 2 in Case A

| Set No. | UE Speed | OW vs PW | Average RSRP-Differ (dB) | The last RSRP-Differ(dB) |
| --- | --- | --- | --- | --- |
| 1 | 90 km/h | N:N | 0.949 | 0.949 |
| 2 | 90 km/h | 2N:2N | 0.855 | 1.014 |
| 3 | 90 km/h | 3N:3N | 0.947 | 1.314 |

Based on the above simulation results, it could be observed that the last measurement result at the end of the prediction window has the maximum RSRP difference. But the prediction accuracy of the last value does not deteriorate obviously compared with the Average RSRP difference when OW and PW have the same length, even if the UE speed is up to 90km/h.

Observation 6: For intra-frequency temporal domain case A, the last measurement result at the end of the prediction window has the maximum RSRP difference.

Observation 7: The prediction accuracy of the last value does not deteriorate obviously compared with the Average RSRP difference when OW and PW have the same length, even if the UE speed is up to 90km/h.

Observation 8: Keeping OW and PW have the close length is helpful to ensure the prediction accuracy for intra-frequency temporal domain case A.

Proposal 2: For intra-frequency temporal domain case A, keep OW and PW have the close length to ensure the RRM measurement prediction accuracy.

2.3 Potential specification impact

Considering the evaluation progress of RRM measurement prediction, we think we could consider to start the study on potential specification impact of RRM measurement prediction.

Based on the simulation results from companies during the previous RAN2 meetings, we can observe that both intra-frequency temporal domain Case A and Case B for RRM measurement prediction based on cell level results could achieve stable prediction performance. Therefore, the specification impact could be further considered to support intra-frequency temporal domain of RRM measurement prediction for measurement overhead reduction or handover performance enhancement.

Intra-frequency temporal domain (i.e., Case A and Case B) for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

For intra-frequency temporal domain Case A, it focus on the enhancement for handover performance and no reduction in measurement overhead in this case. An example of Case A is shown in Figure 4.

Figure 4: Temporal domain Case A example [5]

For intra-frequency temporal domain Case B, it focus on measurement overhead reduction. Two examples of Case B are shown in Figure 5 and Figure 6.

Figure 5: Temporal domain Case B example 1 [6]

Figure 6: Temporal domain Case B example 2 [6]

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR;

Proposal 3: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

Proposal 4: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR.

# 3 Conclusion

In this paper, we provide our updated simulation results of intra-frequency temporal domain prediction (Case 2/4) with the last value at the end of the PW and we also provide our initial consideration on potential specification impact for RRM measurement prediction. We have the following observations and proposals:

Case B: to compare the RSRP difference at different MRRT with the same UE speed

Observation 1: In our simulation, in the case that UE speed is 90 km/h, the average L3 cell level RSRP difference increases from 0.877dB to 2.133dB with the increase of MRRT from 1/2 to 4/5.

Observation 2: With the same UE speed, the prediction accuracy for intra-frequency temporal domain case B reduces as the increase of MRRT.

Observation 3: When the MRRT is up to 2/3 and above, the average RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5.

Observation 4: For intra-frequency temporal domain case B, the last measurement result at the end of the prediction window has the maximum RSRP difference.

Observation 5: With the increase of prediction window, the prediction accuracy of the last value at the end of the prediction window deteriorates seriously.

Proposal 1: RAN2 to define the threshold of the prediction window (e.g., 3N) and MRRT (e.g., 3/4) for intra-frequency temporal domain case B to avoid the prediction accuracy deteriorate seriously.

Case A: to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

Observation 6: For intra-frequency temporal domain case A, the last measurement result at the end of the prediction window has the maximum RSRP difference.

Observation 7: The prediction accuracy of the last value does not deteriorate obviously compared with the Average RSRP difference when OW and PW have the same length, even if the UE speed is up to 90km/h.

Observation 8: Keeping OW and PW have the close length is helpful to ensure the prediction accuracy for intra-frequency temporal domain case A.

Proposal 2: For intra-frequency temporal domain case A, keep OW and PW have the close length to ensure the RRM measurement prediction accuracy.

Potential specification impact

Proposal 3: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

Proposal 4: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR.

# Reference

- R2_125bis_ChairNotes_24-04-19_final, Apr 2024

- R2_126_ChairNotes_24-05-24_final, May 2024

- R2_127_ChairNotes_24-08-23_17-00_eom, Aug 2024

- R2_127bis_ChairNotes_24-10-18_15-45_eom_clean, Oct 2024

- R2-2407781, Summary of offline [AT127][026][AI Mob] Simulation assumptions, OPPO

- R2-2407885, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2, OPPO