3GPP TSG RAN WG2 #128	R2-2410347

Orlando, USA, November 18 – 22, 2024

Agenda Item:	8.9.3
Source:	Toyota ITC
Title:	Discussion on Uplink Capacity Enhancement

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a work item for Internet of Things (IoT) Non-Terrestrial Networks (NTN) [1]. One focus area is uplink capacity enhancements. This contribution discusses some open issues related to this objective.

# Discussion

## Operating point

The previous meeting addressed the operating point for contention-based access and decided to support Diversity Slotted ALOHA (DSA). It was agreed that, for evaluating potential methods to enhance uplink capacity for CB-msg3 ETD-like transmission, the focus should be on a working point where the packet loss rate is below 10%. The objective of this section is to discuss the selection of the operating point.

Many simulation studies have assessed uplink performance using metrics like throughput, packet loss rate, and delay. These studies often use models of physical and data link layers, but without modeling the application behavior. While the physical layer can be modeled with simplified assumptions, the application layer is challenging due to unpredictable traffic. So far, these studies seem to assume that users have always full buffers, and that retransmissions or packet losses do not affect traffic or application behavior, leading to maximized throughput under high traffic load with high packet loss rate and frequent retransmissions. This seems to correspond to store-and-forward scenarios rather than interactive traffic.

Observation 1: The evaluation models seem to correspond to store-and-forward scenarios rather than interactive traffic.

During early 3G standardization, it was assumed that traffic involved downloading large files via FTP. This resulted in specifying long transmission time intervals, up to tens of milliseconds, optimized for this traffic model. However, with TCP/IP, these long transmission time intervals together with retransmissions caused poor performance due to TCP/IP's slow start. The interactive nature of traffic, where clients acknowledge packets and the server increments transmission rate gradually, meant long delays and retransmissions kept transmissions in slow start, preventing broadband-level data rates.

IoT traffic is unlikely to be dominated by TCP/IP or reach broadband data rates. However, application behavior and traffic sources should be considered, based on lessons from 3G.

While it is difficult to accurately predict or model the nature of IoT or uplink traffic, it is essential to focus on two critical factors that may influence application performance: packet loss rate and delays. Managing delays on the propagation path is challenging especially in a satellite system; therefore, efforts should concentrate on the packet loss rate. To enable uplink enhancements for interactive traffic, it is proposed to keep the packet loss rate as low as possible.

Proposal 1: The operating point for packet loss rate should be 1%.

## UE identity for Msg4 transmission

The previous meeting agreed that the RNTI for scheduling Msg4 is derived from the resource used for contention-based Msg3 EDT transmission. The handling of DSA is still under study, as it involves multiple Msg3 replicas, making it unclear how the agreement aligns with DSA without monitoring multiple RNTIs. This is further complicated by the variable number of replicas, including cases with only one replica.

One solution could be to derive the RNTI from a single replica, aligning the agreement with DSA. However, it is not possible to know in advance which one of the replicas will be successfully received. An approach that can be considered is to reuse the concept of CRDSA, where replicas can convey information about the transmission of other replicas. All replicas could include information about the Msg3 PUSCH time-frequency resource used for the transmission of the first replica, which would then be used as the RNTI for scheduling Msg4. In other words, the RNTI for scheduling Msg4 is based on the information contained by the replicas rather than the PUSCH time-frequency resource where a successfully received replica is transmitted. It is irrelevant for the transmitter to know which specific replica was received, and similarly, the receiver responds uniformly regardless of the replica it received.

The solution's drawback is the added overhead on Msg3 due to extra information needed in all Msg3 replicas. This is not however necessary for the case of single replica. An alternative is to derive the PUSCH time-frequency resource from shared data like UE ID (e.g., S-TMSI), SFN, or other system parameters, or by using a hash of Msg3 content. Both the transmitter and receiver could then implicitly infer it.

Proposal 2: The RNTI for scheduling Msg4 is obtained from the resource used for the first replica. The resource information is either conveyed in Msg3 or inferred implicitly from sources such as the UE identity, like S-TMSI.

## Efficient delivery of Msg4

In legacy EDT, the Msg4 (e.g. RRCEarlyDataComplete for CP-EDT) is unicast, via dedicated RRC signalling.

Contributions [2] and [3] from RAN2-127bis suggested the following for EDT without Msg 1/2:

Proposal from [2]: Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

Proposal from [3]: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).

The delivery of msg4 in a non-unicast way is efficient as this avoids the transmission of separate msg4 messages for each UE which had a msg3 decoded correctly. Besides, most of the fields in the msg4 are common for several UEs, such as for example the fields extendedWaitTime, idleModeMobilityControlInfo, or redirectedCarrierInfo. Contribution [4] also showed that omitting some fields from msg4 does not appear as a worthwhile solution, because the fields may be needed in some circumstances, although optional. In addition, the size of those fields is not critical enough to impact resources.

Observation 2: For EDT without Msg 1/2, the transmission of msg4 in a non-unicast way appears as an efficient solution as it avoids the transmission of separate msg4 for each UE having correctly decoded the msg3.

Proposal 3: For EDT without Msg 1/2, introduce the transmission of msg4 in a non-unicast way.

For msg4, lower indication than RRC appears as more efficient than RRC, as this would reduce overhead because RRC encodings are octet aligned, involving padding. This would be in line with the WID objective, that mentions “efficient delivery (reduced overhead) of msg4”. One open point would be whether to use L1 or MAC CE. However, one of the main point of EDT is that msg4 may also contain user data from the NAS container. This therefore favors the MAC CE solution compared to the L1 solution. One point to consider is that the UE only needs to take into account the NAS container assigned to it. This could be achieved by the UE checking the UE Identity appended in the MAC. If this corresponds to its Identity, the UE reads the UE NAS container. Otherwise, the UE ignores the NAS container. Security implications of this would have to be checked by SA3. This would however be in line with the WID objective.

Observation 3: For EDT without Msg 1/2, the transmission of msg4 at MAC layer seems more efficient than RRC, and more achievable than L1.

Proposal 4: For EDT without Msg 1/2, introduce the transmission of msg4 at MAC layer.

Proposal 5: After checking with SA3, for EDT without Msg 1/2, the UE reads the NAS container appended in the MAC layer only if its identity matches the UE identity signaled at MAC layer.

Proposal 6: Send an LS to SA3 about proposal 3.

# Conclusion

This contribution discusses uplink capacity enhancements. Observation and proposals are summarized as follows:

Observation 1: The evaluation models seem to correspond to store-and-forward scenarios rather than interactive traffic.

Observation 2: For EDT without Msg 1/2, the transmission of msg4 in a non-unicast way appears as an efficient solution as it avoids the transmission of separate msg4 for each UE having correctly decoded the msg3.

Observation 3: For EDT without Msg 1/2, the transmission of msg4 at MAC layer seems more efficient than RRC, and more achievable than L1.

Proposal 1: The operating point for packet loss rate should be 1%.

Proposal 2: The RNTI for scheduling Msg4 is obtained from the resource used for the first replica. The resource information is either conveyed in Msg3 or inferred implicitly from sources such as the UE identity, like S-TMSI.

Proposal 3: For EDT without Msg 1/2, introduce the transmission of msg4 in a non-unicast way.

Proposal 4: For EDT without Msg 1/2, introduce the transmission of msg4 at MAC layer.

Proposal 5: After checking with SA3, for EDT without Msg 1/2, the UE reads the NAS container appended in the MAC layer only if its identity matches the UE identity signaled at MAC layer.

Proposal 6: Send an LS to SA3 about proposal 3.

# Reference

- RP-241624, "Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things", MediaTek Inc., RAN#104, June 2024.

- R2-2409181, RAN2-127bis, UL capacity enhancements for IoT NTN, Ericsson.

- R2-2408304, RAN2-127bis, EDT for uplink capacity enhancement in NTN, Lenovo.

- R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.