3GPP TSG-RAN WG2 Meeting #128	R2-2410348

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	6.1.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on segmented UE capability

Document for:	Discussion and Decision

- Introduction

In the latest RAN2#127bis meeting, RAN2 discussed the issue of UL RRC message segmentation for UECapabilityInformation and agreed that the network can indicate the maximum number of UL segments allowed for the UE in the UE Capability Enquiry message.

Agreements 1) Agree on the signalling solution for or RRC message segmentation of UECapabilityInformation where: - The UE indicates its support for network-requested maximum number of UL segments. - The network indicates the maximum number of UL segments the UE is allowed to use. - The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to the maximum number of UL segments the UE is allowed to use. 2) The proposed new signalling solution is introduced from release-17. 3) The RRC processing delay requirement for UE capability enquiry / transfer procedure when the network indicates the maximum allowed number of UL segments is TBD ms. 4) We will send an LS to RAN3 saying that we plan to specify this feature and that we have a need for CN to indicate the support to the RAN.

It was also agreed that RAN2 needs to decide the RRC processing delay requirement for UE capability transfer procedure if the network indicates the maximum allowed number of UL segments. Thus, in this contribution we would like to further discuss the issues about segmented UE capability.

- Discussion

In RAN2#108 meeting, the following agreement was achieved and captured in the chairman notes:

Chairman notes in RAN2#108 We reconfirm that also in this case "processing time" is defined as the time from when the UE receives the UE capability enquiry to when it's ready to receive the grant to transmit the first segment(s). How many segments depends on the size of the grant

Before any discussion begins, we would like to confirm that the specified RRC processing delay for UE capability refers to the time from when the UE receives the Capability Enquiry message until it is ready to receive the grant to transmit the FIRST segment, to avoid any ambiguity.

Proposal 1: RAN2 confirms that the RRC processing delay for UE capability refers to the time from when the UE receives the Capability Enquiry message until it is ready to receive the grant to transmit the first segment.

In last meeting, it was agreed that for UL RRC segmentation of UE capability information, the network can indicate the maximum number of UL segments allowed to be used by the UE. We see the CRs endorsed in last meeting are only for NR specs. However, as LTE also uses RRC segmentation for UE capability information based on the same principles as NR, we think same mechanism should be applied for LTE. The new mechanism is also beneficial when the UE has segmented capability information for LTE and MR-DC scenario.

Proposal 2: The mechanism on network indication of maximum number of UL segments applies to both NR and LTE.

Based on the agreement, it is still FFS on the RRC processing delay for UE capability transfer procedure when the network indicates the maximum allowed number of UL segments. Our observations from UE’s practical implementation indicate that the processing delay mainly comes from the ASN.1 encoding of the entire RRC message. In the segmentation scenario, the processing time for the capability enquiry message remains consistent; however, the processing time for encoding the UECapabilityInformation message tends to increase almost linearly with the size of the capability being processed. This increase is largely attributed to the number of band combinations (BCs) and feature sets included in the capability. As the number of BCs and feature sets grows, so does the size of the capability, leading to a corresponding rise in processing time required to handle these elements.

Observation 1: There is a nearly linear relation between the processing time of UE capability and the size of the processed capability signaling.

Since the maximum number of UL segments that the network can indicate is 16, it is advantageous to extend the processing delay of the UE capability procedure to a larger value. Considering the maximum number of UL segments is 16, we propose to extend the RRC processing delay to 800ms in total.

Proposal 3: RAN2 considers to extend the processing time to 800ms for segmented UE capability transfer procedure.

In our understanding, the issue mentioned above is not only relevant to the current discussion regarding the maximum number of segments for UE capabilities indicated by the network, but also persist in the legacy scenario (i.e., without the network indication of a maximum allowed number of UL segments). According to the current spec, in legacy scenario, the encoding of the capability information is also restricted according to the requirement of the NW. For example, the UE shall compile a list of candidate band combinations according to the band filter requested by the NW in order of FrequencyBandListFilter, more and more network filters have been introduced in previous releases as well. In such cases, there remains a challenge for the UE to complete the segmented capability message processing within 80ms. Assuming a maximum of 16 segments are reported, the encoding for such size of around 144Kbytes capability information needs a much longer time, i.e. 800ms.

Moreover, we think extending the RRC processing delay for capability segmentation would not pose any backward compatibility issues, as the network typically allows a longer waiting time for the UL response before considering the capability enquiry procedure as failed. Therefore, we propose that the extended RRC processing delay should be applied starting from Rel-16, to cover the existing capability segmentation mechanism. To accommodate this, the RRC processing delay specified in NR and LTE for UE capability transfer should be adjusted to the above extended value.

Proposal 4: The proposed extended processing delay is introduced for legacy capability segmentation mechanism for NR and LTE from Release-16.

- Conclusion

In this paper, we discussed on the UE capability processing time and make the following proposal:

Observation 1: There is a nearly linear relation between the processing time of UE capability and the size of the processed capability signaling.

Proposal 1: RAN2 confirms that the RRC processing delay for UE capability refers to the time from when the UE receives the Capability Enquiry message until it is ready to receive the grant to transmit the first segment.

Proposal 2: The mechanism on network indication of maximum number of UL segments applies to both NR and LTE.

Proposal 3: RAN2 considers to extend the processing time to 800ms for segmented UE capability transfer procedure.

Proposal 4: The proposed extended processing delay is introduced for legacy capability segmentation mechanism for NR and LTE from Release-16.

- References

Draft Report of 3GPP TSG RAN WG2 meeting #127bis