3GPP TSG RAN WG2 Meeting #128		R2-2410390

Orlando, U.S., 18th – 22nd November 2024

Agenda Item: 	8.7.4

Source: 	Sony

Title: 	UL scheduling enhancements for Multi-modal traffic flows

Document for: 	Discussion & Decision

# Introduction

eXtended Reality (XR) for NR phase 3 work item (WI) was revised in RANP#105 [1]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications.

This WI aims amongst other to facilitate efficient and effective support of scheduling to enable high system capacity, e.g. by relaxing time constraints for resource allocation as far as possible, while meeting delay requirements/avoiding too late PDUs, including the following objective:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

At RAN2#127 [2], the following agreements were made related to Uplink scheduling.

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

At RAN2#127bis [3], the following agreements were made related to LCP enhancements.

Agreements on LCP enhancements

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

- FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

- Introduce an independent per-LCH remaining time threshold for applying delay-critical priority.

- We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

This contribution provides our views on the support for packet delay dependency information between different QoS flows supporting multi-modality.

# Discussion

## Background

XR applications are characterized by high data rate requirements and relatively small packet delay budget (PDB). Therefore, low-latency communication is critical for XR applications. Additionally, XR traffic may consist of multiple flows with different QoS requirements in both directions, uplink and downlink. The traffic may consist of video transmission, audio transmission, data, and tactile information. Currently, Intra-UE prioritization between different flows for uplink, is handled by the logical channel prioritization (LCP) procedure specified in MAC specifications.

Below are the key items related to XR operation for the uplink direction from UE to gNB:

Uplink scheduling:

The MAC layer [4], is amongst other responsible for handling scheduling information, this means configuration from RRC layer with parameters to be considered when preparing UL transmission.

When the UE has prepared a MAC PDU for uplink transmission it will initiate a Scheduling Request (SR) to request resources on UL-SCH (PUSCH).

The MAC CE including the SR is transmitted on PUCCH which carries the Uplink Control Information ( UCI). [5].

The gNB responds with a Downlink Control Information (DCI) to schedule the Uplink resources to be used for UL data transmission.

Buffer Status Reporting (BSR)

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity [4].

The MAC entity triggers transmission of a BSR based on certain conditions like UL data becomes available with a certain priority.

If UL-SCH resources are available for new transmission, these are used to transmit the BSR MAC CE, otherwise the UE will have to transmit an SR to request UL resources.

Delay Status Reporting (DSR)

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of Logical Channels Groups(LCGs).

The delay status is to indicate the remaining time for buffered PDCP SDU, i.e., buffered data packets.

In Rel-18 XR WI, 3GPP introduced DSR MAC CE for UE to report the delay-critical buffer size information to gNB. The purpose is to only indicate delay-critical buffer status, the non-delay critical data can be handled by legacy BSR or refined BSR.

Logical Channel prioritization (LCP)

The LCP procedure is used when UL data transmission is to be transmitted, to prioritize the data transmission related to different logical channels [4].

Multi-modality

For immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization), particularly when the synchronization or time difference  between two or more modalities e.g., between audio and video and tactile  of packets that relatedis higher than a defined latency KPI. [6]

## LCP enhancements:

The purpose of LCP priorities is to handle delay critical data

Some agreements have been made related to LCH enhancements, but the relation between different traffic flows has not been discussed from an LCP point of view, i.e., the interdependency between packets from different QoS flows, belonging to different modalities, like e-g-, audio and video.

What is still a potential problem is how to handle delay critical data belonging to different QoS flow or Logical channels, which have an interdependency in time, where the packets need to be delivered, hence transmitted within a given time budget.

If some sort of packet interdependent time budget isprovided from the application to lower layers, the scheduler would be able to consider when and how to performing LCP prioritization. Basically, the LCP mechanism needs to be enhanced with functionality to handle packet transmission from different Logical channels to some extent overriding the configured priorities for respectively Logical channels, in order to respect the packet interdependency.

To achieve this, the UE should be configured with logical channel interdependence information, and then for each packet, MAC should be able to derive packet interdependent delay information between packets that are belonging to different logical channel buffers. See figure 1.

Figure 1

Proposal 1: MAC should be provided with packet interdependency information for packet belonging to different Logical channels.

Alternatively, since SDAP is the layer responsible for mapping QoS flow to Logical channels, the functionality of potentially creating a new Logical channel where QoS flows related to different modalities are organized in a certain order considering the inter dependency from the packets related to the different flow. What needs to be considered is both the remaining absolute time as well as the delay budget between the different packets, PDUs or PDU sets.

Proposal 2: Alternatively, SDAP has the possibility to analyse different QoS flows and able to combine them into a new multi-modality logical channel.

## DSR Enhancements:

Separately from any LCP enhancements, the DSR functionality may also have to consider any packet interdependency from different QoS flows.

In order for the UE to indicate packet interdependent delay information, the DSR could be updated to include some packet delay budget information. E.g. information such as packet delay budget between specific packets belonging to different Logical channels/QoS flows, and/or between PDU sets.

Observation 1: The DSR feature may benefit from being able to indicate packet inter-dependency information between different logical channels.

For this the DSR MAC CE, could be updated with QoS flow related information, PDU set or PDU specific inter dependent packet delay information.

See figure 2:

Figure 2, DSR MAC CE (with QoS)

Proposal 3: Enhance the DSR MAC CE with packet interdependent QoS flow information.

# Conclusion

In this contribution, we provide our views on the support for packet delay dependency information between different QoS flows supporting multi-modality.

Proposal 1: MAC should be provided with packet interdependency information for packet belonging to different Logical channels.

Proposal 2: Alternatively, SDAP has the possibility to analyse different QoS flows and able to combine them into a new multi-modality logical channel.

Observation 1: The DSR feature may benefit from being able to indicate packet inter-dependency information between different logical channels.

Proposal 3: Enhance the DSR MAC CE with packet interdependent information.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- Minutes RAN2#127, Maastricht

- Minutes RAN2#127bis, Hefei

- 3GPP TS 38.321

- 3GPP TS 38.300

- TS 22.261, Service requirements for 5G system, stage 1 (Release-19)