3GPP TSG-RAN WG2#128												R2-2410395

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.9.2 Support of Store & Forward

Source: 	NEC

Title:	        Radio Interface Aspect of S&F

Document for:	 Discussion

# Introduction

So far we have achieved some agreements to clarify scope ,scenario and architecture regarding S&F:

- RAN2 will adopt SA2 agreed full CN and spit-MME payload options as the baseline for further discussion.

- S&F feature is common for NB-IOT and eMTC,

- S&F feature is applicable to both CP and UP solutions

- consider both single satellite pass and multiple satellite pass scenarios

- consider both MO and MT data within scope

Besides, we also concluded on the details of barring/ S&F indication bit in SIB:

Agreement at RAN2#127bis 1. The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) 2. RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. 3. An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed 4. If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure 5. When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

And note that RAN2 agreed following general procedures at RAN2#126:

Agreements at RAN2#126: 1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available) 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

Based on all above agreements, we share our view on UE AS behaviour and procedure aspects from radio interface point of view.

# Discussion

## Scenario specific issues

SA2 agree split MME and full CN on-board architecture:

- With Split MME on board

Part of MME function will be on satellite, and other part of MME will be on ground.

- With full CN on board

The whole CN will be on board, this may be beneficial NAS procedures, e.g., NAS procedure like attach/TAU procedure might be able to complete during the coverage of one satellite and during one round of RRC connection.

Regardless split MME or full CN architecture, there is no continuous full connection from UE to the data network. i.e. there is no end to end the data transmission connection. Due to lack of end to end data transmission connection, a MO data would need to be sent and stored at satellite when UE firstly connects to a eNB, the responding data from peers would need to be transmitted back to UE when UE secondly connects to the same or a different eNB. A UE connect to satellite for S&F service would inevitably need to reconnect to the same or different satellite to continue the data transmission.

In case of UP-CIOT procedure, the data transmission will rely on stored UE context at UE and network side. if the UE connects to a satellite different from the previous/anchor satellite, context retrieving is needed. Without available feeder link, this will cause further complication and delay. For simplicity, it is better that the UE connects to the same eNB/satellite where its UE context was suspended.

Because SA2 discussed the possibility that MME provides the UE with a list of satellites IDs, during attach/TAU. The UE may uses the satellites in the list for MO/MT data/signalling transfer with the CN. RAN2 could continue discuss AS aspect on how to use a recommended satellite/set of satellite for camping and RRC connection establishment.

RAN2 discuss how UE should be encouraged to camp/reconnect to the same satellite/satellite sets (provided by upper layer) at least when there is unfinished store and forward data/signalling transmission.

Transmission using PUR applicable to NB-IoT UEs currently allows pre-configured resources with periodicity ranging from 8 H-SFN to 8192 H-SFN:PUR-PeriodicityAndOffset-r16 ::= CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..255),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

And the TA validity, which controls by pur-TimeAlignmentTimer, can has a largest length of 8*PUR periodicity.

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

Base on above information, the overall validity of PUR resources could be configured for a relative long duration however in IoT-NTN scenario changing satellites may cause the release of the resources. In this case, UE needs to repeatedly release and request PUR resources, which reduces the benefits of using PUR.

Therefore, if UE is encouraged to camp/reconnect to the same satellite/satellite sets, PUR resources can be made to be valid within the same satellite/satellite sets. During the time gap before the same satellite or satellite from the satellite sets serves the UE again, UE can suspend the PUR resources.

If UE is encouraged to camp/reconnect to the same satellite/satellite sets, consider to let the PUR resources to be valid within the same satellite/satellite sets, which means UE can suspend the PUR resources untill the coverage of the same satellite comes back.

In WID justification, it is mentioned the benefits associated with the support of Store&Forward (S&F) Satellite operation:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

From above justification, a common scenario would be

- From gateway and core network perspective, there is no continuous feeder link availability.

- from UE /service link perspective, There is likely no enough satellite/cells to provide a continuous coverage, which means UE/NW supporting store and forward will experience discontinuous coverage .

Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Considering the cases when there is no neighbour cell, for UE in RRC_IDLE, the network may provide information for operating during discontinuous coverage (e.g., ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage). And UE shall use these discontinuous coverage information to predict its in and out of coverage time window. If UE is out of coverage, it could suspend all AS idle procedure for power saving.

For UE in RRC_CONNECTED, we expect NW will release the UE connection before it is out of coverage. Otherwise, upon declaring RLF, UE may determine not to reestablishment the RRC connection by implementation (e.g., based on ephemeris parameters).

UE(s) who support Store&Forward feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

- For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

- For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

## Basic procedure over radio interface

In general, S&F data transmission will experience two stages:

- When service link is available while feeder link is unavailable, UL/DL data transmission happens over Uu interface. this should be discussed in RAN2, taking into account which part of CN is onboard.

- When feeder link is available while service link is unavailable, UL/DL data transmission happens over Nx interface. What is Nx depending on which part of CN is on board, but Nx interface procedure is obviously out of RAN2 scope and should be discussed in other WG (e.g., RAN3 or SA2) .

RAN2 discussion focus on Uu procedure when service link is available but feeder link is not available.

We agreed the basic procedure for MO as follows RAN2#126, later in RAN2#127, we agree to consider both MO and MT data :

“For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it; 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN; 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available); 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again”

When service link is available but feeder link is not available, there might be either uplink data/signalling at UE side or downlink data/signalling at satellite side waiting for transmission. Fortunately, MT can reuse MO procedure apart from additional paging step.

Hence we could revise the agreement on MO procedure above to cover both MO and MT, and focus on Uu interface only. Following steps could be taken as baseline for S&F satellite operation:

- the eNB send paging to UE if there is stored data/signalling at the eNB for the UE (FFS is a RAN initiated  paging or CN initiated paging).

- the UE establish RRC connection with the eNB

- The UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

This MO/MT data transmission procedure

Figure 2.2-1: Example of procedure when service link is available

RAN2 revisit the MO procedure agreed in RAN2#126, and agree following legacy procedure as starting point for further discussion. (covering both MO and MT UL/DL data transmission):

- the eNB send paging to UE if there is stored data/signalling at the eNB/satellite for the UE.

- the UE establish RRC connection with the eNB

- the UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

# Conclusion

In summary, we would like to share our view on S&F as follows:

- Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Proposal 1:	RAN2 discuss how UE should be encouraged to camp/reconnect to the same satellite/satellite sets (provided by upper layer) at least when there is unfinished store and forward data/signalling transmission.

Proposal 2:	If UE is encouraged to camp/reconnect to the same satellite/satellite sets, consider to let the PUR resources to be valid within the same satellite/satellite sets, which means UE can suspend the PUR resources untill the coverage of the same satellite comes back.

Proposal 3:	UE(s) who support Store&Forward feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

-	For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

-	For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

Proposal 4:	RAN2 discussion focus on Uu procedure when service link is available but feeder link is not available.

Proposal 5:	RAN2 revisit the MO procedure agreed in RAN2#126, and agree following legacy procedure as starting point for further discussion. (covering both MO and MT UL/DL data transmission):

1)	the eNB send paging to UE if there is stored data/signalling at the eNB/satellite for the UE.

2)	the UE establish RRC connection with the eNB

3)	the UE sends uplink data/signalling to the eNB for storing and forwarding

4)	the eNB forward the stored downlink data/signalling to the UE

5)	the eNB release the RRC connection