3GPP TSG-RAN WG2 Meeting #127-bis	R2-2410408

Orlando, US, 18 – 22 November, 2024

Agenda item:	8.7.4.2

Source:	InterDigital

Title:	DSR enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The WID objective wrt. DSR enhancements was focused as follows [RP-241771]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

RAN2#127-bis made the following agreements wrt. the DSR enhancement:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: • Buffer size of data volume in each portion • Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

# 2	Discussion

## 2.1	Data to be included in the Enhanced DSR MAC CE

Before discussing how the setting of triggering and reporting thresholds should/could be made, it should be discussed what data should be reported by the Enhanced DSR MAC CE. In Rel-18 DSR MAC CE, only the strictly delay-critical data (as defined by TS 38.321/322/323) is included in the DSR MAC CE. It seems important to keep such principle also for Rel-19 given otherwise the gNB cannot deduce from the Enhanced DSR MAC CE how much of the reported data is actually delay critical – this would defeat the whole purpose of reporting DSR and would not differ from BSR too much anymore.

Proposal 1: Only delay critical data is reported as part of the Enhanced DSR MAC CE.

However, the following FFS point was left in RAN2#127 which is yet to be discussed:

FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

It should be noted that such data includes only non-delay critical data that neither the RLC SDU nor a segment thereof has been submitted to the lower layers by RLC entity – RLC data PDUs that are pending re-transmission (with or without delay critical RLC SDU (segment)) are already considered as delay critical data. Hence, it was argued by chipset vendors in the previous RAN2 meeting that such non-delay critical data ahead of delay critical data in the buffer could be handled by UE implementation (e.g., not assigning the SDU with PDCP SN and consequently not doing any preprocessing before a grant is received). While this could be acknowledged, it is worth noting that this compromises the pre-processing capabilities in the UE which was one of the design factors for NR UP protocols in Rel-15. It would be hence preferable to retain the design factor of allowing pre-processing and support reporting of the non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.

Proposal 2: Define the non-delay critical data ahead of delay critical data in the buffer as delay critical and include it in the buffer size calculation for DSR.

## 2.2	Triggering and reporting thresholds

Considering that the Rel-19 Enhanced DSR MAC CE only reports the delay critical data (similarly to Rel-18) as proposed above, there seems to be no use case to set any of the reporting thresholds to a larger value than the triggering threshold (ie., remainingTimeThreshold):

-	Only in the case pdu-SetDiscard is configured, there could be a PDCP SDU with remaining time till discardTimer expiry more than the remainingTimeThreshold – this is the case if the PDCP SDU belongs to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

However, this principle is there already in Rel-18 and such data can be reported as part of the Buffer Size of the largest remaining time threshold in the Enhanced DSR MAC CE (ie., same as with Rel-18).

Hence, it would seem reasonable to keep this Rel-18 principle and require the triggering threshold to be the largest of the configured threshold values. Then, for each of the reporting thresholds (with lower value than the triggering threshold), the UE would report all the delay-critical data that has remaining time up to the threshold value. After each of these reporting thresholds have been encoded into the Enhanced DSR, the UE then reports the remainder of the delay-critical data with any remaining time. Hence, a requirement could be specified that all the reporting thresholds have the time threshold shorter than the remainingTimeThreshold (ie., the triggering threshold).

Proposal 3: Specify a requirement that the NW sets the DSR triggering threshold (ie., remainingTimeThreshold) as the largest time threshold among the multiple remaining time thresholds configured.

## 2.3	Enhanced DSR MAC CE

The following was agreed in the previous RAN2#127-Bis meeting:

For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: • Buffer size of data volume in each portion • Shortest remaining time among PDCP SDUs buffered in each portion.

Based on the agreement and the discussion in above sections, in the below Figure 1, an illustration of constructing the Enhanced DSR MAC CE for an LCG is provided:

-	For reporting threshold 1: the Buffer Size of the red packets is reported with Remaining Time being set to the shortest remaining time of the red packets;

-	For reporting threshold 2: the Buffer size of the lavender packets is reported with Remaining Time being set to the shortest remaining time of the lavender packets;

-	For reporting threshold 3: reporting is omitted since there is no packet with remaining time more than reportingThreshold2 but less than reportingThreshold3;

-	For reporting threshold 4 (ie., remainingTimeThreshold):

-	The Buffer size of orange packets is reported – including the packets being regarded as delay critical data due to being part of a PDU set with a PDCP SDU having remaining time till discardTimer expiry less than remainingTimeThreshold.

-	Remaining Time being set to the shortest remaining time of the orange packets.

Figure 1. Illustration of constructing Enhanced DSR MAC CE for an LCG.

Hence, in the above example, for LCGX, three pairs of remaining time and buffer size is reported in the Enhanced DSR MAC CE (regardless of UE being configured with 4 thresholds). NW knows based on the reported Remaining Time to which “portion” and, hence, to which reporting threshold the reported Buffer Size is related to. This being the case, simplest approach for constructing the Enhanced DSR MAC CE is to take the legacy DSR MAC CE as baseline and use the available R bit in the octet of Remaining Time to indicate if further pair of Remaining Time/Buffer Size will follow for the LCG. Since the R bit is in an already used octet, no additional byte needs to be added (e.g., as with bitmap solution). Bitmap solution would require additional byte for each LCG while it would not provide any additional information compared to the allocating the available R bit as extension bit (E in the below figure) Hence, the Enhanced DSR MAC CE format should be the following:

Figure 2. Enhanced DSR MAC CE format.

Proposal 4: The R bit in the octet of Remaining Time in the Rel-18 DSR MAC CE is replaced with extension bit which indicates if further pair of Remaining Time/Buffer Size follows for the same LCG or not.

## 2.4	DSR truncation

In the previous RAN2#127-Bis meeting, it was discussed whether UE should be allowed/required to send the legacy Rel-18 DSR MAC CE in case the Enhanced DSR MAC CE does not fit to the available grant size scheduled by the NW – the following FFS was captured:

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Such optimization would work only for the special case that the legacy DSR MAC CE fits to the available grant and the Enhanced DSR MAC CE does not. This assumption would only hold for a case where at least one LCG would otherwise report multiple pairs of Remaining Time/Buffer Size thresholds – otherwise, the Enhanced DSR MAC CE also fits to the grant being the exact same size as the legacy DSR MAC CE. Such small optimization seems unnecessary and we should rather discuss whether we create new truncated Enhanced DSR MAC CE format (which could be optimized better and give more information to NW than the legacy format), or we assume NW knows there could be available DSR report (and the maximum size of the DSR) and schedules the UE accordingly.

Proposal 5: Legacy DSR MAC CE is not used when at least one LCG is configured with multiple reporting thresholds.

Proposal 6: If DSR truncation is supported, create a new Truncated DSR MAC CE format for the purpose.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Only delay critical data is reported as part of the Enhanced DSR MAC CE.

Proposal 2: Define the non-delay critical data ahead of delay critical data in the buffer as delay critical and include it in the buffer size calculation for DSR.

Proposal 3: Specify a requirement that the NW sets the DSR triggering threshold (ie., remainingTimeThreshold) as the largest time threshold among the multiple remaining time thresholds configured.

Proposal 4: The R bit in the octet of Remaining Time in the Rel-18 DSR MAC CE is replaced with extension bit which indicates if further pair of Remaining Time/Buffer Size follows for the same LCG or not.

Proposal 5: Legacy DSR MAC CE is not used when at least one LCG is configured with multiple reporting thresholds.

Proposal 6: If DSR truncation is supported, create a new Truncated DSR MAC CE format for the purpose.