3GPP TSG-RAN WG2 Meeting #128	R2-2410425

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.1.3

Source:	Nokia

Title:	Data Collection for Training of NW side ML models

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution elaborates on the issues related to beam management data collection for network-side models. In particular, we will discuss in context of the open points identified in RAN2#127bis:

RAN2#127bis Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for NW sided/UE sided BM model training, at least L1 RSRPs and/or beam IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress.

# 2	Discussion

## 2.1	Framework and Immediate MDT procedure scope

For network-side data collection related to beam management, RAN2#126 agreed to enhance Immediate Minimization of Drive Test (MDT). The aim is to establish common framework for collecting measurements for both: gNB-centric and OAM-centric approaches. The basic procedures of an OAM centric network-side data collection can be realized with the following steps illustrated in Figure 1:

Step 1, Step 7: RAN2 can assume that data collection configuration activation will reuse the existing procedure of activating Immediate MDT through Trace Activation and Trace Record procedures.

Step 2: UE capability information (e.g. logging support) exchange ensures relevant UEs selection.

Step 3: CSI framework configuration triggers data collection between the gNB and the UE, to measure, log and report L1 measurements via RRC message.

Step 4-5: The UE measures L1 measurements and log in AS layer, if logging condition (periodic or event-based trigger) is configured, otherwise, Step 7 applies.

Step 6: UE reports the logged data. Details can be found in Section 2.3.

Figure 1: Basic procedure of an OAM centric data collection.

Proposal 1: RAN2 to adapt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

These steps can set the basis for the common procedure to enable data collection configuration and reporting for beam management for both: gNB-centric and OAM-centric approach.

Observation 1: The scope of the Trace Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

In the follows, we discuss about configuration and reporting of logged measurements.

## 2.2	Logged Measurement configuration

Training of network-side models for beam management will rely on the UE that receives a CSI-RS measurement configuration for the set of RS resources (e.g., NZP-CSI-RS resources or SSB resources). This assumption serves as a basis for data collection, also in line with RAN1 recognition noted as RAN1#116bis agreement:

Observation 2: For network-side model for beam management Case 1 and beam management Case 2, RAN1 agreed to rely on existing CSI framework.

The UE performs required measurements on the configured RS resources. The configuration of RS resources (NZP-CSI-RS-Resource(s) and NZP-CSI-RS-ResourceSet(s)) specified in CSI-MeasConfig is a required pre-requisite for AI/ML beam management, as these RS resources specify necessary reference signals to be measured.

Proposal 2: For data collection for NW-side models, legacy CSI-MeasConfig is re-used to configure measurement resources (e.g. resource type, resource set) as a pre-requisite for logging.

Under the CSI framework, CSI-MeasConfig is neatly collocated with CSI-ReportConfig. This Information Element is used to configure either a periodic or semi-persistent report sent via PUCCH in the cell where the CSI-ReportConfig resides, or for establishing a semi-persistent or aperiodic report communicated through PUSCH when triggered by DCI received in that specific cell. The reporting conditions are determined based on reportConfigType of CSI-ReportConfig.  The conditions correspond directly to a reporting instance based on recent measurement(s), therefore CSI-ReportConfig specifies reporting conditions that are not necessary for logging.

Observation 3: The reportConfigType of CSI-ReportConfig corresponds directly to a reporting instance based on recent measurement(s), whereas for data collection, considering a log of measurements, the reporting instance is non-deterministic, and what is more important is the timing of measurements, not the time of reporting of measurements.

For network-side model, in particular, L1-RSRP and beam IDs will be used as training data. A separate trigger needs to be defined to activate logging of these CSI-RS measurements, because the current setup is unable to handle logging activation and logged reporting conditions. It can be configured with a new specific configuration parameter for logging configuration fields (e.g., CSI-LoggedConfig). Once the network initiates the RRC Reconfiguration with CSI-MeasConfig configuration as a pre-requisite and a new Information Elements, it will trigger the logging. While such configuration will activate at least logging, it may also imply how the reports will be provided.

Proposal 3: RAN2 to define a new IE, e.g., CSI-LoggedConfig, under CSI-MeasConfig, for configuration of logged measurement data collection for training of network-side AI/ML beam management models, which includes at least logging events (periodical or event-based).

Proposal 4: RRCReconfiguration message is used to activate logged CSI measurements data collection.

If the above proposals agreed, then we can confirm our understanding with RAN1.

Proposal 5: Confirm RAN2’s assumption with RAN1 on logging configuration within CSI-MeasConfig.

### 2.2.1	Multiple configurations

At RAN2#127 the following agreements were made on multiple measurement configuration:

RAN2 #127 Agreements

1. As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

According to the above agreement, UE will receive logging configurations, measurement configurations from the network. The network can explicitly configure trigger for data collection process initiation in two different ways: immediately and UE initiated. In this regard, multiple configurations can be understood as a UE is configured based on, for example, different logging types, different LCM operations, different data collection configurations. If UE is configured with same data collection RSes (say a subset of resources) with multiple different data collection initiation configurations, then there will be duplicate measurements reporting. If UE is configured with different RSes in multiple configurations, the CSI computation complexity of the UE can increase. Since this will impact UE behaviour at the physical layer which would require input from RAN1. In our understanding, UE shall report what it is being configured by the network which will not require any additional spec impact. Moreover, we observe that CSI framework enables multiple instances of the configurations (of measurement resources and reporting types), with certain allowable maximum values (cf. TS 38.331 [5]). Since data logging will be based on RRC configuration, we should ensure that ensure total number of CSI-RS resources and applied configurations do not exceed the maximum allowable values, even if shared with other LCM purposes (such as inference).

Observation 4: Multiple instances of configurations are supported by adequate RRC configurations.

Proposal 6: RAN2 assumes that configurations for NW-side training data collection share total allowable number of CSI-RS resources and reporting configurations for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed it.

### 2.2.2	Logged Measurements configuration release

The CSI-MeasConfig used to configure reference signals belonging to the serving cell uses regular RRC configuration approach with “AddModList” and “ReleaseList” to configure or deconfigure resources. It is a straightforward implication that a release operation for the CSI-RS measurements resources causes that the UE cannot continue with data collection, if the resources are released. In addition, the solutions for AI/ML-enabled beam management for Rel-19 are limited to UE in RRC CONNECTED state by Work Item objectives, thus, regular operations for RRC configuration release will also impose the resources configuration release when the UE leaves RRC CONNECTED mode.

Proposal 7: UE will follow the legacy procedures to release the configurations. That is, UE releases configuration for network-side data collection for beam management, if RRC Reconfiguration releases the associated CSI-MeasConfig configuration or the UE leaves RRC CONNECTED mode.

## 2.3	Logged Measurement Reporting

### 2.3.1	Availability indicator

A default behaviour that can be assumed is that having logged measurements data collection activated, the UE will make measurements constantly. That is, for periodical logging event, the measurements need to be collected in given intervals, and for event-based trigger configurations, the measurements will be performed to enable assessment that the event took place. Although, RAN2#127bis agreed that UE can indicate an availability indicator to the network when the logged data is available, details remain to be specified.

In general, data availability indicator is used to assist the network in triggering the transmission of logged data from UE. This has been essential for Logged MDT, as the UE performs logged measurements data collection in IDLE/INACTIVE modes and could reselect to a gNB unaware of the previous logged MDT configuration. During Rel-19 data collection for network-side ML models, data will be logged and reported in RRC CONNECTED mode. Therefore, if the UE is configured with periodic logging, then, network can roughly estimate and expect when it is optimal to retrieve the logged data from the UE (for instance, considering the minimum UE’s buffer size (e.g., 64Kbytes), logging interval (e.g., CSI-ResourcePeriodicityAndOffset), and allowable maximum size per RRC message (e.g. 9Kbytes)). The availability indicator would create an unnecessary overhead in this case. By the reuse of the UEInformationRequest/ UEInformationResponse procedure, the network can request, and UE can respond without the availability indication.

Observation 5: With UEInformationRequest/UEInformationResponse procedure data can be retrieved without availability indication.

However, if UE is configured with event-based logging, then network may not know the frequency that events were triggered within UE and that the UE will have data available to report. In such situations, it might be more useful to indicate to network that data is available for retrieval.

In this case there are at least two options (i) the UE indicates the availability of data, or (ii) the UE sends the data in an opportunistic manner without any explicit request from the network. The advantage of availability indication is that the UE has no knowledge about network traffic load and when is the best time for network to receive the logged report. The drawback of availability indication is the extra overhead especially if the availability indication is not piggybacked in an otherwise sent RRC message. Therefore, it is strongly recommended to enable the piggybacking of the availability indication in RRC messages, such as RRCReconfigurationComplete and UEInformationResponse. Another issue with the availability indication is that it might be possible that network could receive this indicator too late, and as a consequence, there might be not enough resources to schedule UE and handle the report. Thus, in our understanding, the availability indicator to retrieve report should not be a default behaviour rather an optional mechanism.

Proposal 8: The availability indicator should be an optional mechanism in report retrieval procedure for data collection for beam management.

Proposal 9: The availability indication of logged data can be piggybacked in legacy RRC messages.

A compromised solution is that when the UE indicates the report availability in a dedicated RRC message the UE may also send the first segment of the report along with the indication. After receiving the first segment the network can decide when to retrieve the rest of the available data via the agreed on-demand reporting mechanism. A UE can make the decision of including the first segment of the data e.g., based on the report size, the available buffer size etc. In this solution the UE cannot overload the network with long reports, but the extra overhead of availability indication can be decreased, especially in the case of short reports that can fit in a single RRC message.

Proposal 10: The UE may send the first segment of the logged data along with the availability indication of logged data when a dedicated RRC message is used for availability indication.

### 2.3.2	Report retrieval

We recognize three possibilities for logged CSI measurements retrieval (as presented in Figure 2):

- Alt#1: On-demand through UEInformationRequest/UEInformationResponse procedure with availability indicator,

- Alt#2: On-demand through UEInformationRequest/UEInformationResponse procedure without availability indicator,

- Alt#3 Opportunistic retrieval with lower priority SRB.

Figure 2: Reporting procedure options.

Regardless of whether the availability indicator is supported, it should not have an impact on the actual data retrieval in the on-demand procedure. The purpose of availability indicator is to give the network some flexibility to schedule the transmission of the UE collected data, but in general the network requesting data by UEInformationRequest, enables and expects requested data transmission. Therefore, for on-demand retrieval, we see no need to use low priority SRB, as the network considers its load situation before initiating the demand. That is, the legacy procedure can be reused. However, in both cases (with availability indicator or without), the required enhancement is to enable the UEInformationRequest message with a flag enquiring logged CSI reports.

Proposal 11: UEInformationRequest is extended with a query bit for logged CSI measurements to indicate the data transmission is requested.

Each log entry could be of a different size, depending on report quantities collected and on how many beams were detectable and measurable by the UE. Therefore, for both periodic and event-based logging triggers, the estimation by the network on when the log is full may lack precision. While attempting to align with a single RRC message size, the estimates may fall short in accuracy. For this issue, we envision that the third approach with opportunistic report transmission, can be a good solution. In addition, it would be a good alignment with RAN2#127bis agreement on “low priority SRB will be used. FFS new SRB or use of SRB4”.

Proposal 12: Support opportunistic logged data transmission initiated by UE via new uplink message.

When the network can request to retrieve the data (with on-demand based report retrieval), lower priority SRB is not critical, and it does not seem to offer any additional benefits. However, opportunistic approach for data transmission seems suitable and optimal. When the reporting conditions met, such as event-triggered reporting (e.g. buffer-based, which can be specified, left to implementation (until matching 9KB) or configured), the UE could attempt to send the logged report with a low priority SRB either SRB4 or a new SRB to report the logged CSI reports.

Proposal 13: Reporting of logged measurements can be carried out via SRB2 if on-demand report retrieval mechanism (UEInformationRequest/UEInformationResponse) is used.

Proposal 14: Reporting of logged measurements can be carried out via SRB4 or a new SRB if opportunistic data transmission (initiated by the UE) via new UL message is supported.

# 3	Conclusion

This document has made the following observations:

Observation 1: The scope of the Trace Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

Observation 2: For network-side model for beam management Case 1 and beam management Case 2, RAN1 agreed to rely on existing CSI framework.

Observation 3: The reportConfigType of CSI-ReportConfig corresponds directly to a reporting instance based on recent measurement(s), whereas for data collection, considering a log of measurements, the reporting instance is non-deterministic, and what is more important is the timing of measurements, not the time of reporting of measurements.

Observation 4: Multiple instances of configurations are supported by adequate RRC configurations.

Observation 5: With UEInformationRequest/UEInformationResponse procedure data can be retrieved without availability indication.

And proposed the following:

Proposal 1: RAN2 to adapt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

Proposal 2: For data collection for NW-side models, legacy CSI-MeasConfig is re-used to configure measurement resources (e.g. resource type, resource set) as a pre-requisite for logging.

Proposal 3: RAN2 to define a new IE, e.g., CSI-LoggedConfig, under CSI-MeasConfig, for configuration of logged measurement data collection for training of network-side AI/ML beam management models, which includes at least logging events (periodical or event-based).

Proposal 4: RRCReconfiguration message is used to activate logged CSI measurements data collection.

Proposal 5: Confirm RAN2’s assumption with RAN1 on logging configuration within CSI-MeasConfig.

Proposal 6: RAN2 assumes that configurations for NW-side training data collection share total allowable number of CSI-RS resources and reporting configurations for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed it.

Proposal 7: UE will follow the legacy procedures to release the configurations. That is, UE releases configuration for network-side data collection for beam management, if RRC Reconfiguration releases the associated CSI-MeasConfig configuration or the UE leaves RRC CONNECTED mode.

Proposal 8: The availability indicator should be an optional mechanism in report retrieval procedure for data collection for beam management.

Proposal 9: The availability indication of logged data can be piggybacked in legacy RRC messages.

Proposal 10: The UE may send the first segment of the logged data along with the availability indication of logged data when a dedicated RRC message is used for availability indication.

Proposal 11: UEInformationRequest is extended with a query bit for logged CSI measurements to indicate the data transmission is requested.

Proposal 12: Support opportunistic logged data transmission initiated by UE via new uplink message.

Proposal 13: Reporting of logged measurements can be carried out via SRB2 if on-demand report retrieval mechanism (UEInformationRequest/UEInformationResponse) is used.

Proposal 14: Reporting of logged measurements can be carried out via SRB4 or a new SRB if opportunistic data transmission (initiated by the UE) via new UL message is supported.