3GPP TSG RAN WG2 Meeting #128                          	   R2-2410430

Orlando, US, Nov. 18th – 22nd, 2024

Agenda item:	8.12.3

Source:	Qualcomm Incorporated

Title:	Discussion on UE initiated beam reporting

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For UE-initiated/event-driven beam management, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss potential RAN2 work for supporting UE initiated beam reporting.

# Discussion

## Event evaluation

RAN1 has made progress and decisions on the following design subjects [2] [3].

Beam reporting modes - Mode A: dynamically scheduling UCI by gNB - Mode B: UCI in pre-configured resource(s) for second UL channel #### UL signaling content(s) of L1-RSRP report - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB Beam reporting trigger events - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. RS measurement for the current beam for Event-2 - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. RS measurement for the new beam for Event-2 - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

However, RAN1 has not concluded if the triggering event determination is handled at PHY or MAC layer.

Observation 1. RAN1 has not conclude whether PHY layer or MAC layer handles the event evaluation.

At RAN2 #127, the following agreements were made for LTM event evaluation [4].

Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

Observation 2. It has been agreed that MAC layer handles the event evaluation and L1 measurement report triggering for LTM.

Currently, MAC layer handles beam failure detection based on L1 measurement (e.g., counting beam failure instances within a window) and sends beam failure recovery on PUSCH if a grant is available or on PRACH otherwise. Additionally, beam failure recovery is also conducted at MAC.

Observation 3. MAC layer handles beam failure detection based on L1 measurements.

At RAN1 #117, the following agreement was made for triggering event determination for event 2 [5].

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Based on the above RAN1 agreement, event evaluation involves counting the number of event-triggering occasions within a time window (similar to beam failure detection). Therefore, it has less spec impact for MAC layer to handle the event evaluation based on L1 mesurements.

Observation 4: It has less spec impact for MAC layer to handle the event evaluation for UE-initiated/event-driven beam reporting.

Based on the above observations, we make the following proposal.

Proposal 1. RAN2 supports that the event evaluation is handled at MAC layer.

- Consider an LS to RAN1 if it is agreed to support event evaluation at MAC layer.

## First PUCCH

In RAN1 #118, the following agreements were made regarding the first PUCCH configuration for Mode-A and Mode-B [6]:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

If the event evaluation is managed by the MAC layer, Alt 1 (dedicated SR) is preferred as it allows reusing the legacy SR design, including SR retransmission, prohibit timer operation, and multiplexing / dropping / prioritization rules. Otherwise, if the event evaluation is managed by the PHY layer, a new UCI type is preferred to avoid inter-layer signaling between the PHY and MAC layers, which may cause more specification work in PHY layer.

Observation 5. If the event evaluation is handled by the MAC layer, the first PUCCH for both Mode-A and Mode-B should be defined as a dedicated SR resource (Alt 1).

Proposal 2. RAN2 waits for RAN1’s decision on the first PUCCH design.

# Conclusion

In this contribution, we discussed potential RAN2 work for supporting UE initiated beam reporting and concluded with the following observations and proposals.

Event evaluation

Observation 1. RAN1 has not conclude whether PHY layer or MAC layer handles the event evaluation.

Observation 2. It has been agreed that MAC layer handles the event evaluation and measurement report triggering for LTM.

Observation 3. MAC layer handles beam failure detection based on L1 measurements.

Observation 4. It has less spec impact for MAC layer to handle the event evaluation for UE-initiated/event-driven beam reporting.

Proposal 1. RAN2 supports that the event evaluation is handled at MAC layer.

- Consider an LS to RAN1 if it is agreed to support event evaluation at MAC layer.

First PUCCH

Observation 5. If the event evaluation is handled by the MAC layer, the first PUCCH for both Mode-A and Mode-B should be defined as a dedicated SR resource (Alt 1).

Proposal 2. RAN2 waits for RAN1’s decision on the first PUCCH design.

# References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] RP-241775, Status Report of NR MIMO Phase 5, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[3] RP-241051, Status Report of NR MIMO Phase 5, RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[4] R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, RAN2 Meeting #127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[5] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024.

[6] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024.