3GPP TSG-RAN WG2 Meeting #128	R2-2410440

Orlando, USA, 18th – 22nd of November 2024

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On the Open Points for Supporting MBS in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2#127bis meeting, the following decision has been made [2]:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

Few other topics have been marked as “Come back in the next meeting”. In this paper we present our further view on what remains to be addressed for this Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Service area signalling – MCCH or SIB

The design of service area signalling for MBS in NTN has been made in baby steps.

At RAN2#126 (May 2024) the following has been agreed [4]:

For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

At RAN2#127 (August 2024) the topic was discussed via offline thread [6] and the following was agreed [5]:

2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

At RAN2#127bis the next decision towards the final design has been made [2]. MCCH shall comprise one or more service area IDs per each MBS service. However, it still remains to be decided if the actual service area definition is also in MCCH or in SIB (e.g. SIB20 or in new SIB).

Observation 1: As per RAN2#127bis, MCCH comprises a service area ID per each service area. It remains to be decided if the actual service area definition is also provided in MCCH or in SIB.

According to the MBS description in [3], the MBS configuration information is carried on MCCH logical channel. It comprises the information on the MBS broadcast sessions that are provided, related scheduling information and it can also contain the list of neighbour cells where the same broadcast MBS services are provided.

Observation 2: MCCH already provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

In order to receive MCCH, the UE needs to acquire appropriate information from SIB1 and SIB20 [3].

Observation 3: The information from SIB1 and SIB20 is essential for the UE to be able to receive MCCH.

One of the Rel-19 NTN questions that remain is where this service area definition is to be provided for NTN cells supporting MBS. The intended service area information might be provided in SIB (e.g. SIB20 or SIB21), then we need a separate list of provided broadcast sessions in the cell, although the list already exists in the MCCH. Another approach could be providing the intended service area for each broadcast sessions in the MCCH in the same order.

At RAN2#127 it has been agreed that the UE that is not within the intended service area may not reacquire the MCCH.

Observation 4: UE that is not within the intended service area may not reacquire the MCCH.

This behaviour seems to be reasonable, and it shall be up to the UE implementation how to realize whether it is within or outside of the intended service area. This RAN2#127 conclusion also does not restrict the solution to be necessarily SIB20-based or MCCH-based. MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI, so just MBS-interested UEs are receiving this notification. The indication can explicitly tell if new service is to be provided or if it has changed.

Observation 5: MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI and indicate with a single bit if new service will be provided.

The solution relying on SIB20 has at least the following drawbacks:

- changes to service area impacts all UEs in the cell

- there should be a mapping between the service area provided in SIB20 (or other SIB) and the MBS session info available in MCCH

Thus, in our view MCCH-based solution is clearly a better approach, so once again we propose the following:

Proposal 1: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions, intended broadcast service area) in MCCH (within MBSBroadcastConfiguration).

## 2.2	MBS Interest Indication (MII)

At RAN2#127bis the following has been discussed and not resolved :

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. Come back in the next meeting

In section 5.9.4.2 of [3] the following text can be found:

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell.

As can be seen, the MII might be sent when UE enters or leaves the broadcast service area and this is already supported by NR RRC [3].

Observation 6: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Thus, it that nothing new has to be agreed for MBS II in NTN. Furthermore, the behaviour described in [3] is not binding (the term used in the procedure is “UE may initiate”), so in our view this issue is not worth spending additional time. Note that the MII feature can be used by the UE in case UE cannot find the MBS broadcast service that it is expecting in a cell. That would make, e.g., a gNB decides whether to handover the UE to a possible neighbor gNB/cell providing MBS broadcast service UE has interest in. Thus, as has been indicated at RAN2#127bis, it is a UE implementation issue and a UE can decide when to send the MII (which could be somehow related to the intended service area).

Proposal 2: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

## 2.3 Configuration aspects of MBSBroadcastConfiguration and service continuity in SIB21

At RAN2#127bis the following has been discussed and not decided :

In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

As can be seen in [3], MBS-SessionInfo (a part of MBSBroadcastConfiguration) comprises mtch-neighbourCell. This IE indicates neighbour cells which provide this service on MTCH. Thus, achieving the first goal stated above seems to be possible using the existing means (i.e. it is feasible to tell the UE in which neighbouring cells the same service is provided). There is no exact indication about the intended service area, but that is perhaps not critically needed (the UE will know the service area details from its current serving cell).

Proposal 3 RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be roughly known thanks to mtch-neighbourCell.

Regarding the second aspect highlighted above, whether intended service area is provided in SIB21 for each session or per FSAI, this would somewhat change/extend the purpose of SIB21. SIB21 is defined as follows [3]:

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). SIB21 information element -- ASN1START -- TAG-SIB21-START SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 } MBS-FSAI-r17 ::= OCTET STRING (SIZE (3)) -- TAG-SIB21-STOP -- ASN1STOP

Thus, introducing the service area information to SIB21 does not match its main purpose (i.e. frequency mapping to FSAI). FSAIs are configured per gNB or per cell. If these are to be configured per intended service area, this requires lots of changes to core and service announcements. Overall, the benefits are limited, if any, while it might have a large specification impact, not only in RAN2.

Proposal 4: FSAIs are configured per gNB or per cell. RAN2 does not pursue FSAI to intended service area mapping, as this might have impact on core and service announcements (not limited to RAN2) for no major benefits.

## 2.4 Entering and leaving the service area versus MRBs

At RAN2#127 it was agreed that [4]:

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

As can be noticed, no firm requirements have been defined for the UE regarding MRBs handling when the UE is outside of intended service area.

Observation 7: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

In our view at least the action when the UE can establish the MRBs should be clear, i.e. the UE should be allowed to establish the MRBs based on its location versus MBS service area, not just whenever it can receive the MBS. Thus, we suggest the following:

Proposal 5: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

At RAN2#127 it was also decided the UE may release the MRBs when it leaves the intended area. That is acceptable, but we shall not let the UE keep the MRBs for excessively long time. Thus, we suggest the following:

Proposal 6: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

It was debatable how the UE determines it has left the service area. We acknowledge that it requires new means when compared to MBS in terrestrial networks. Nevertheless, it is expected the NTN-capable UE will monitor its geo-position, so it should be able to realize it has left or entered the MBS service area.

Proposal 7: NTN-capable UE can determine whether it is inside or outside of MBS service area based on the GNSS location it is using for other NTN purposes, i.e. there is no need to update UE’s GNSS location specifically for this purpose.

## 2.5	Format of the service area

The following agreements were made during the RAN2#125bis meeting that are relevant for the format of the intended service area [2]:

RAN2#125bis agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

The topic was concluded during RAN2#127 [4] with the following decision:

1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

For the set of referenceLocation and radius, RAN2 can reuse the solution defined in Rel-18 NTN. In order to signal the terrestrial network (TN) coverage area, a new system information block (SIB25) conveys the following information elements (IEs) [7]:

CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65536) }

TN coverage area is described using a reference location and the radius. Together they constitute a circular area with a centre defined by the referenceLocation IE. A list of such referenceLocation and radius pairs should be defined also for MBS intended service area.

Proposal 8: RAN2 introduces a list of ReferenceLocation-r17 and corresponding radius to signal the intended broadcast area.

Regarding polygons, signalling impact can be heavier than for the reference location and radius. Here for a single polygon multiple vertices need to be signalled, possible each with a full ReferenceLocation IE. RAN2 shall discuss if this approach can be optimized somehow and what is the number of vertices that shall be given per each polygon.

Proposal 9: RAN2 to investigate how many vertices should be signalled for a single polygon. In addition it shall be discussed if each vertex of the polygon needs to be represented with a ReferenceLocation  IE.

## 2.6 LS from RAN3 on supporting MBS broadcast service for NR NTN

RAN3 has recently discussed how to support MBS via NTN. More specifically, how the AMF indicates to the gNB when the intended service area is smaller than the satellite footprint. As a result of this discussion, an LS has been sent [8]. The LS lists two options and corresponding text proposals: intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area. RAN2 is asked to discuss this and inform RAN3 when any decisions are made. In our opinion the option with the reuse of Mapped Cell ID (used already in the previous NTN releases) makes more sense. It allows the gNB to do the translation between the Mapped Cell ID and the intended service area coordinates which can be subsequently provided to the UE via Uu interface. RAN2 needs to decide if it selects just one of the options listed in RAN3 LS [8] or intends to support any signalling considered there (i.e. Mapped Cell ID(s) and/or TAI(s), and/or geographical service area).

Proposal 10: RAN2 is asked to discuss which option from RAN3 LS makes most sense and whether down-selection is to be made.

Proposal 11: RAN2 responds to RAN3 and informs on the decision taken (name, format, possible down-selection).

# 3	Conclusion

This document has made the following observations:

Observation 1: As per RAN2#127bis, MCCH comprises a service area ID per each service area. It remains to be decided if the actual service area definition is also provided in MCCH or in SIB.

Observation 2: MCCH already provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

Observation 3: The information from SIB1 and SIB20 is essential for the UE to be able to receive MCCH.

Observation 4: UE that is not within the intended service area may not reacquire the MCCH.

Observation 5: MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI and indicate with a single bit if new service will be provided.

Observation 6: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Observation 7: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

And proposed the following:

Proposal 1: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions, intended broadcast service area) in MCCH (within MBSBroadcastConfiguration).

Proposal 2: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

Proposal 3 RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be roughly known thanks to mtch-neighbourCell.

Proposal 4: FSAIs are configured per gNB or per cell. RAN2 does not pursue FSAI to intended service area mapping, as this might have impact on core and service announcements (not limited to RAN2) for no major benefits.

Proposal 5: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

Proposal 6: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

Proposal 7: NTN-capable UE can determine whether it is inside or outside of MBS service area based on the GNSS location it is using for other NTN purposes, i.e. there is no need to update UE’s GNSS location specifically for this purpose.

Proposal 8: RAN2 introduces a list of ReferenceLocation-r17 and corresponding radius to signal the intended broadcast area.

Proposal 9: RAN2 to investigate how many vertices should be signalled for a single polygon. In addition it shall be discussed if each vertex of the polygon needs to be represented with a ReferenceLocation  IE.

Proposal 10: RAN2 is asked to discuss which option from RAN3 LS makes most sense and whether down-selection is to be made.

Proposal 11: RAN2 responds to RAN3 and informs on the decision taken (name, format, possible down-selection).

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- 3GPP TS 38.331 NR RRC, v18.3.0, September 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.

- R3-245844 LS on Supporting MBS broadcast service for NR NTN 3GPP TSG RAN WG3 125-bis Hefei, China, 14 - 18 October, 2024