3GPP TSG-RAN WG2 Meeting #128	R2-2410441

Orlando, USA, 18th – 22nd of November 2024	revision of R2-2408948

Agenda item:	8.6.3

Source:	Nokia

Title:	On L1 Measurement Reporting Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreements have been made in the previous RAN2 meetings [2-4]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this paper we present our views on the remaining open issues for event-triggered L1 measurement reporting framework for LTM.

# 2	Discussion

## Event triggered Reporting for LTM

### 2.1.1 On using the same RS type for Event Evaluation

12 . Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

In RAN2, agreement was made to use the same RS type for both serving and candidate beams for events LTM3 and LTM5. As discussed, the RSs of a candidate cell could be SSBs and/or CSI-RSs. For the serving cell, the measurement RS would be derived from the serving beam, which could also be an SSB or a CSI-RS. It is important to note that the serving beam could change dynamically based on serving cell measurements. Therefore, in scenarios where the serving beam and candidate beam are associated with different RS types (e.g., the serving cell beam is QCL’ed with an SSB, and the candidate RS is a CSI-RS), it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported when the event criteria are met.

In scenarios where the candidate RS is an SSB, it may be straightforward to use the SSB derived from the serving beam via the QCL chain (i.e., the SSB that is QCLed with the QCL RS of the serving beam) for event evaluation. Similarly, when the candidate RS is a CSI-RS, and one of the QCL RSs (i.e., with QCL type of ‘typeD’) of the serving beam is also a CSI-RS, these CSI-RSs can be used for event evaluation.

However, when the candidate RS is a CSI-RS but the serving beam is associated with an SSB as the measurement RS (i.e., QCL RS of serving beam is a TRS which is QCLed with a SSB), it would make sense to use the SSB QCLed with the candidate CSI-RS for event evaluation. Note that TRS is typically not considered as a measurement RS.

We also note that the discussion on this aspect is expected to take place in RAN1. Therefore, it would make sense to leave the details of the solution for this issue to RAN1.

Observation 1: When the serving beam and candidate beam are associated with different RS types it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported.

Proposal 1: From RAN2 perspective same type of RS is used for event evaluation and the UE derives the same type of RS. Details can be handled by RAN1.

### 2.1.2		Event Evaluation at MAC Layer

The RAN plenary has revised the Mobility enhancement WI objectives [1] and it is not required anymore to manage potential overlaps between the MIMO WI and Mobility WI.

To prevent too frequent event triggered reporting, a mechanism to perform at least time domain filtering would be needed. It has been agreed that for LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific offset can be applied.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

For the LTM event evaluation, the TTT concept, similar to L3 reporting, was adopted. In the TTT approach, the UE is configured to observe whether the evaluated criteria is fulfilled for the duration of the TTT (e.g. LTM3 event). In a certain case, the TTT duration could be configured to be ‘0’ which means that if the event criteria is fulfilled, the event and the associated reporting is triggered immediately. As the MAC layer is used for event evaluation, the TTT related procedure (start /stop/expiry) is performed at MAC layer where the event evaluation procedure is defined.

In the agreed operation, the L1 is providing the measurement results to MAC layer (e.g. on RS1… RSk) that includes the candidate RS measurements and serving cell RS measurements, depending on the LTM event type. MAC layer determines based on the configured event criteria when the event criteria is fulfilled and starts the TTT timer. MAC layer also observes that whether the criteria is fulfilled for the duration of the TTT. Once the TTT expires, the event triggered report is generated, reporting the RS or RSs fulfilling the criteria.

Figure 1. Event evaluation by MAC layer processing the L1 measurements.

Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Observation 2: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Proposal 2: the value of TTT is configurable by NW and one TTT value is configured per reporting configuration (per event).

The actual values of TTT can be decided later.

Proposal 3: The TTT range is to be decided in the Stage-3 discussions.

### 2.1.3 TTT per Beam vs. Cell

The main intention of LTM cell switch is to switch the UE to a specific candidate cell’s beam that is confirmed to have good quality in terms of radio signal. In the cell switch command (MAC CE) the NW is always required include the TCI state information. Thus the (R18) LTM is not operating on the cell level evaluation as L3 would work, but on the beam level.

Observation 3: In LTM, the UE is not switched to a cell where any beam can be selected but the UE is switched to a specific beam (inter-cell).

Observation 4: The TCI state information is always present in the cell switch command. Thus the event report triggering should be operating on beam level and not cell level.

Proposal 4: The event evaluation is performed at MAC layer by having a beam specific time to trigger (TTT) timer.

As per the proposal, the TTT is on beam level thus the UE may be required to run multiple TTT timers per each cell.

### 2.1.4 TTT operation on current beam changing

Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Based on the aforementioned agreement. the RS associated with indicated TCI state is used for event evaluation. Since the event triggered procedure and BM management are separate procedures and as result of L1 beam management reporting, the RS corresponding to the indicated beam may change, it should be discussed how it affects the LTM event evaluation. As an example the TTT timer may be running for candidate RS based on the event criterion and the serving cell reference (current beam) may change and it should cause the evaluation to be restarted.

Similar approach is used also for beam failure recovery (BFR) i.e. the change of the RS set used for beam failure evaluation causes the BFD procedure to reset.

Proposal 5: If the current beam (i.e. a beam corresponding to the indicated TCI state) for event evaluation changes, the evaluation for the candidate RS is restarted.

### 2.1.5 Report on Leave

The measurement framework for NR has numerous components which were defined over several releases, starting from Rel-15. One such configurable aspect is reportOnLeave. It is a flag indicating if the UE should report also when one of the cells that previously met a reporting criteria has now left the cellsTriggeredList. A similar mechanism was agreed to be defined for LTM L1 measurements events where the report on leave could report on beam level that the reporting criteria is not fulfilled anymore:

MR can be sent when the leaving condition is met, based on NW configuration.

Nevertheless, RAN2 needs to discuss how report on leave in MAC layer should look like. Among the questions to be answered one can list the following:

- Report on leave is RS/beam-specific or candidate cell specific?

- Does report on leave use the same MAC CE format as the report when entry condition is met?

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 6: RAN2 considers the aforementioned list of open issues when designing the report on leave for L1 measurement reporting.

### 2.1.6 Periodic Transmission of the MAC CE Report

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

RAN2 agreed in the previous meeting to support the periodic event triggered reporting for LTM. At least the periodicity i.e. one-time report/periodic and the number of repetitions should be the configurable parameters. The configuration should be per event triggered reporting configuration.

Proposal 7: The periodic reporting of the LTM event triggered report can be configured per reporting configuration with configurable periodicity and/or number of repetitions.

One further discussion point for periodic reporting is the resource allocation. RAN2 should discuss how to support periodic resource allocation for the report. Also, if the network does not configure periodic resource allocation, the UE needs to request the resources separately for each. The measurement report should be able to be provisioned on any available UL grant.

Proposal 8: RAN2 to discuss support for resource allocation: whether the UE requests resource for each repetition of the report, or the NW provides a periodic UL grant.

### 2.1.7 MAC CE Reporting Container

#### 2.1.7.1 Candidate Cell Identifier

Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

The R18 CSI-resource config may include reference signals associated with one or more candidate cells. As the result of event evaluation, multiple RS indices of the same or different candidate cells may fulfil the reporting criteria, the MAC CE format should potentially have a way to indicate RSs of multiple candidate cells for which the event is reported.

Proposal 9: The LTM Report MAC CE includes indication of a candidate cell identifier that points to the configured LTM candidate identifier(s).

#### 2.1.7.2 Number of Reported Cells

The LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting RS for multiple candidate cells in a single MAC CE.

Observation 5: As the LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting for multiple candidate cells in a single MAC CE.

Proposal 10: The LTM Report MAC CE supports reporting of candidate RSs for multiple candidate cells in a single MAC CE.

#### 2.1.7.3 Event/Reporting Identifier

Basic information included in MR MAC CE:

- Triggered event information (e.g., ReportConfigID)

Since the event triggered reporting supports multiple event types, at least one field in MAC CE should indicate the event for which the reporting is triggered and the reporting configuration. For example, a field in the MAC CE should point to the reporting configuration associated with the event and resource configuration for which the reporting is performed.

Proposal 11: the LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

#### 2.1.7.4 RS index

Candidate cell beam(s) for event evaluation:

For the candidate cell measurements, the measurement resources (e.g. SSBs) listed in the LTM-CSI-Resource config can be used to determine whether certain beam of a candidate cell fulfils the reporting criteria. the LTM CSI resource config may comprise of measurement resources of one more candidate cells.

Figure . CSI resources and reporting configuration for event LTM3/5

Additionally, if the CSI resource config includes the RSs of one candidate cell (or the RSs of a subset of candidate cells) the network could configure multiple LTM CSI resource configs with respective report configurations.

Same resource configuration approach could be used also for the LTM4 event (Figure ). Also the same approach could be used for the LTM2 event, but without candidate cell resources (for reporting) associated with the event.

Figure . CSI resources and reporting configuration for event LTM4.

Rel-18 L1 reporting format for LTM includes RS index value that is determined based on the configuration order of the RS in the resource configuration. The bit length of the RS index field in R18 is LI reporting format is determined by the number of configured RS in the resource set.

Observation 6: In L1 UCI reporting format the number of configured RS in the resource set is used to determine the bit width of the Resource identifier field.

The bit width for resource indicator field may thus vary from (0 bits) to 9 bits for LTM: For each resource set up to 64 indices can be included and RS for up to 8 candidate cells can be configured. This results in up to 512 RS indices which means that up to 9 bits per RS index may be needed.

Supporting such range and flexibility is not feasible for MAC CE format which typically has fixed size fields. The value for RS index bit field should be based on the maximum number of RS index values (64) supported per candidate cell:

Proposal 12: The LTM Report MAC CE includes RS index field wherein the RS index field size is fixed 6 bits (to reflect up to 64 RS indices).

If lower number of resource indicators (thus fewer bits) is needed for reporting for a candidate cell, the RS index field can only encode resource indicators on required bit positions while maintaining the size allocated per RS index.

To differentiate reported RSs of different candidate cells, the RS index field encodes the resource indicator field that points to the reference signals, in the LTM CSI-ResourceConfig per PCI. The resource indicator ID space (of 64) is reused for each candidate cell in the CSI-resource config.

Observation 7: The resource indicator ID space can be reused for each candidate cell in the LTM-CSI-configuration associated with reported event.

Proposal 13: The LTM Report MAC CE includes RS index field that encodes resource indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 14:  The resource indicator SSBRI/CRI in the RS index field is associated with Candidate identifier

#### 2.1.7.5 Number Of Reported RSs

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

The LTM Report MAC CE supports reporting of variable number of candidate RSs, up to the configured number of RSs. To be more precise the value N should refer to maximum of reported RS per candidate cell. If the LTM report MAC CE supports reporting of RSs for multiple candidate cells, the UE can report up to N per each of  candidate cells.

Observation 8: up to N beams can be reported per each of the candidate cell.

Proposal 15: The LTM Report MAC CE supports reporting of variable number of candidate RS per candidate cell(s), up to the configured number of RSs.

We don’t see a benefit to include RS indices in the LTM report MAC CE that correspond to RSs that do not fulfil the reporting criteria. Those RSs are not the ones that triggered the report and are not candidates for cell switch.

Proposal 16: The LTM Report MAC CE includes the RSs that have fulfilled the reporting criteria for the reported event. RSs that do not fulfil the reporting criteria are not reported.

#### 2.1.7.6 Measurement Quantity reporting

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

Providing the RSRP value associated with reported RS identifier gives the network essential information on the quality of the RS that triggered the event. In the previous RAN1 meeting there was no conclusion to support L1-SINR measurement based on CSI-RS:

Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells

Proposal 17: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

#### 2.1.7.7 Differential RSRP reporting

For the Rel-18 LTM L1 report, the RSRP is reported in differential format. In differential RSRP reporting format the first listed RSRP value is in absolute format (7 bit) and other listed values are in differential format (4 bit) relative to the absolute RSRP value. The differential RSRP reporting could bring overhead savings. Whether to use absolute RSRP values in each report or rely on differential RSRP values could be also specific to certain events.

Proposal 18: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE.

#### 2.1.7.8 Serving Cell Information

10. Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

It is important for the NW to know the serving’s beam RSRP, especially when LTM events using the RSRP of the serving beam are applied. At the same time, it can be safely assumed the network knows current RS index associated with the serving cell.  Thus, it can be omitted from the report.

Observation 9: the serving cell RS index/identifier should be implicitly known by the network thus it could be omitted in the MAC report content.

Observation 10: Although events such as LTM 3/5 make already the comparison between current beam RS and candidate RS i.e. so that reported RS quality is relative to the current beam quality it is beneficial to include the serving cell quality to provide more information to network.

Proposal 19: Include serving cell RSRP measurement for events for which the current beam RS is used for event evaluation i.e. LTM 2/3/5.

In order to consider to inter-working with the beam management procedure, it might be beneficial if the UE includes RS associated with indicated (and activated TCI states) in the event based measurement report. Network can use this to delay LTM cell switch if another serving cell beam is strong e.g. based on beam management procedure.

Proposal 20: UE can be configured to include RS associated with indicated (and activated TCI states) in the event based measurement reporting.

#### 2.1.7.9 Truncated Format for MAC CE

When multiplexing the LTM Report MAC CE on UL grant, the UL grant may have limited size i.e. not all the reported information can be included. Thus, the truncation of the MAC CE should be supported. The details can be discussed later once the work on the MAC CE format matures.

Proposal 21: Support truncation of LTM report MAC CE. Details FFS (once the work on MAC CE format matures).

### 2.1.8 Scheduling Request to indicate a reporting event

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

If the SR ID is associated with LTM event triggered reporting, it should be possible to share the same SR ID with SR for other purposes. If the SR is not configured, the MAC CE reporting the event is provisioned by triggering CBRA. The dedicated SR should not be associated with specific event but shared between configured events. Further information on the reported event is provided by the MR MAC CE.

Proposal 22: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE.

Proposal 23: The dedicated SR ID is not associated with a particular event, further information on the event is provisioned by MR MAC CE.

Proposal 24: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 25: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

### 2.1.9. Transmitting the MAC CE for LTM Reporting

RAN2 agreed MAC CE is used as a reporting container. The LTM report MAC CE should be able to be provisioned on any UL grant.

Proposal 26: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 27: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

# 3	Conclusion

This paper discussed RAN2 aspects of LTM L1 measurement framework which shall be enhanced as a part of Release 19.

The following observations have been made:

Observation 1: When the serving beam and candidate beam are associated with different RS types it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported.

Observation 2: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Observation 3: In LTM, the UE is not switched to a cell where any beam can be selected but the UE is switched to a specific beam (inter-cell).

Observation 4: The TCI state information is always present in the cell switch command. Thus the event report triggering should be operating on beam level and not cell level.

Observation 5: As the LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting for multiple candidate cells in a single MAC CE.

Observation 6: In L1 UCI reporting format the number of configured RS in the resource set is used to determine the bit width of the Resource identifier field.

Observation 7: The resource indicator ID space can be reused for each candidate cell in the LTM-CSI-configuration associated with reported event.

Observation 8: up to N beams can be reported per each of the candidate cell.

The following has been proposed:

Proposal 1: From RAN2 perspective same type of RS is used for event evaluation and the UE derives the same type of RS. Details can be handled by RAN1.

Proposal 2: the value of TTT is configurable by NW and one TTT value is configured per reporting configuration (per event).

Proposal 3: The TTT range is to be decided in the Stage-3 discussions.

Proposal 4: The event evaluation is performed at MAC layer by having a beam specific time to trigger (TTT) timer.

Proposal 5: If the current beam (i.e. a beam corresponding to the indicated TCI state) for event evaluation changes, the evaluation for the candidate RS is restarted.

Proposal 6: RAN2 considers the aforementioned list of open issues when designing the report on leave for L1 measurement reporting.

Proposal 7: The periodic reporting of the LTM event triggered report can be configured per reporting configuration with configurable periodicity and/or number of repetitions.

Proposal 8: RAN2 to discuss support for resource allocation: whether the UE requests resource for each repetition of the report, or the NW provides a periodic UL grant.

Proposal 9: The LTM Report MAC CE includes indication of a candidate cell identifier that points to the configured LTM candidate identifier(s).

Proposal 10: The LTM Report MAC CE supports reporting of candidate RSs for multiple candidate cells in a single MAC CE.

Proposal 11: the LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

Proposal 12: The LTM Report MAC CE includes RS index field wherein the RS index field size is fixed 6 bits (to reflect up to 64 RS indices).

Proposal 13: The LTM Report MAC CE includes RS index field that encodes resource indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 14:  The resource indicator SSBRI/CRI in the RS index field is associated with Candidate identifier.

Proposal 15: The LTM Report MAC CE supports reporting of variable number of candidate RS per candidate cell(s), up to the configured number of RSs.

Proposal 16: The LTM Report MAC CE includes the RSs that have fulfilled the reporting criteria for the reported event. RSs that do not fulfil the reporting criteria are not reported.

Proposal 17: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

Proposal 18: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE.

Proposal 19: Include serving cell RSRP measurement for events for which the current beam RS is used for event evaluation i.e. LTM 2/3/5.

Proposal 20: UE can be configured to include RS associated with indicated (and activated TCI states) in the event based measurement reporting.

Proposal 21: Support truncation of LTM report MAC CE. Details FFS (once the work on MAC CE format matures).

Proposal 22: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE.

Proposal 23: The dedicated SR ID is not associated with a particular event, further information on the event is provisioned by MR MAC CE.

Proposal 24: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 25: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

Proposal 26: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 27: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

# References

[1]	RP-242356 Revised Work Item: NR mobility enhancements Phase 4 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] R2-2405701	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #126	 Fukuoka, Japan, May 20 – 24, 2024

[4] R2-2409211 Report from session on V2X/SL, R18/19 MOB, and R19 NES 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024