3GPP TSG-RAN WG2 Meeting #128	R2-2410467

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.2.5

Source:	Nokia

Title:	RAN2 Aspects for AIoT Operation in Topology 2

WID/SID:	FS_Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# Introduction

RAN2 has made following agreements in RAN2-128 related to Topology 2.

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this paper we discuss the remaining issues related to E2E procedures, architecture and control plane aspects of Topology 2.

# 2	Discussion

## 2.1	Architecture And E2E procedures

### 2.1.1 RAN2 Impacts for Topology 2 Procedures

For the reader functionality to be operation with suitable resource allocation for AIoT interface and also communication mechanism with gNB multiple procedures needs to be completed including reader authentication and registration. In this section we discuss about reader registration and selection procedure and impacts to Uu Interface for these procedures

GNB Awareness of Reader Registration

For Topology 2, the radio resource in the Uu Interface needs to be allocated inline with the type of AIoT procedure /traffic being carried out in the Uu Interface. Moreover, for AIoT interface operating in the same band as Uu Interface the GNB needs to be involved in resource allocation for the AIoT radio interface. This coordination is needed for co-existence of AIoT radio interface and Uu-Interface operation. Hence the gNB involved in topology 2 need to know the reader functionality of UE and also the capabilities of the reader related to AIoT-interface that impacts the Uu Interface operation. The reader functionality activation can be informed from CN at the end of registration. However the capability related to AIoT-functions can be either provided from CN or directly from UE via the UE capability information.

Proposal  1:  The reader registration procedure supports indication of registration status to gNB  . FFS How the Reader capability on AIoT-Interface is indicated to GNB.

### 2.1.2	User Plane Architecture for T2

The AIoT messages can be carried transparently as NAS payload or UP traffic over the interface between reader and GNB. However, if there could be some cases where GNB may need to know some additional information about the AIoT related message transfers that can be used by GNB for reader selection and discovery later. For better radio resource control for downlink RRC based AIoT Information is preferred. For uplink transmission either NAS based on transperant option can be considered

Proposal 2: For downlink AoT RRC based option is supported. For uplink either NAS based or Transparent architecture can be considered.

The signalling procedures for Inventory and command procedures are under discussion in RAN2.  In case of topology 2 there can be two options to handle the complete AIoT transaction via reader. Either the reader can complete the whole procedure and provide the outcome towards AIoT-Function via NR Air-interface. Alternatively, the reader may forward each transaction transparently.  Each approach have different impacts and advantages. We propose that RAN2 to conclude on the AIoT signalling procedure for above use-cases and analyse the changes needed in data forwarding based on the same.

Proposal 3: RAN2 to study the data forwarding schemes and modifications needed in data forwarding over Uu-radio-interface once the AIoT interface signalling procedure is finalised.

## 2.2	Control Plane Aspects

### 2.2.1 Reader RRC State during AIoT Operation

In AIoT, although the amount of data per message is low (~1000 bits) the data rate is also within the area of few, or even fractional kbps. Given a larger set of devices, and multiple messages being transferred within a single procedure, the duration of an AIoT session i.e. from the initial paging being sent till all devices has replied, may comprise several seconds. In this case, having a UE being in RRC_CONNECTED for the entire duration of the AIoT session seems not practical. Additionally, there is no need to require that for communication with an AIoT device the reader UE shall be in RRC_CONNECTED state since this communication is possible for the AIoT device in a stateless manner. Furthermore, Rel-18 brings SDT to both UL and DL, and thus it is possible to communicate with the reader even when it is in RRC_INACTIVE.

Observation 1: Maintaining the Reader UE in RRC_CONNECTED state during the entire AIoT session is not resource efficient.

Observation 2: SDT transmission may be sufficient to start/stop an AIoT session within a reader and provide reader data back to the AIoT access function.

Proposal 4: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE for delivery of AIoT data transmission over Uu Interface. FFS on RRC_IDLE.

The UEs in RRC_INACTIVE state can connect to the network by performing Random Access procedure and move to RRC_CONNECTED state, or if the data to be transmitted is small, the UE can perform RA-SDT procedure and transfer the data via RRC Resume request message. The RA-SDT procedure can use 2 step/4 step RA procedure based on the UE configuration. To engage a reader UE for AIoT procedure, it needs to be configured with the inventory or command information. It would be beneficial if the UE can stay in RRC_INACTIVE state and receive the AIoT configuration and perform AIoT communication with AIoT devices while being in RRC_INACTIVE. This would help to save the transition of the UE to RRC_CONNECTED state to receive the AIoT configuration. This maybe an even more significant consideration for applications that require periodic paging of AIoT devices.

Proposal 5: RAN2 to discuss whether a Reader UE should be paged and moved to RRC_CONNECTED to perform an AIoT session or the Reader UE can be configured for AIoT and enabled to perform AIoT communication while being in RRC_INACTIVE state.

### 2.2.2 Uu Interface Resource Allocation

The AIoT related data transmission over Uu Interface can be sent using existing means of connected mode resource allocation. However depending on the nature of the AIoT command and interaction either periodic scheduling or semi-persistent resource allocation may be needed.  For UE in RRC-INACTIVE state instead of setting up RRC connection for delivering the AIoT packets SDT or CG allocation can be considered. However these resource allocation may need to be changed based on the status of AIoT procedure in the AIoT interface. In such cases some adaptation to the Uu Interface resources also may be needed.

Proposal 6: RAN2 to discuss signalling procedure impacts related to UU-Interface resource allocation and adaptation for Topology 2 operation.

## 2.2.3  AIoT Interface Resource Allocation

AIoT request is terminated at gNB or at reader UE based on the means in which it is communicated. In case of the AIoT request terminating at the reader UE, the reader UE need to provide this information to gNB and request for resources. In case of the AIoT request terminating at gNB, the gNB can provide the AIoT request and the resource allocation to the reader UE.

The resources allocation by the gNB can be performed in a one-step process after receiving the AIoT request. Upon reception of AIoT request by UE or gNB, the complete resources and resource validity information is indicated to the reader UE by the gNB. Other alternative is to indicate the resources and resource validity information in a two-step process. The initial set of resources can be indicated to the reader UE during the reader registration process and subsequently the remaining information on the resources and resource validity information can be indicated upon receiving the AIoT request. In this scenario, the initial set of resources allocated in step-1 can be modified in step-2 and the reader UE can use the resources allocated together in step-1 and step-2 for AIoT communication.

Proposal 7A: RAN2 to consider 1-step and 2-step resource allocation from gNB to the reader UE. RAN2 to consider steps 3 and 4 of Figure 1as baseline for resource allocation and steps 5, 6 and 7 can be pursued based on the agreements in RAN3 and SA2.

In case of a bad channel between the reader UE and the AIoT device, it would not help the reader UE to use the resources allocated to reader UE if the layer 1 received signal strength of the received signal is below certain threshold. In case of RLF, the validity time of the allocated resources need to be different from the regular validity time.

The reader UE might sense that the resources allocated for a given time is not enough as the procedure might extend and the resources may be required beyond the validity time of the resources. In such scenarios, it is required for the reader UE to know if a request for extension of already allocated resources are allowed.

Apart from this information, a default timer for the resource validity information is required to indicate the validity time of the allocated resources in a normal scenario.

Proposal 7B: RAN2 to consider including the following aspects in the AIoT resource as part of resource validity information.

- RSRP threshold beyond which the AIoT resources are not valid

- RLF recovery time beyond which the resources are not valid in case of RLF

- Access slot length to be used along with the allocated resources

- Default validity timer for the resources under normal circumstances

The reader UE may complete the AIoT request before the validity time of the resources allocated for it. In such scenarios, the allocated resources will not be used by the reader UE for which it has been allocated or by other reader UEs as well. It is required by the reader UE to indicate the gNB that the resources are not needed anymore. The reader UE can also indicate the completion of AIoT request by sending AIoT report. The gNB can also release the resources allocated (if needed) upon reception of AIoT report.

Proposal 7C: RAN2 to consider message exchanges between gNB and reader UE in resource release request from reader UE and resource release indication from gNB.

## 2.3 Reader Selection

### 2.3.1	Reader Selection Procedure

When multiple readers are registered with AIoT access function, a specific reader needs to be selected for triggering AIoT services for the AIoT devices at specific location. In case of two different readers involved, i.e., one for transmitting on R2D link and another on D2R link, a reader pair needs to be selected by the AIoT access function. This selection of readers for devices may help avoiding a device responding to multiple readers and occupying network resources. The reader registration may include additional information that assist the AIoT access function to select specific reader(s) towards AIoT devices in certain location or towards certain group of AIoT devices.  AIoT procedures to trigger the AIoT access function need to identify the anchor BS that servers the relevant reader devices to trigger the reader discovery procedure. There could be some RAN2 aspects in reader discovery  and providing additional information for either the BS or AIOT-function to select the suitable reader to start the AIoT procedure.

Proposal 6: In Topology 2 GNB is responsible for selection of reader(s) for AIoT operation.

For AIoT operation in specific area, the GNB need to select suitable reader based on the location and the radio-condition of Uu-Interface and possibility the radio-condition for AIoT operation. For the radio condition for AIoT Operation the reader assistance information based one earlier AIoT interactions will be beneficial.

Proposal 6A: RAN2 to agree that readers are selected based on actual radio condition/ location. FFS whether this includes reader assistance information .

### 2.3.2	Multi-reader Support

Another aspect of the architecture is to discuss whether the AIoT design should include diversity like behaviour, in the sense that multiple readers could be allowed to receive from the same device, on the same paging request. RAN1 already has defined scenarios of having different readers associated to a particular device, i.e. one reader for activating the device and another reader for reading the signal from the device. Along with this, the scenario of independent CW provider is also considered. This means that it is already expected that reader operation can be dedicated, and thus RAN2 should not preclude a AIoT access function configuring multiple readers for i.e. D2R communication, or even R2D.

Observation 6: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 7: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

### 2.3.3	GNB Coordination for multi-reader scenario

In topology 2, the gNB may select multiple reader UEs to serve a specific AIoT request. In this scenario, the gNB will have to assign the session id to multiple readers because the gNB will need to collect responses from multiple readers, collate and report it back to the AIoT function. The gNB may also allocate the same session id for different sessions in multiple readers associated with the same AIoT service request.

Observation 7: Session id allocation is required and to be done by the gNB in case of multi-reader scenario.

Proposal 8: RAN2 to agree that in case of multi-reader scenario in topology 2, the session id is assigned by the gNB.

In case of multi-reader scenario in topology 2, when one of the reader UE is undergoing mobility, the data collected during the time of mobility need to be reported back to the source gNB as the source gNB need to collate the AIoT information from multiple readers and send it to the AIoT function.

In such scenarios, post successful handover, it would be helpful for the reader UE to indicate the data availability prior to or during the handover procedure that need to be sent to source gNB, as the source gNB may not need all the information available from the handed over reader UE. Indicating the data availability alone is not enough for the source gNB to decide if it requires the data to be sent, for instance the session from which the data is available is important to decide by the source gNB. Hence it is required to identify the reader UE and the session uniquely to decide to fetch this data from the handed over reader UE. Hence the reader UE and each of the sessions in those reader UEs should be uniquely identifiable. A unique id representing reader UE and a session can be allocated to each session of the reader UE. This unique id can be used by the source cell along with the data availability to decide whether or not that data need to be retrieved from the reader UE.

Observation 8: In case of a handover in multi-reader scenario, the data collected at the reader UE per session corresponding to source gNB post-handover need to be indicated.

Proposal 9: RAN2 to agree on a unique identifier identifying the reader UE and an AIoT session in that UE which can be used in case of multi-reader handover scenarios to indicate the data availability and aid in deciding the need to retrieve the data.

## 2.4 Open Issues related to Topology 2 for study item completion

Based on the above analysis, following are the open issues in the study item for Topology 2 aspects

- The resource allocation signalling procedure for AIoT interface requires further study on latest agreed functional aspects for complete unified procedure. Moreover, the resource allocation signallng procedure may differ depending on the different architecture options for user-plane.

- The resource control for multi-reader scenario and coordination of operation across multiple-readers for T2 are not yet discussed.

- The Uu Interface impacts correspond to Uu-Interface resource allocation for different RRC states requires further discussion.

Proposal 10 : RAN2 to consider the status of following open issues of T2 for completion of study item

- AIoT Resource allocation and maintenance for the agreed functional aspects of AIoT-Interface.

- Multi-Reader scenario handling in T2

- Uu-Interface resource allocation considering different RRC-states of reader.

# 3	Conclusion

In this discussion paper we analyse the RAN2 impacts for AIoT procedures in T2 and made following observations and proposals

Architecture And E2E Procedures

Proposal  1:  The reader registration procedure supports indication of registration status to gNB  . FFS How the Reader capability on AIoT-Interface is indicated to GNB.

Proposal 2: For downlink AoT RRC based option is supported. For uplink either NAS based or Transparent architecture can be considered.

Control Plane Aspects

Proposal 3: RAN2 to study the data forwarding schemes and modifications needed in data forwarding over Uu-radio-interface once the AIoT interface signalling procedure is finalised.

Observation 1: Maintaining the Reader UE in RRC_CONNECTED state during the entire AIoT session is not resource efficient.

Observation 2: SDT transmission may be sufficient to start/stop an AIoT session within a reader and provide reader data back to the AIoT access function.

Proposal 4: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE for delivery of AIoT data transmission over Uu Interface. FFS on RRC_IDLE.

Proposal 5: RAN2 to discuss whether a Reader UE should be paged and moved to RRC_CONNECTED to perform an AIoT session or the Reader UE can be configured for AIoT and enabled to perform AIoT communication while being in RRC_INACTIVE state.

Proposal 6: RAN2 to discuss signalling procedure impacts related to UU-Interface resource allocation and adaptation for Topology 2 operation.

Proposal 7A: RAN2 to consider 1-step and 2-step resource allocation from gNB to the reader UE. RAN2 to consider steps 3 and 4 of Figure 1as baseline for resource allocation and steps 5, 6 and 7 can be pursued based on the agreements in RAN3 and SA2.

Proposal 7B: RAN2 to consider including the following aspects in the AIoT resource as part of resource validity information.

- RSRP threshold beyond which the AIoT resources are not valid

- RLF recovery time beyond which the resources are not valid in case of RLF

- Access slot length to be used along with the allocated resources

- Default validity timer for the resources under normal circumstances

Proposal 7C: RAN2 to consider message exchanges between gNB and reader UE in resource release request from reader UE and resource release indication from gNB.

Reader Selection And Multi-Reader scenario handling

Proposal 6: In Topology 2 GNB is responsible for selection of reader(s) for AIoT operation.

Proposal 6A: RAN2 to agree that readers are selected based on actual radio condition/ location. FFS whether it includes reader assistance information

Observation 6: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 7: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

Observation 7: Session id allocation is required to be done by the gNB in case of multi-reader scenario.

Proposal 8: RAN2 to agree that in case of multi-reader scenario in topology 2, the session id is assigned by the gNB.

Observation 8: In case of a handover in multi-reader scenario, the data collected at the reader UE per session corresponding to source gNB post-handover need to be indicated.

Proposal 9: RAN2 to agree on a unique identifier identifying the reader UE and an AIoT session in that UE which can be used in case of multi-reader handover scenarios to indicate the data availability and aid in deciding the need to retrieve the data.

T2 Open Issues for AIoT-Study Item

Proposal 10 : RAN2 to consider the status of following open issues of T2 for completion of study item

- AIoT Resource allocation and maintenance for the agreed functional aspects of AIoT-Interface.

- Multi-Reader scenario handling in T2.

- Uu-Interface resource allocation considering different RRC-states of reader.