3GPP TSG-RAN WG2 Meeting #128	R2-2410469

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.5.3

Source:		Lenovo

Title:			Case 2 and remaining essential issues

Document for:	Discussion and Decision

# Introduction

This paper builds on the agreements made on OD-SIB1 in previous (RAN2#127 and 127bis) meeting:

RAN2#127:

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

RAN2#127bis:

1.	We will inherit all agreements made during SI phase to WI phase.

2.	A NES cell can include neighbouring NES cell’s WUS configuration.

3.	NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

4.	In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

5.	The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

6.	The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

7.	A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

9.	Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

10.	NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

11. RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

# Discussion

## Case 2

Discussion around “checkpoint” that started in RAN Plenary meeting #105 in Melbourne on Case 2 were like below:

RP-242141 On demand SIB 1 and the need for case 1 for multi-vendor RAN Vodafone, Deutsche Telekom, Orange, Lenovo, Google, Fraunhofer IIS and Fraunhofer HHI, ETRI, CEWiT Replaces RAN WG1 and RAN WG2 discussed 3 scenarios for the support of on demand SIB1: - Case 1: Standalone, where configuration to request SIB1, SIB1 request and SIB1 are delivered within the SIB1-less NES cell - Case 2: Anchor: configuration to request SIB1 is delivered by an Anchor cell, but SIB1 request and SIB1 are delivered within the SIB1-less NES cell - Case 3: Anchor: configuration to request SIB1, SIB1 request and SIB1 (for the SIB1-less NES cell) are delivered by the Anchor cell Proposal 1: The co-signing companies recommend supporting Case 1 in Rel 19 in addition to case 2. DT: some clarification is needed before we could go forward with case 1, anchor cell can be same as NES cell? a cell could work as normal cell during the day and as NES cell in the night, if these are 2 different cells, then we cannot agree to case 1 RAN chair: focussing on case 2 is acceptable? Qualcomm: if WUS configuration for neighbours is still possible, then we would be ok RAN chair: asks RAN1 chair to draft a way forward in RP-242353 The document was noted. Replaced by RP-242353 Way forward on NES checkpoint RAN1 chair (Samsung) The document was revised. Replaced by RP-242384 RP-242384 Way forward on NES checkpoint RAN1 chair (Samsung) Replaces RP-242353 DT: would like to remove restriction that Cell A and NES cell must be different (at different times) Panasonic: things that this is possible RAN chair: as you talk about different times, this is possible conclusion: proposal is endorsed

RAN2 agreed that a NES Cell my provide SIB1 request WUS configuration for itself. Further, according to the agreed WID for NES, only case 2 is accepted to be worked on. Case 2 however hinges on presence of an Anchor cell, without it there will be no WUS configuration available to be used for requesting SIB1 of a neighbouring NES Cell.

To fill this gap, as agreed in RAN2#127bis, the NES Cell may provide WUS configuration for itself such that received WUS configuration can be used to request SIB1 from the NES Cell when it has stopped regular broadcasting of SIB1. But to obtain WUS Configuration contained in a new SIB from a NES Cell, UE needs to request this new SIB which may be provided on on-demand basis. To request new on-demand SIB1, UE needs the SIB1 to have the SI Request configuration. This is only possible when the UE already has valid SIB1 of the NES Cell!

Observation 1: To request SIB1 of a NES Cell, UE needs its SIB1 Request WUS configuration. To even receive WUS configuration of the NES Cell from the NES Cell, the UE must be having the NES Cell’s SIB1 already!

The UE can only obtain SIB1 of NES Cell by using NES Cell’s WUS configuration provided from a Cell A – which greatly diminishes NES’s cell’s provisioning of its own SIB1 WUS configuration, unless it could be assured that once obtained that the obtained WUS configuration remains valid for a time duration. Thereby, the UE shall be able to request SIB1 using WUS configuration sometimes later as well, without necessarily having to find the Anchor Cell again.

Observation 2: Design based on past RAN2 agreement(s) greatly diminishes (even renders useless) NES’s cell’s provisioning of its own SIB1 WUS configuration.

Therefore, to enable the SIB1 request the previously acquired WUS configuration (SIB1 request configuration) must be considered valid at the time of making the SIB1 request. This can be done if the WUS configuration includes a validity timer e.g., in hours. As long as a UE has a valid WUS configuration, it can request on-demand SIB1. Validity should exceed the general SIB validity time (of 3h) and use values like: 1h, 3h, 6h, 12h, 24, 48, 96, 192 … etc., which is really as Stage-3 matter.

This would enable an energy saving deployment in campus scenarios, in shopping malls etc., where during nighttime not many users are expected to be present, except residents/ employees present in daytime as well and the operator is therefore not forced to deploy another layer for providing an anchor cell.

So, we propose:

Proposal 1: WUS configuration includes a validity timer to control validity of the WUS configuration.

Proposal 2: Network configures the “remaining validity time” of WUS configuration as part of WUS configuration in the new agreed SIB.

## SIB1 on-demand indication and barring legacy UEs

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

RAN1 has made some agreements (copied above) that uses the MIB information to indicate legacy UEs that SIB1 is not being provided in the cell and they are still discussing if specific kSSB values >23/11 (for FR1 and FR2 respectively) can be used to indicate another GSCN where the legacy UE find SIB1. Except for kSSB values = 30/14, the other values in this range will lead legacy UEs to look for SIB1 “elsewhere” – whereas the NES on-demand SIB1 Cell should not be transmitting SIB1 at this “elsewhere” location – otherwise, how is it saving energy!

So, we should avoid making agreements that make legacy UEs make un-necessarily look for SIB1.

Proposal 3: We strive to restrict kSSB to values that will avoid legacy UEs make un-necessarily look for SIB1 when the same is not transmitted on Sync raster.

However, if RAN1 do have some reason to use specific kSSB values >23/11, then RAN2 needs to discuss how a UE acquires broadcasted SIB1? UE needs to check if SIB1 is currently broadcasted before it could go ahead and make SIB1 Request from the NES cell, as agreed in the last meeting:

8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Therefore, if the ssb-SubcarrierOffset, is used for special purpose and can’t really provide a valid SSB Subcarrier offset, and the UE still needs to verify if it can receive broadcasted SIB1, the kSSB should be provided along with the WUS configuration by the Cell A. We assume here that the pdcch-ConfigSIB1 can be used directly from the MIB of the NES cell i.e., RAN1 has no need to repurpose this IE. Conservatively, both kSSB and pdcch-ConfigSIB1 can be provided in a new SIB of Cell A along with already agreed WUS Configuration.

Proposal 4: Both kSSB and pdcch-ConfigSIB1 can be provided in a new SIB of Cell A along with already agreed WUS Configuration, at least if the detected kSSB in the NES MIB is between 24 and 29.

The legacy UEs (and UEs not supporting on-demand SIB1 feature) can be barred using either:

Option a) MIB barring: This should generally be the last option for barring (operator don’t like to use) but since otherwise the SIB1 can’t be obtained by such UEs, these will not even be able to make emergency calls.

Option b) Using kSSB as value 30. This will not lead the legacy UEs to look for SIB1 ‘elsewhere’ and can also signal on-demand SIB1 feature capable UE to indicate that the cell is providing SIB1 on on-demand basis.

We prefer the latter option b) since it has the added advantage for the on-demand SIB1 feature capable UE and do not lead the legacy UEs to look for SIB1 ‘elsewhere’.

Proposal 5: Using kSSB value 30 in NES MIB is sufficient to effectively bar legacy UEs on a on demand SIB1 cell when SIB1 is not being broadcasted.

## Cell reselection:

RAN2 should strive to reuse the current principles as much as possible. As an example, we can reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ) to decide if intra/ inter frequency measurements need to be performed.

If the best ranked cell or a higher priority inter-frequency cell is R19 NES Cell UE should go on and request SIB1. Here we assume Network can ensure SIntraSearchP and SIntraSearchQ are sensible to allow time to the UE to acquire SIB1 of NES Cell before the serving cell deteriorates too much.

Proposal 6: For cell reselections, reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ) to decide if intra/ inter frequency measurements need to be performed.

Proposal 7: If the best ranked cell or a higher priority inter-frequency cell is a R19 NES Cell, then UE should go on and request SIB1.

Following agreement was made in RAN2#127bis:

  - NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

To avoid the legacy UE camping at the Cell A attempting to switch to the NES Cell, the NES cell can be indicated in the legacy exclude-list, legacy UE does not consider the cell in the exclude-list as candidate cell for cell-reselection. To allow the R19 NES UE camping at the Cell A attempting to switch to the NES Cell, additional exclude-list cells related information relevant for cell re-selection may be configured in Cell A, R19 NES UE ignores the legacy set of excluded cells and only consider the additional set of excluded cells, i.e., R19 UE does not use the additional set of excluded cells for cell re-selection.

Proposal 8: Cell A transmits additional set of exclude-list cells related information relevant for cell re-selection, R19 NES UE uses the additional set of excluded cells for cell re-selection and ignore the legacy set of excluded cells.

To avoid the legacy UE camping at the Cell A attempting to switch to the NES Cell, the NES cell can be configured with lower priority level in the neighbor cell list, legacy UE deprioritizes the NES cell for cell-reselection. For R19 NES UE, a configuration is introduced to configure the frequency priority or sub-priority of the NES cells, which is different with the frequency priority of the non-NES cells of the same frequency.  Therefore, R19 NES UEs is allowed to reselect to cells that are deprioritized for legacy UEs.

Proposal 9: Cell A transmits additional cell reselection priority of a set of NES cell relevant for cell re-selection, R19 NES UE uses the additional cell priority for cell re-selection and ignore the legacy priority.

RAN2 has agreed if the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell, e.g., UE has not acquired WUS config of the NES cell. However, if the UE would acquire WUS configuration for the barred NES Cell, then there’s no reason to keep the NES cell as barred.

Proposal 10:  If a NES Cell is considered as barred since UE was previously unable to acquire SIB1 (or even WUS config), UE considers the NES cell as not barred anymore once it obtains WUS config of the same NES cell.

# Conclusion

Remaining essential issues were discussed in the paper for the on-demand SIB1 topic. Following proposals are made:

Observation 1: To request SIB1 of a NES Cell, UE needs its SIB1 Request WUS configuration. To even receive WUS configuration of the NES Cell from the NES Cell, the UE must be having the NES Cell’s SIB1 already!

Observation 2: Design based on past RAN2 agreement(s) greatly diminishes (even renders useless) NES’s cell’s provisioning of its own SIB1 WUS configuration.

Proposal 1: WUS configuration includes a validity timer to control validity of the WUS configuration.

Proposal 2: Network configures the “remaining validity time” of WUS configuration as part of WUS configuration in the new agreed SIB.

Proposal 3: We strive to restrict kSSB to values that will avoid legacy UEs make un-necessarily look for SIB1 when the same is not transmitted on Sync raster.

Proposal 4: Both kSSB and pdcch-ConfigSIB1 can be provided in a new SIB of Cell A along with already agreed WUS Configuration, at least if the detected kSSB in the NES MIB is between 24 and 29.

Proposal 5: Using kSSB value 30 in NES MIB is sufficient to effectively bar legacy UEs on a on demand SIB1 cell when SIB1 is not being broadcasted.

Proposal 6: For cell reselections, reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ) to decide if intra/ inter frequency measurements need to be performed.

Proposal 7: If the best ranked cell or a higher priority inter-frequency cell is a R19 NES Cell, then UE should go on and request SIB1.

Proposal 8: Cell A transmits additional set of exclude-list cells related information relevant for cell re-selection, R19 NES UE uses the additional set of excluded cells for cell re-selection and ignore the legacy set of excluded cells.

Proposal 9: Cell A transmits additional cell reselection priority of a set of NES cell relevant for cell re-selection, R19 NES UE uses the additional cell priority for cell re-selection and ignore the legacy priority.

Proposal 10:  If a NES Cell is considered as barred since UE was previously unable to acquire SIB1 (or even WUS config), UE considers the NES cell as not barred anymore once it obtains WUS config of the same NES cell.

# References

[1]: RAN1 meeting minutes from RAN2#126

[2]: RAN1 meeting minutes from RAN2#117