3GPP TSG-RAN WG2 #128	R2-2410474

Orlando, US, 18th – 22th October 2024

Agenda Item:	8.3.2.1

Source:	Ericsson

Title:	Simulation results for Temporal, Frequency and Spatial domain RRM measurement predictions

Document for:	Discussion

# 1	Introduction

A new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 with the following objectives [1]. In RAN2#126 meeting, priorities for RRM measurement prediction were agreed to well-proceed the simulation in the next meetings [2].

Case A is an urban micro-scenario, and Case B is an urban macro-scenario, more details in the annex.

| Case number | Prioritization | Evaluation scenario combination | Target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 2nd goal |

In this contribution, the simulation results for those cases highlighted as high priority as well as the spatial domain prediction evaluation (with medium priority) are discussed.

# 2	Discussion

### 2.1	Evaluation of temporal prediction

Since the simulation results for the first goal of the study i.e., to study the RRM measurements for the sake of reducing the measurement overhead, is stable without any relevant/serious open question, in this contribution we only present the evaluation of the AI/ML performance targeting to enhance the handover performance, assuming that in 3GPP we can conclude the usefulness of the AIML for the measurement overhead reduction.

In general, in this contribution, we evaluate the following scenarios:

- Case A, sub-case 2: prediction cell level measurements from cell level measurements targeting to mobility performance improvement

- Case A, sub-case 3: prediction cell level measurements from beam level measurements targeting to mobility performance improvement

Cell level measurement prediction based on cell level measurements (Case-A, sub-case 2)

As mentioned above in sub-case 2, we use cell-level measurement results for both model input and the output. Specifically, we use cell-level results sampled every 80ms at each time instance depicted with { …, T0-1, T0} in Figure 1. As shown, the observation window is fixed to 5x80ms to predict the cell-level results which is (N x 80 ms), wherein N Î {1, 2, 3, 4, 5}, starting right after the observation window T0. For example, as the case of prediction ratio 5, prediction step 80ms illustrated in Figure 1 we use 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input to predict cell-level measurements at time instance T0+1 which is 80ms after time instance T0. For the case of ratio 1, with the prediction step 80ms illustrated in Figure 1. We predict cell-level results at multiple time instances in the period of T0+1 ~T0+5 by using 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input.

In addition, as the case of prediction ratio 5’ in Figure 1, we use 25 cell-level RSRP measurement sampled at time instance T0~T0-24 as model input to predict cell-level results at multiple time instances in the period of T0+1 ~T0+5

As mentioned above in sub-case 2, we use cell-level measurement results for both model input and the output. Specifically, we use cell-level results sampled every 80ms at each time instance depicted with {T0-4, …, T0-1, T0} in Figure 1. As shown the observation window is fixed to 5x80ms to predict the cell-level results which is (N x 80 ms), wherein N Î {1, 2, 3, 4, 5}, starting right after the observation window T0. For example, as the case of prediction ratio 5, prediction step 80ms illustrated in Figure 1 we use 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input to predict cell-level measurements at time instance T0+1 which is 80ms after time instance T0.

For the case of ratio 1, with the prediction step 80ms illustrated in Figure 1. We predict cell-level results at multiple time instances in the period of T0+1 ~T0+5 by using 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input.

Figure 1: L3 filtered cell level measurement predictions from L3 filtered cell level measurement results; Case 2 and Case A. Cell level measurement sampling rate is 80ms. For ratio 5, prediction window is 80ms and prediction step size is N*80ms wherein N Î {1, 2, 3, 4, 5}. For ratio 1, prediction window is 400ms and prediction step fixed to 80ms. For ratio 5’, prediction window is 2000ms and prediction step size is N*80ms wherein N Î {1, 2, 3, 4, 5}

We evaluate the prediction accuracy (measured as RSRP difference) with different prediction window size that is a multitude of N, which leads to prediction window size between 80ms to 400ms. The evaluation results are provided in the following.

Table 1. Case A, sub-case 2: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results. Predicting single instance of RSRP in different prediction window i.e., a separate model is trained for each instance.

| Ratio 5 (OW: 400ms, PW: 80ms) | Averaged RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction step(ms) | 80 | 160 | 240 | 320 | 400 |
| UE moving speed 60km/h | 0.217 | 0.467 | 0.767 | 1.139 | 1.497 |
| UE moving speed 120km/h | 0.269 | 0.571 | 0.971 | 1.356 | 1.732 |

Table 2. Case A, sub-case 2: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results. Predicting 5 instances of RSRP in a fixed prediction window i.e., a single model predicts all 5 instances. Impact of different size of observation window is negligible.

| Ratio 1 (OW: 400ms, PW: 400ms) | Average RSRP difference(db) | RSRP difference (dB) for each position in prediction window |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction step(ms) | 80 | 1st | 2nd | 3rd | 4th | 5th |
| UE moving speed 60km/h | 0.817 | 0.222 | 0.458 | 0.78 | 1.13 | 1.494 |
| UE moving speed 120km/h | 0.997 | 0.294 | 0.603 | 0.973 | 1.362 | 1.754 |

| Ratio 5’ (OW: 2000ms, PW: 400ms) | Average RSRP difference(db) | RSRP difference (dB) for each position in prediction window |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction step(ms) | 80 | 1st | 2nd | 3rd | 4th | 5th |
| UE moving speed 60km/h | 0.761 | 0.232 | 0.439 | 0.713 | 1.038 | 1.384 |
| UE moving speed 120km/h | 0.976 | 0.345 | 0.59 | 0.942 | 1.308 | 1.696 |

The evaluation results captured in Table 1, shows that in a scenario with UE mobility set to 60 km/h the AIML model can predict the RSRP value with up to 1dB difference compared to the ground truth value, when prediction window size is less than 320ms. In a higher mobility scenario (i.e., when UE speed is 120 km/h) the RSRP difference/error is slightly higher.

Comparing ratio-5 and ratio-1 case, similar prediction accuracy is obtained for the respective prediction steps (note the green highlighted RSRP differences in different scenarios in Table 1 and Table 2). Comparing ratio 5 and ratio 5’ case, prediction accuracy of ratio 5’ case is only slightly better than ratio 5 case even though there are 5 times more measurements in the OW in ratio 5’ case. In other words, the increase of the OW size for the Case A has negligible impact on the prediction accuracy.

Therefore, we have the following observations:

The performance of a single model with 5 output RSRPs (e.g., ratio 1 shown in Table 2) is comparable to the multi-model training and inference (e.g., ratio 5 in Table 1 with one model per each prediction steps).

Unlike Case B, for Case-A sub-case 2, increasing OW length has negligible impact on prediction accuracy.

Maximum prediction step is below 320ms (irrespective of UE speed) if we consider 1 dB as acceptable threshold for RSRP difference.

Cell level measurement prediction based on beam level measurements (Case A, sub case 3)

As mentioned above in sub-case 3, we use beam-level measurements for model input and cell-level measurements is the label/output. Given that in this scenario we use raw beam level measurements as input to the model (i.e., no filtering mechanism is used), hence, difference from case 2, larger observation window length is considered to sample more beam-level measurements (e.g., beam-level results of 20 time instances, e.g., T0-19~T0 are sampled according to Figure 2) to give sufficient time domain information in each training and inference samples.

Specifically, we use beam-level results sampled every 80ms in observation window fixed to 20*80ms to predict the cell-level measurements which is N*80ms later from the end of the observation window (i.e., T0 (current time) + N*80ms). The prediction window size is set to 320ms to keep the “ratio 5” as agreed in last meeting. For example, as the case of prediction step 80ms illustrated in Figure 2, we use 20 beam-level RSRP measurements sampled in the observation window as model input to predict cell-level results at time instance T0+1~T0+4 which is 80ms~320ms later from time instance T0.

Figure 2: L3 filtered cell level measurement predictions from L1 filtered beam level measurement results; Case 3 and Case A. Beam level measurement sampling rate is 80ms. For ratio 5, prediction window is fixed to 320ms and prediction step size is N*80ms wherein N ∈{1, 2, 3, 4, 5}.

We simulate the prediction accuracy with different length of prediction step. The simulation results are provided in the following. For temporal prediction case A – sub-case 3 we evaluated the RSRP difference with prediction step 80ms~400ms. Simulation results is summarized in Table 3.

Table 3. Case A, sub-case 3: RSRP difference (prediction error) for Cell-level measurement prediction from beam level measurement results. Beam level measurement sampling rate is 80ms. prediction window is 320ms and prediction step size is N*80ms wherein NЄ {1, 2, 3, 4, 5}.

| Ratio 5 | Prediction steps(ms) | Averaged RSP difference(dB) | Separate RSRP difference(db) for each position in prediction window |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st | 2nd | 3rd | 4th |
| UE moving speed 60km/h | 80 | 0.741 | 0.628 | 0.638 | 0.745 | 0.954 |
|  | 160 | 0.831 | 0.698 | 0.722 | 0.849 | 1.054 |
|  | 240 | 1.025 | 0.882 | 0.919 | 1.042 | 1.26 |
|  | 320 | 1.104 | 0.95 | 1.028 | 1.137 | 1.299 |
|  | 400 | 1.125 | 1.006 | 1.025 | 1.139 | 1.33 |
| UE moving speed 120km/h | 80 | 0.825 | 0.676 | 0.698 | 0.842 | 1.085 |
|  | 160 | 0.94 | 0.796 | 0.821 | 0.959 | 1.183 |
|  | 240 | 1.02 | 0.894 | 0.896 | 1.025 | 1.266 |
|  | 320 | 1.184 | 1.091 | 1.097 | 1.187 | 1.372 |
|  | 400 | 1.269 | 1.203 | 1.214 | 1.243 | 1.386 |

In our simulation submitted as part of contribution [5] to RAN2#127b, we observed there is a substantial performance degradation in Case 3 - Case A comparing to results in Case 2 – Case A. We used the measurement results of the 8 beams of the cells as input to the model, while the actual cell level RSRPs are derived by linear combination of the 3 best beams of each cell (i.e., 5 beams are filtered out as part of beam consolidation procedure). This might have been a reason to get such degradation in the model performance in that case. In this contribution, we used measurement results of consolidated beams of cells as input to the model (consolidation procedure was illustrated in in Figure 3), and the prediction performance was on average ~30% better compared to the results shown in [5]. Improvement up to 47% was observed especially when model predict farther time instance, e.g., last position in prediction window with 400ms prediction step.

Figure 3. Impact of beam consolidation on the set of beams selected as model input.

In addition, beams are spatially different (transmitted to non-orthogonal directions) and such phenomenon might create uncorrelated behaviour among the input that finally affects the performance of the AIML model. Therefore, we have the following observation.

Using the beams used in consolidation procedure (for cell quality derivation) as input provides better performance compared to using all the measured beams as input.

## 2.2	Inter-frequency RRM measurement predictions

In this section, we show the prediction of 4 GHz RSRP with 2 GHz as input for both sub-case 2 and sub-case 3. We also considered the impact of different clustering methods, defined later, Spatial- and Frequency- clustering. We further assess the impact of frequency correlation vs frequency clustering on prediction performance. To assess such frequency clustering vs frequency correlation, we had to use other frequencies (other than 2 GHz and 4 GHz) either as input or output. Therefore, we included other frequencies, i.e., 0.8 GHz and 2.6 GHz, to help forming the input or output cluster in addition to 2 GHz and 4 GHz.

In the below analysis, ML RSRP difference (ML-Loss): is the average of the difference between measured L3 filtered Cell-RSRP (ground truth) and the predicted L3 filtered Cell-RSRP, measured in dB.

It is worth mentioning the 2 GHz (and 0.8, 2.6 GHz) is an FDD band whereas the 4 GHz is a TDD band, hence we aligned our system with such setup.

Furthermore, in the Meeting #127bis the impact of cluster-based approach on the AIML performance is discussed but no clear conclusion was drawn. Therefore, further investigation is needed on the impact of the  cluster to cluster approach. In inter-frequency prediction, we think that input/output cluster can be formulated in different methods:

- Spatial-Cluster: the cells within input-cluster are spatial cells operating in the same frequency but different spatial characteristics, and the cells output-cluster are in different frequency (to the input cluster), but they are collocated to each cell in the input-cluster. Hence, given a 7-sites, and 3-cells per-site, we can have up-to 21 cells (but not necessarily 21) belonging to single frequency within a single input-cluster. Then the output should have also up-to 21 cells belonging to another frequency (called Freq-Out). Example of InputOutput cells indexing is: In=[0,1,2]  Out=[63,64,65], where, cells [0,1,2] belong to a single frequency, while cells [63,64,65] belong to another (single) frequency, and pairs of cells, [0,63], [1,64], [2,65], are collocated spatial cells.

- Frequency-Cluster: the input-cluster cells should be collocated cells and the output cells should also be collocated cells (not inter-spatially). Here, we execute on four frequencies [0.8, 2, 2.6, 4] GHz, the maximum number of clusters can be only 3 frequencies in either input or output, where is the other is 1 frequency. Example of InputOutput cells Indexing is: In=[0,21,42]Out=[63], where [0,21,42] represents collocated cells in three frequencies, and [63] represent collocated cell on a fourth frequency.

### 2.2.1	Inter-frequency prediction sub-case 2

In this subsection, we focus our evaluation on mainly 2 GHz and 4 GHz, as shown in Table 4, in which we evaluate the model performance of L3 Cell-Level measurement as input on one (or more) frequency(s) and L3 Cell-Level measurement on other frequency(s) as output.

As shown in Table 4, we observe prediction performance loss with lowering the size of cluster (in Spatial-Clustering), as shown in rows 1, 2, 3, with RSRP difference = 0.23, 0.32, 0.66 dB, respectively. The opposite is also true, i.e., we observe performance prediction improvement with the increase of cluster size in Spatial-Clustering. Additionally, we observe when using Frequency-Clustering, the performance is worse than the spatial-Clustering of the same size, this can be observed by comparing rows 3, 4, and 6. Another point is observed when we compared row 4 and 5, where in row-4 we have 3-cells of Frequency-Cluster as input and one cell as output, RSRP difference = 0.83 dB, (we only have 4-frequencies), outperforms row-5 which has single to single frequency (with RSRP difference 1.07 dB). However, as we show later in Table 6, the improvement is not because of the cluster size but because the largest frequency in input cluster in row-4, i.e., 2.6 GHz, which is larger than input frequency in row-5 which is 2 GHz. Finally, it is shown, in row-6, that using single cell as input to predict Frequency-Cluster as output have the worst performance across all, i.e., 1.23 dB (more on this case is elaborated in Table 4.

Table 4. Inter-Frequency Prediction for sub-case 2

| row | Input Cells | Output Cells | RSRP difference (dB) |
| --- | --- | --- | --- |
| 1 | 9 Spatial Cells @ 2GHz [0, 1, 2, 3, 4, 5, 6, 7, 8] | 9 Spatial Cells @ 4GHz [63, 64, 65, 66, 67, 68, 69, 70, 71] | 0.23 |
| 2 | 6 Spatial Cells @ 2GHz [0, 1, 2, 3, 4, 5] | 6 Spatial Cells @ 4GHz [63, 64, 65, 66, 67, 68] | 0.32 |
| 3 | 3 Spatial Cells @ 2GHz [0, 1, 2] | 3 Spatial Cells @ 4GHz [63, 64, 65] | 0.66 |
| 4 | 3 Collocated Cells @(2,0.8,2.6) GHz [0, 21, 42] | 1 Cell @ 4GHz [63] | 0.83 |
| 5 | 1 Cell @ 2GHz [0] | 1 Cell @ 4GHz [63] | 1.07 |
| 6 | 1 Cell @ 2GHz [0] | 3 Collocated Cells @(0.8,2.6,4) GHz [21, 42, 63] | 1.23 |

Based on the above discussion we have the following observations:

In sub-case 2, Spatial-Clustering performance improves with higher number of spatial cells in cluster.

In sub-case 2, Pure Frequency-Clustering (without Inter-spatial Cells) have worse performance than Spatial-Clustering with Inter-spatial cells as cluster’s elements.

It is not clear if the improvement, occurs with higher cluster size (in Spatial-Clustering approach), is due to spatial intercorrelation or frequency correlation, or due to the way the averaging of loss is calculated at the output.

### 2.2.2	Inter-frequency prediction sub-case 3

In this section, we evaluate sub-case 3, where beam-level measurements (of e.g., s serving cell) is input and cell-level measurement of a neighboring cell in a different frequency is the output of the model, for mainly 2 GHz and 4 GHz (but also including 0.8 and 2.6 GHz), as shown in Table 5.

Table 5. Inter-Frequency Prediction for sub-case 3

| Nb | Consolidated Beam @ Input Cells | Output Cells | ML (dB) |
| --- | --- | --- | --- |
| 1 | 9 Spatial Cells @ 2GHz [0, 1, 2, 3, 4, 5, 6, 7, 8] | 9 Spatial Cells @ 4GHz [63, 64, 65, 66, 67, 68, 69, 70, 71] | 0.72 |
| 2 | 6 Spatial Cells @ 2GHz [0, 1, 2, 3, 4, 5] | 6 Spatial Cells @ 4GHz [63, 64, 65, 66, 67, 68] | 0.99 |
| 3 | 3 Spatial Cells @ 2GHz [0, 1, 2] | 3 Spatial Cells @ 4GHz [63, 64, 65] | 1.78 |
| 4 | 3 Collocated Cells @ [2,0.8,2.6] GHz [0, 21, 42] | 1 Cell @ 4GHz [63] | 2.70 |
| 5 | 1 Cell @ 2GHz [0] | 1 Cell @ 4GHz [63] | 3.04 |
| 6 | 1 Cell @ 2GHz [0] | 3 Collocated Cells @(0.8,2.6,4) GHz [21, 42, 63] | 3.27 |

We have similar observations to the one discussed in sub-case 2 (previous section), as follows:

In sub-case 3, Spatial-Clustering performance improves with higher number of spatial cells in cluster.

In sub-case 3, Pure Frequency-Clustering (without Inter-spatial Cells) have worse performance than Spatial-Clustering with Inter-spatial cells as cluster’s elements.

### 2.2.3	Impact of Frequency-Clustering

In this section, we evaluate the Frequency-Clustering and impact of frequency correlation in Table 6 and Table 7. In this evaluation, we assume that closer the input and output frequencies the higher the correlation between input and output. Note that all results, in Table 6 and Table 7, are for sub-case 2 and without assistant information.

In Table 6, when comparing row-1 and row-2 we realize that bringing the extra frequency 0.8 GHz to form input cluster did not improve the prediction performance and it remain at 0.55 dB, this is because the closest frequent to 4 GHz is 2.6 GHz which is available in both row-1 and row-2. On the contrary, when forming a cluster of 0.8 and 2 GHz as in row-3, with maximum frequency (i.e., 2 GHz) less than row-2 (i.e. 2.6 GHz), the prediction performance of cluster in row-3 (0.9 dB) is worse than singular in row-2 (0.55 dB). However, row-3’s RSRP difference = 0.9 dB outperforms row-4’s RSRP difference=1.3 dB, because the maximum frequency in the cluster of row-3 is closer to the output frequency, than the 0.8 GHz in row-4.

Table 6. Input Frequency-Clustering comparison with frequency distance impact.

| Row | Input Cell-0 @ | Output Cell-0 @ | RSRP difference (dB) |
| --- | --- | --- | --- |
| 1 | [0.8, 2.6] GHz | 4 GHz | 0.55 |
| 2 | [2.6] GHz | 4 GHz | 0.55 |
| 3 | [0.8, 2] GHz | 4 GHz | 0.9 |
| 4 | [0.8] GHz | 4 GHz | 1.3 |

On the other hand, Table 7 shows a comparison when varying the cluster at the output. Main conclusion is that there is minor differences when adding frequencies in the output cluster, given a single input but different frequencies (0.8 or 2 GHz). We observe that predicting 4 GHz from 2 GHz in row-1, row-2, and row-4, all have shown same RSRP difference of about 1.08 dB. But predicting 4 GHz from 0.8 GHz in either row-3 or row-5 shows worse RSRP difference about 1.3 dB.

Table 7. Output Inter-Frequency Clustering Comparison with frequency distance impact.

| Row | Input Cells | Output Cells | RSRP difference (dB) |
| --- | --- | --- | --- |
| 1 | [2GHz] | [2.6GHz, 4GHz] | [0.55, 1.08] |
| 2 | [2GHz] | [4GHz] | [1.08] |
| 3 | [0.8GHz] | [2.6GHz, 4GHz] | [1.12, 1.3] |
| 4 | [2GHz] | [0.8GHz, 2.6GHz, 4GHz] | [2.059, 0.55, 1.08] |
| 5 | [0.8GHz] | [4GHz] | [1.3] |

In Frequency-Clustering, what improve the performance is not necessary the input cluster size, but the closest frequency (highest correlated frequency) in the input cluster to the output.

When clustering the output, It is essential to evaluate the loss per single element in the output-cluster.

Based on the above discussion, we propose

When evaluating RSRP difference with clusters, it is essential to keep an eye on the performance per individual element of the cluster, and don’t draw conclusion based on average RSRP difference for the whole cluster.

### 2.2.4	Measurement reduction rate for inter-frequency predictions

The main goal of the inter-frequency predictions is to reduce the UE measurement overhead, as the UE can predict cell quality in another frequency instead of using measurement gaps to tune the RF and measure the cells in the neighboring frequencies. In the meeting RAN2#126 measurement overhead reduction rate formulas have been defined for the temporal predictions (Case B) and for the spatial predictions, as following.

Definition of measurement reduction for intra-frequency scenario is defined as: Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured

Similar to the above cases we propose to define measurement overhead reduction rate formula, specific for the inter-frequency predictions, as following.

MRRF = skipped inter-freq. measurements / (total number of measured and predicted inter-freq). measurement

Use the following formula for measurement reduction for inter-frequency scenario:
Measurement reduction rate in frequency domain (MRRF):
MRRF = skipped inter-frequency measurements / (total number of measured and predicted inter-freq. measurements)

## 2.3	Spatial RRM measurement predictions

Case-6 is recognized as middle priority case, where we predict spatial cell(s) in one frequency but neighbor spatial cells. All below results are for 2 GHz frequency. In the below subsections, we evaluated both sub-case 2 and sub-case 3, we also evaluate with and without assistant information (i.e., UE-Location). We also evaluate spatial clustering approach vs singular prediction scenarios.

- Spatial-Cluster: the cells within input-cluster and output-cluster are spatial cells. That is given a 7-sites, and 3-sector per-site, if we divide similarly, we can have a maximum of about 11 input cells and 10 output cells (both input and output cells have to sum-up to 21, and they belong to a single frequency, i.e., 2 GHz). There are many approaches for clustering, but we select the following:

  - SC-M1: This method selects even cells as input and odd cells as output (which we use in the below evaluation). Example of InputOutput cells indexing is: In= [0, 2, 4, 6, 8]  Out=[1, 3, 5, 7, 9].

### 2.3.1	Investigation of Clustering for Sub-case 2 and Sub-case 3

Table 8 shows evaluation of different cluster size (using SC-M1) for both input and output for sub-case 2 and sub-case 3, without assistant information. In this table, larger cluster size improves prediction performance. For instance, row-6, 7, 8, 9, 10 (which represent cluster-size=11,6,5,3,1, respectively) shows degraded RSRP difference of 1.08, 2.12, 2.89, 5.22, 7.56 dB, respectively. Furthermore, Table 8 shows that performance of sub-case 2 outperforms that of sub-case 3 for all cluster-sizes, as shown when comparing row-1<->row-6: RSRP difference = 0.55 dB<->1.08 dB, row-2<->row-7: RSRP difference= 1.07 dB<->2.12 dB.

Table 8. Spatial prediction - Evaluation of clustering for two-subcases.

| Row | Sub-Case | Input Cells @ 2GHz | Output Cells @ 2GHz | RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | 0.55 |
| 2 | 2 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | 1.07 |
| 3 | 2 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5- Odd Spatial Cells [1, 3, 5, 7, 9] | 1.60 |
| 4 | 2 | 3-Even Spatial Cells [0, 2, 4] | 3- Odd Spatial Cells [1, 3, 5] | 3.74 |
| 5 | 2 | 1 Spatial Cell [0] | 1 Spatial Cell [2] | 7.07 |
| 6 | 3 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | 1.08 |
| 7 | 3 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | 2.12 |
| 8 | 3 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5- Odd Spatial Cells [1, 3, 5, 7, 9] | 2.89 |
| 9 | 3 | 3-Even Spatial Cells [0, 2, 4] | 3- Odd Spatial Cells [1, 3, 5] | 5.22 |
| 10 | 3 | 1 Spatial Cell [0] | 1 Spatial Cell [2] | 7.56 |

Given Odd/Even Clustering method (cells with even index as input and cells with odd index as output), larger cluster size improves prediction performance.

Prediction performance of Sub-case 2 outperforms that of Sub-case 3.

### 2.4.2	Impact of Assistant Information

Table 9 shows evaluation result of cluster based (SC-M1) spatial prediction in sub-case 2 for the case of using assistant information (i.e., UE-Location) and no assistant information. Using UE-Location as assistant information improve performance considerably especially when cluster size is low, as show in row-8 (RSRP difference=7.98 dB) in comparison to row-4 (RSRP difference = 0.55 dB). However, such improvement is not as clear when applying on large cluster size, this is show when comparing row-1 (RSRP difference=0.39 dB) and row-5 (RSRP difference=0.55 dB).

Table 9. Spatial Prediction - Comparing with and without assistant information.

| Row | Input Cells @ 2GHz | Output Cells @ 2GHz | Assist. Info. | ML Loss |
| --- | --- | --- | --- | --- |
| 1 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial Cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | UE Location | 0.39 |
| 2 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | UE Location | 0.42 |
| 3 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5-Odd Spatial Cells [1, 3, 5, 7, 9] | UE Location | 0.52 |
| 4 | Single-Even Spatial Cell [0] | Single-Odd Spatial Cell [1] | UE Location | 0.55 |
| 5 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial Cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | NO | 0.55 |
| 6 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | NO | 1.07 |
| 7 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5-Odd Spatial Cells [1, 3, 5, 7, 9] | NO | 1.60 |
| 8 | Single-Even Spatial Cell [0] | Single-Odd Spatial Cell [1] | NO | 7.98 |

Based on the above discussion, we have the following observations:

Using UE location as assistance information outperforms not using assistance information.

The assistant information improves the prediction accuracy when the cluster includes few cells. For bigger cluster, the assistance information has less effect in improving the prediction accuracy.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	The performance of a single model with 5 output RSRPs (e.g., ratio 1 shown in Table 2) is comparable to the multi-model training and inference (e.g., ratio 5 in Table 1 with one model per each prediction steps).

Observation 2	Unlike Case B, for Case-A sub-case 2, increasing OW length has negligible impact on prediction accuracy.

Observation 3	Maximum prediction step is below 320ms (irrespective of UE speed) if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 4	Using the beams used in consolidation procedure (for cell quality derivation) as input provides better performance compared to using all the measured beams as input.

Observation 5	In sub-case 2, Spatial-Clustering performance improves with higher number of spatial cells in cluster.

Observation 6	In sub-case 2, Pure Frequency-Clustering (without Inter-spatial Cells) have worse performance than Spatial-Clustering with Inter-spatial cells as cluster’s elements.

Observation 7	It is not clear if the improvement, occurs with higher cluster size (in Spatial-Clustering approach), is due to spatial intercorrelation or frequency correlation, or due to the way the averaging of loss is calculated at the output.

Observation 8	In sub-case 3, Spatial-Clustering performance improves with higher number of spatial cells in cluster.

Observation 9	In sub-case 3, Pure Frequency-Clustering (without Inter-spatial Cells) have worse performance than Spatial-Clustering with Inter-spatial cells as cluster’s elements.

Observation 10	In Frequency-Clustering, what improve the performance is not necessary the input cluster size, but the closest frequency (highest correlated frequency) in the input cluster to the output.

Observation 11	When clustering the output, It is essential to evaluate the loss per single element in the output-cluster.

Observation 12	Given Odd/Even Clustering method (SC-M1), larger cluster size improves prediction performance.

Observation 13	Prediction performance of Sub-case 2 outperforms that of Sub-case 3.

Observation 14	Using UE location as assistance information outperforms not using assistance information.

Observation 15	The assistant information improves the prediction accuracy when the cluster includes few cells. For bigger cluster, the assistance information has less impacts in improving the prediction accuracy.

Based on the discussion in the previous sections we propose the following:

Proposal 1	When evaluating RSRP difference with clusters, it is essential to keep an eye on the performance per individual element of the cluster, and don’t withdraw conclusion based on average RSRP difference for the whole cluster.

Proposal 2	Use the following formula for measurement reduction for inter-frequency scenario: Measurement reduction rate in frequency domain (MRRF): MRRF = skipped inter-frequency measurements / (total number of measured and predicted inter-freq. measurements)

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#126

R2-2407849, Summary of [POST127][030][AI mobility] RRM simulation assumptions – Phase 1(OPPO)

R2-2407885, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2(OPPO)

R2-2408747, Simulation results for temporal and inter-frequency RRM measurement predictions, Ericsson