3GPP TSG-RAN2 Meeting #128	R2-2410476

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the RAN2 objectives and Ambient IoT random access should be studied.

In RAN2 #125bis meeting, the following agreements were made regarding the Ambient IoT random access.

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

RAN2 #126 made the following agreements related to Ambient IoT random access.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4  The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5  “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

RAN2 #127 made the following agreements related to Ambient IoT random access.

=> wait for further RAN1 progress on indication of the start of access occasion. => RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design). Wait for RAN1 for further details.

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

RAN2 #127bis made the following agreements related to Ambient IoT random access.

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this paper, we discuss the random-access aspects of Ambient IoT (AIoT) and provide our view.

# Discussion

In AIoT access, it could be contention-based or contention-free. Since all types of AIoT access are initiated by the trigger message, whether the access is contention-based or contention-free should be determined by the Reader. Essentially, the contention-free or contention-based is about whether the resource for AIoT access is dedicatedly configured to the specific AIoT device, and whether the access resource is unique to the AIoT device.

From AIoT device perspective, if the resource is dedicatedly configured to the specific AIoT device or the access resource configured to AIoT device can be one-to-one mapping, the AIoT device simply selects the corresponding dedicated AIoT access occasions for contention-free AIoT access. If multiple resources are configured by Reader, the AIoT device randomly selects the access occasion among the configured resource for contention-based AIoT access.

Proposal 1: For contention-based AIoT access, if Reader provides multiple access resources for AIoT devices, the AIoT device randomly selects the AIoT access occasions among the resources provided by Reader.

Proposal 2: For contention-free AIoT access, the reader configures dedicated AIoT access occasions for each AIoT device and the AIoT device selects the corresponding access occasion.

The trigger message may optionally include the filter criteria sent from the upper layer to identify the AIoT devices that are supposed to respond to the inventory request or command message.

The Reader forwards the filter criteria to AIoT devices. The AIoT device should check the filter criteria provided by Reader before performing AIoT access. Only if the filter criteria are satisfied, the AIoT devices are allowed to perform the access towards Reader. The details of the filter criteria is up to SA2 for further study.

Proposal 3: If Reader provides filter criteria in the trigger message, only those AIoT devices that meet the filter criteria perform AIoT access.

Reader may have preference on which type of AIoT access (e.g., 2-step or 3-step) to be triggered in response to the different service requests. For example, if Reader would like to issue the ‘Write’ command to a small number of AIoT devices, the 2-step AIoT access is for simple confirmation feedback after the command execution. In another example, Reader may indicate AIoT device to perform 3-step AIoT access if Reader estimates a large number of AIoT devices to be inventoried and the contention-based resource is provided.

In RAN2 #127bis, it has been agreed that AIoT device determines the random-access type from the paging message. Considering that random ID may have to be sent in both 2-step and 3-step contention-based AIoT access, the difference of AIoT Msg1 in 2-step and 3-step AIoT access is whether the AIoT data is allowed to be carried in AIoT Msg1. Thus, one possible way is that Reader explicitly indicates the type of AIoT access (e.g., 2-step or 3-step) to AIoT device in the trigger message, or alternatively, Reader informs the AIoT device by the trigger message whether the AIoT device ID and/or AIoT upper layer data is allowed to be sent in AIoT Msg1.

Proposal 4: In trigger message, Reader indicates the type of AIoT access (e.g., 2-step or 3-step) to AIoT devices, or inform AIoT devices whether the AIoT data is allowed to be sent in AIoT Msg1.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information and no intention for whether acknowledgement reception or not).

It is beneficial for AIoT device to have correct expectation on the next step after transmitting AIoT device ID and/or AIoT upper layer data in AIoT Msg3. Since Reader can determine the next step (after AIoT Msg3) of AIoT device and the information carried in the subsequent R2D message may be different, the earlier R2D message can indicate the presence and what type of information (e.g., R2D failure/success feedback indication) to be carried in the subsequent R2D message.

Proposal 5: The R2D message (e.g., the AIoT trigger message) can indicate the presence and what type of information (e.g., R2D failure/success feedback indication) to be carried in the subsequent R2D message.

In RAN2 #127bis, it has been agreed that Reader uses the optional explicit R2D failure/success feedback indication to determine the re-access of AIoT device. At least such indication can be used for re-access for addressing the transmission failure of AIoT Msg3. It is necessary to inform AIoT device the status of D2R transmission and the AIoT device also needs to monitor the feedback from Reader since the AIoT access and data communication is based on the trigger from Reader. There could be several possible ways to resolve this issue, e.g., the AIoT device which does not receive the failure indication may assume the AIoT Msg3 or D2R data is successful and may not response the next trigger message, or the AIoT device who receives the success indication of other AIoT device determines the AIoT Msg3 or D2R data failure and should assume such success indication of other AIoT device triggers this AIoT device to re-send AIoT Msg3 or D2R data.

Thus, the R2D failure/success feedback indication is useful to indicate the D2R data transmission failure (also AIoT Msg3). In addition, the AIoT device performs the AIoT access upon reception of the subsequent trigger message.

Proposal 6: The R2D failure/success feedback indication is used for AIoT device to re-send the D2R data (also AIoT Msg3).

Proposal 7: AIoT devices performs AIoT re-access upon reception of the subsequent trigger message.

For AIoT re-access, AIoT device may select the access occasion to transmit the AIoT Msg1 again. The access occasions for re-access are configured by the subsequent trigger message and the access occasions may be different from the resource configured by the initial trigger message.

Proposal 8: In AIoT re-access, AIoT device transmits AIoT Msg1 again in a newly selected access occasion indicated in the subsequent trigger message.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For contention-based AIoT access, if Reader provides multiple access resources for AIoT devices, the AIoT device randomly selects the AIoT access occasions among the resources provided by Reader.

Proposal 2: For contention-free AIoT access, the reader configures dedicated AIoT access occasions for each AIoT device and the AIoT device selects the corresponding access occasion.

Proposal 3: If Reader provides filter criteria in the trigger message, only those AIoT devices that meet the filter criteria perform AIoT access.

Proposal 4: In trigger message, Reader indicates the type of AIoT access (e.g., 2-step or 3-step) to AIoT devices, or inform AIoT devices whether the AIoT data is allowed to be sent in AIoT Msg1.

Proposal 5: The R2D message (e.g., the AIoT trigger message) can indicate the presence and what type of information (e.g., R2D failure/success feedback indication) to be carried in the subsequent R2D.

Proposal 6: The R2D failure/success feedback indication is used for AIoT device to re-send the D2R data (also AIoT Msg3).

Proposal 7: AIoT devices performs AIoT re-access upon reception of the subsequent trigger message.

Proposal 8: In AIoT re-access, AIoT device transmits AIoT Msg1 again in a newly selected access occasion indicated in the subsequent trigger message.

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA