3GPP TSG-RAN WG2 Meeting #128	R2-2410484

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:	8.9.4

Source:	Samsung

Title:	Discussion on impact of PWS broadcasting for NB-IoT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#105 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions related to PWS broadcasting for NB-IoT.

# Discussion

- RAN2#127bis agreements:

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

## PWS broadcasting in LTE

- Public Warning System indication have been introduced in LTE from the very first releases.

- Earthquake Tsunami Warning System (ETWS) was introduced in Release 8 LTE and is also supported for LTE-M. The ETWS is indicated via SIB10 and SIB11, where the SIB10 provides the primary indication and SIB11 provides secondary notification.

- Commercial Mass Alert System (CMAS) was introduced in Release 9 LTE and is also supported for LTE-M. The CMAS is indicated via SIB12.

Observation 1: PWS is broadcasted via ETWS in SIB10 and SIB11 and CMAS in SIB12.

- In LTE there are two ways in which PWS signalling may be indicated. For normal LTE devices, i.e non-eMTC devices, ETWS or CMAS is indicated via Paging, where the UE is indicated in the Paging RRC message whether ETWS or CMAS is being broadcasted. Thus the UE monitors for P-RNTI and once a Paging message is received, a flag indicates either ETWS or CMAS. For eMTC, Direct Indication is supported. Direct Indication is an 8-bit message that is inserted directly into the PDCCH message with a specific format, which means that the UE does not need to acquire the corresponding PDCCH message. The eMTC UE monitors P-RNTI or SI-RNTI to receive the Direct Indication. P-RNTI is monitored in RRC_IDLE and SI-RNTI is monitored in RRC_CONNECTED. This is because originally eMTC was not specified to be required to monitor P-RNTI in connected mode, but in release 16 it was agreed to support PWS indication in connected mode. Instead of monitoring the P-RNTI, a new SI-RNTI and a new Direct Indication was introduced. The Direction Indication has 1-bit flags that indicate whether CMAS or CMAS is indicated. System information update may also be indicated via the Direct Indication. The Direct Indication was also adopted for 5G NR.

Observation 2: eMTC UE monitors P-RNTI for ETWS and CMAS indication in RRC idle and  monitors SI-RNTI in connected mode.

## General procedures

- In the last meeting the following was agreed:

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

- The question is whether the etws-Indication and cmas-Indication shall be added in Paging-NB or not.

- Direct Indication was introduced for eMTC and allows a UE to not acquire the PDSCH and is a more efficient manner of signalling the PWS indication. For 5G, it is only supported to signal the PWS indication via Direct Indication.

- One technical issue with potentially using Paging-NB to signal etws-Indication and cmas-Indication is that it may affect legacy UEs that are not capable of receiving the ETWS and CMAS indication. These UEs would still be forced to acquire the PDSCH and would not find a matching UE ID. For Direct Indication, the design is already more forwards-compatible and the specification of a specific release will already instruct the UE to ignore any value received in the Direct Indication that the UE does not support. Therefore we think that there is no need to add ETWS or CMAS indication to Paging-NB as there is no technical purpose for it.

Proposal 1: ETWS or CMAS indication is not added to Paging-NB.

## Connected mode indication

- As mentioned above, in eMTC and ordinary LTE, UE can receive emergency indication in connected mode. For ordinary LTE, the UE will monitor for P-RNTI in connected mode and in eMTC Rel-16 an enhancement was introduced to allow for the UE to monitor for Direct Indications on SI-RNTI.

- Being able to receive the emergency indications is important in connected mode as the UE shall attempt to acquire the emergency indication as fast as possible. We believe that the same motivation holds for NB-IoT NTN, i.e that a UE shall attempt to acquire PWS broadcasting as fast as possible. In fact, since the delays in NB-IoT are expected to take longer, we believe it is more crucial that starting to receive the PWS broadcast as fast as possible is more important.

Observation 3: Due to the associated delays in NB-IoT and NTN, it is important that UE start to acquire PWS broadcasting as fast as possible.

- However, requiring a UE to monitor a new RNTI is against the design of NB-IoT to keep the device low complexity and power efficient. Therefore there must be other ways to notify the UE that PWS is being broadcasted and to indicate to the UE to start acquiring the PWS broadcast. One method could be that the network indicates in a dedicated fashion to each connected mode UE that is capable of receiving PWS broadcast.

Proposal 2: Support a dedicated indication of PWS broadcasting to NB-IoT UE in connected mode.

## Idle mode operation for PWS broadcasting

- In eMTC and normal LTE it is supported to camp on a cell and then receive PWS broadcasting. As long as the UE is camping on a cell, the UE shall acquire PWS broadcasting. This is regardless whether the cell that the UE camps on is considered a suitable cell or an acceptable cell. This means that the UE can always receive PWS broadcasting, even if the UE is not camping on the selected, registered or an equivalent PLMN.

- For NB-IoT, the UE can only camp on a suitable cell, meaning that “acceptable cells” does not exist in NB-IoT. This is because there was never any reason for this to exist. In NB-IoT NTN PWS broadcasting, one of the key scenarios is UE is a normal UE that is 4G/5G capable, which is also capable of NB-IoT NTN for emergency situations. In this case, the UE may need be able to camp on an NB-IoT NTN cell only for the purpose of receiving emergency broadcasts. We think that this scenario is a valid use case.

Proposal 3: A UE shall be able to camp on an NB-IoT NTN cell only for the purpose of receiving emergency broadcast.

# Conclusion

In this contribution, we discussed issues related to PWS signaling for NB-IoT.

Observation 1: PWS is broadcasted via ETWS in SIB10 and SIB11 and CMAS in SIB12.

Observation 2: eMTC UE monitors P-RNTI for ETWS and CMAS indication in RRC idle and monitors SI-RNTI in connected mode.

Proposal 1: ETWS or CMAS indication is not added to Paging-NB.

Proposal 2: Support a dedicated indication of PWS broadcasting to NB-IoT UE in connected mode.

Proposal 3: A UE shall be able to camp on an NB-IoT NTN cell only for the purpose of receiving emergency broadcast.

# Reference

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#105, Melbourne, Australia, September 2024.

3GPP TS 23.401 “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”, Release 19, V19.0.0, September 2024.