3GPP TSG-RAN WG2 Meeting #128						            R2-2410492

Orlando, USA, 18th – 22nd November, 2024

Title: 		Discussion on LCM for UE-sided model for Beam Management use case

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #127bis meeting, RAN2 made the following agreements for the UE-side model for BM case, including the functionality applicability report, functionality activation/deactivation/fallback, data collection aspects.

Agreements for BM 1. UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. 2. FFS if inference configuration can be signalled in step3. 3. UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) 4. Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server) 5. For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1). 6. For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models. Can come back to this discussion later.

In RAN2 #127 meeting, RAN2 had sent a LS [1] to RAN1, and in RAN1 #118bis meeting RAN1 made the following agreements for some of the questions.

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

In this paper, we discuss the FFSs highlighted in the above box. In addition, following the R2#128 agenda item guidelines, we also discuss the issues not dependent on RAN1 or which haven’t yet discussed.

# Discussion

## Applicable functionality determination

The network side additional conditions are needed to be known to the network as part of the functionality applicability reporting procedure, to ensure the consistency across the training and inference for UE-sided model. As agreed by RAN1 for question Q2, associated ID is used to indicate the network side additional conditions, so RAN2 can follow this as working assumption. The network side additional condition alignment can be done in the following ways.

- First, the UE trains the model based on the dataset and associates it with the related associated ID, if provided by the network.

- The UE reports the applicable functionality to the NW, which may be associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

In the LS [1], RAN2 asked RAN1 in Q4-1 that:

- Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Based on the above Q4-1, RAN1 discussed three options in RAN1 118bis related to the applicable functionality reporting for inference configuration.

- Option 1: NW configures one or more CSI-ReportConfig for inference configuration in step 3, where the associated ID may be configured in the CSI framework.

- Option 2: NW may provide the associated ID along with the inference related parameters in step 3. In step 4, the UE may indicate the associated ID when reporting the applicable functionality, which depends whether the associated ID is indicated independently with the inference related parameters.

- Option 3: NW may provide the associated ID, and UE report the inference related parameters and optionally the associated ID.

In all the options, the NW-side additional condition can be indicated in step 3 optionally. For Option 2 and 3, the associated ID can also be optionally indicated by the UE to the NW in Step 4. In our view, associated ID is optional for the whole LCM process, i.e. even for training phase the NW might not provide it and the UE should still be able to determine the applicability of the functionality based on implementation, but this needs to be confirmed by RAN1 based on the question above.

Observation#1: Based on all the RAN1’s option, the NW-side additional condition can be indicated in Step 3 optionally. For Option 2 and 3, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Based on Observation#1, the UE reporting behavior can be as follows depending on whether the network indicates or not the NW-side additional condition in Step 3 in all options or whether the UE indicates back or not in Step 4 for Options 2 and 3:

- Case 1: If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports only the applicable functionalities corresponding to the indicated associated ID(s).

- Case 2: If the network does not indicate the associated ID, the UE can determine which of the functionalities are applicable from UE perspective considering model availability and UE side additional conditions and report those functionalities together with the corresponding associated IDs, if available. Based on this, the network can select and configure the selected applicable functionality for inference according to the current network-side addition condition.

For Case 1, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicable functionalities associated with the indicated associated IDs. In this case, the UE can do the filtering based on the associated ID information, so that it does not need to report all the applicable functionality information stored. However, if the NW-side additional condition changes to the one not requested earlier from the UE, the network will have to ask the UE to report the applicable functionalities again, with the new associated ID.

For Case 2, when the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. However, in some situations, it may cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting of other IDs is meaningless.

Based on the above analysis, both Case 1 and Case 2 are useful and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it should be left to NW implementation which option to use at a certain time. If the NW indicates associated ID(s) to UE, the UE checks the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and model availability and reports applicable functionalities together with the supported associated IDs, if available.

Proposal 1: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

## Applicable functionality reporting procedure

Based on the previous discussion, the UE can report the applicable functionality in two ways:

- Proactive reporting: the UE reports applicable functionalities based on the network configuration and when applicability of some functionality changes the UE updates the information

- Reactive reporting: the UE reports applicable functionalities as requested by the network configuration, the reporting is one-shot.

The two procedures are quite similar. The main potential difference mentioned by the companies lies in the contents of the configuration provided in Step 3 and whether Step 5 is needed.

(a)                                                                      (b)

Figure 1. Applicable functionality reporting: (a) proactive reporting, and (b) reactive reporting

In last meeting, we have the agreement that

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content.

As we listed in section 2.1, RAN1 has provided three options for the applicable functionality reporting procedure, they can lead to different applicability reporting content. For example. For option 1, the UE may report a bitmap corresponding to the inference configuration to indicate the applicability of the functionality. For option 2 and option 3, the UE may indicate the inference related parameters of the applicable functionalities. RAN2 can wait for further RAN1 progress on their 3 options on applicable functionality reporting before discussing the applicability reporting contents.

Proposal 2: RAN2 can wait for further RAN1 progress on the 3 options on applicable functionality reporting before discussing the applicability reporting contents.

For the scope of the associated ID, RAN1 has the following working assumption from RAN1#118 meeting.

RAN1#118 meeting progress: Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the above working assumption, associated IDs for one applicable functionality may not be the same from one cell to another cell. It means that when the UE moves from one cell to another cell, the UE needs to update the reporting. For example, first the UE is in cell 1, it reports the applicable functionality and its associated IDs #1, #2, and #3. When the UE moves to cell 2, it reports the applicable functionality and its associated IDs #5, #6, and #7. Associated IDs #1, #2, and #3 should be allocated and valid in cell 1, while associated IDs #5, #6, and #7 should be allocated and valid in cell 2.

Each time the UE changes the serving cell, the legacy applicable functionality information will be invalid. When the UE connects to a new cell, the NW needs to trigger the applicable functionality reporting for the new cell, or the UE needs to update and report the applicable functionality automatically. It means that each time the cell change happens, we need to go through the applicable functionality reporting and inference configuration once again in order to configure the proper functionality in the new cell, which can cause AI/ML operation interruption. In principle, this does not cause major issues as the UE may simply perform legacy BM operation during this short period of time. However, it would be useful to have a mechanism to guarantee the continuity of the AI/ML operation even the associated ID is allocated locally in one cell.

Similar issue exists if the associated IDs can be applicable for multiple cells. When the UE moves from one cell group to another cell group, the previous applicable functionality information can be invalid and the UE needs to update the reporting.

Proposal 3: As the associated ID is not globally unique, it is proposed RAN2 to discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

If the UE is configured to report in the HO Command, the UE can send the applicable functionality information after the handover in the RRCReconfigurationComplete message if there is no urgency for the applicable functionality reporting on the target node as the UE/NW may rely on legacy beam management operation. If the target node needs to know the applicable functionality urgently, for example, to guarantee the service continuity of the AI/ML, applicable functionality reporting in the HO complete message does not solve the problem, since the target node needs the applicable functionality information before preparing the HO configuration.

Observation 1: Usage of RRCReconfigurationComplete message for applicable functionality reporting does not improve the continuity of AIML feature.

If a continuity of AIML based beam management is needed during handover, one of the options is for the source node to help the target node to collect the applicable functionality information. For example, the source node can get the applicable functionality reporting requirement (e.g. associated ID, functionalities supported by the target node/cells) at the target node side based on the inter-gNB interaction, and configure the UE to report the applicable functionality information for the target node, by the applicable functionality reporting (e.g. UAI) in the source cell.  Once the information is available at the source node, when the handover needs to be triggered, this information can be forwarded to the target node, e.g. during a handover preparation. Then the target node can provide the configurations for the applicable functionality directly in the HO command during the HO preparation based on the applicable functionality information for the target node from the source node.

Proposal 4: If the service continuity of the AI/ML should be supported, RAN2 should focus on solutions allowing for proper AIML configuration to be provided already in HO command, e.g. the source node can help collect the applicable functionality information for the target node.

## Applicability status changes

In this section, the reporting of the changes of the applicability of a functionality is discussed based on the following agreement.

Agreement: UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

When the UE initiates the first applicable functionality reporting, the UE only needs to report the applicable functionalities since NW can consider the other functionalities are non-applicable.

It seems a common understanding that a functionality as indicated by a UE may not remain applicable or non-applicable all the time. UE internal conditions and/or NW side conditions and/or the availability of the functionality can impact the status of the functionality. The status of a functionality may change from “non-applicable” to “applicable” or from “applicable” to “non-applicable”. Once an applicable functionality is activated, the functionality has to be deactivated if it becomes non-applicable, otherwise system performance will degrade. Also, when applicable functionality reporting was requested by the NW before, the UE should report the functionality once it becomes applicable. Therefore, when the functionality has been previously reported as applicable or non-applicable and its status changes, such information needs to be delivered to the network.

As on whether such information needs to be explicitly/implicitly signalled as in the FFS of the agreement from last meeting for the status change, the non-applicable functionalities do not need to be indicated explicitly (i.e. the UE always reports only applicable functionalities). NW can assume that functionalities not reported in the applicable functionality reporting in Step 4 as non-applicable. This will align with when a functionality is indicated as applicable (or absence means non-applicable) and when a functionality changes from applicable to non-applicable or vice versa.

Proposal 5a: When the UE initiates applicable functionality reporting for the first time in a cell, the UE only needs to report the applicable functionalities.

Proposal 5b: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, such information needs to be delivered to the network.

Proposal 5c: UE always reports only applicable functionalities in the applicable functionality reporting, regardless of whether it is reported first time or when there is a status change.

However, it should be noted that as opposed to the changes of NW side conditions, UE internal conditions are unseen and unpredicted from NW perspective. Even if UE could have the flexibility to timely indicate the status changes of the applicable functionality due to UE internal conditions, signaling overhead is introduced, especially for the cases where the changes among different functionality statuses are frequent. On the other hand, if status updates frequency is limited by the network (e.g. using a prohibit timer), then there may be a delay in UE reporting a functionality as non-applicable. In this case, the NW may need to configure more resources for performance monitoring, but this will introduce additional resource overhead.

Observation 2: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

From network perspective, it is preferred to choose such functionalities for inference which are likely to be used for a longer time and which are not prone to frequent status changes, e.g. due to dynamic changes of UE-side conditions. To help the network to choose proper configurations, it would be useful if on top of bare applicability information, the UE also provided also some kind of “stability of applicability” indication. For example, if the UE knows a certain inference operation is likely to exceed its processing power or that the model is only applicable for the current UE speed, the network should be aware of this. Such indication does not have to mention an explicit reason why the functionality status may change, but instead the UE could for example indicate the time during which the UE commits to keep the applicability of the functionality. Such information would allow the network to achieve a better tradeoff between signaling/resource overhead and system performance guarantee and would be helpful for performing appropriate functionality management. For example, if the applicable status can be maintained for a relatively long time, less resources for functionality monitoring are needed. If the applicable status cannot be maintained for a relative long time or cannot be determined reliably, the NW may decide not to activate the functionality, even though it is applicable for the time being, or the NW may still choose to activate the functionality, but configure more resources for performance monitoring.

Observation 3: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 6: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g. UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

As a commonly used method to avoid the frequent reporting, a prohibit timer can be configured. However, with the prohibit timer, the NW may not get the applicability information timely and cause some problem, as shown in the following cases.

- Case 1: if a functionality becomes “non-applicable” but cannot be updated because the prohibit timer is still running, meanwhile the NW decides to configure this functionality for inference, the inference configuration can fail. In this case, the UE needs to indicate the inference configuration failure to the NW after receiving the inference configuration.

- Case 2: a functionality becomes “applicable” but cannot be updated timely, the NW cannot configure the functionality for inference immediately if the NW is interested in this functionality.

From the two cases, it can cause different results if the NW cannot know the applicability update from “applicable to non-applicable” and from “non-applicable to applicable”. Thus, we need to configure different prohibit timer for these different cases. For example, if the NW wants to avoid the first cases and can endure the second case, the NW can configure the value of the prohibit timer for “applicable to non-applicable” update as 0 or a small value, and configure a proper value (which is larger than the prohibit timer for “applicable to non-applicable”) for the prohibit timer for “non-applicable to applicable” update.

Observation 4: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 7: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently active (i.e. not currently configured for inference operation). If a functionality that are currently active (i.e. currently configured for inference operation) becomes non-applicable, the UE should immediately report the status change.

## Data Collection for training

In the last meeting, it was agreed that data collection initiation and configuration are under network control as follows:

Agreement: Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

Regarding which entity should initiate data collection, there are two methods: NW-initiated and UE-initiated.

For UE-sided data collection option 1a:

- NW-initiated method means that NW can use some assistance information from UE side for its decision, e.g. UE capability reporting, applicable functionality reporting. How NW makes decision is up to NW implementation

- UE-initiated method means OTT server may request the UE to collect some data, and then UE may require some radio measurement configuration from the network, and it may because UE does not have sufficient data. After that, NW can decide whether/when to respond to the request from UE side

For UE-sided data collection options other than 1a:

- NW-initiated method means the requested configurations may come from OTT/CN node/OAM to the gNB, without the need of involving the UE. So UE-initiated method is not needed here

Proposal 8: For UE-sided data collection option 1a, it is proposed RAN2 to discuss both NW-initiated method and UE -initiated method, and some comparisons may be required.

Proposal 9: For UE-sided data collection options other than 1a, UE-initiated method is not needed, and it is proposed RAN2 to discuss NW-initiated method.

## Performance monitoring

For the UE-side AI/ML model for BM-Case1 and BM-Case2, RAN1 had the following progress for performance monitoring.

RAN1 117 meeting progress Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring RAN1 118 meeting progress Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) RAN1 118bis meeting progress Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN1 #118bis meeting, RAN1 has agreed type 1 option 2 where UE-assisted performance metric is reported to NW for performance monitoring. Thus far, the focus was on the performance monitoring while the UE was configured for inference. However, it should be noted that even when the inference is activated at the moment, the network may be interested in receiving the performance metrics from the UE. For example, the network may deactivate the inference when the performance drops below an acceptable threshold, but may still be interested in reactivating it when the performance improves. Hence, it should be possible for the UE to report the performance metric also when inference is currently not activated.

Proposal 10: The network should be able to configure the UE to report performance metric also while the inference is deactivated.

RAN1 is still discussing the detail of option 2. How and when to report metrics are still FFSs.

For the implementation details, we think that RAN2 can wait for RAN1 for more progress, and if RAN1 would like RAN2 to check some parts of the solution, RAN2 can have further discussions in future.

Observation 5: In option 2, how and when to report performance metric is FFS, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 11: RAN2 waits for RAN1 detail progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

# Conclusion

In this paper, our observations and proposals are summarized as follows:

Applicable functionality determination

Proposal 1: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

Applicable functionality reporting procedure

Proposal 2: RAN2 can wait for further RAN1 progress on the 3 options on applicable functionality reporting before discussing the applicability reporting contents.

Proposal 3: As the associated ID is not globally unique, it is proposed RAN2 to discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

Observation 1: Usage of RRCReconfigurationComplete message for applicable functionality reporting does not improve the continuity of AIML feature.

Proposal 4: If the service continuity of the AI/ML should be supported, RAN2 should focus on solutions allowing for proper AIML configuration to be provided already in HO command, e.g. the source node can help collect the applicable functionality information for the target node.

Applicability status changes

Proposal 5a: When the UE initiates applicable functionality reporting for the first time in a cell, the UE only needs to report the applicable functionalities.

Proposal 5b: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, such information needs to be delivered to the network.

Proposal 5c: UE always reports only applicable functionalities in the applicable functionality reporting, regardless of whether it is reported first time or when there is a status change.

Observation 2: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

Observation 3: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 6: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g. UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

Observation 4: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 7: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently active (i.e. not currently configured for inference operation). If a functionality that are currently active (i.e. currently configured for inference operation) becomes non-applicable, the UE should immediately report the status change.

Data Collection

Proposal 8: For UE-sided data collection option 1a, it is proposed RAN2 to discuss both NW-initiated method and UE -initiated method, and some comparisons may be required.

Proposal 9: For UE-sided data collection options other than 1a, UE-initiated method is not needed, and it is proposed RAN2 to discuss NW-initiated method.

Performance monitoring

Proposal 10: The network should be able to configure the UE to report performance metric also while the inference is deactivated.

Observation 5: In option 2, how and when to report performance metric is FFS, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 11: RAN2 waits for RAN1 detail progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

# References

[1] R1-2407604, LS on applicable functionality reporting for beam management UE-sided modell