3GPP TSG-RAN WG2 Meeting #128	R2-2410503

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	Data Collection for Network-Sided Model Training for the beam management use case

Document for:	Discussion

# 1. Introduction

Several agreements regarding network side data collection were made in RAN2-126bis [1], RAN2-127 [2] and RAN2-17bis [3] (see Annex for all the agreements from these meetings). In this contribution, we address some of the open issues.

# 2. Discussion

In RAN2-127 [2], it was agreed that:

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

And in RAN2-127bis [3], it was agreed that:

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

Currently, the UE-MeasurementsAvailable IE is used to indicate all relevant indicators for UE measurements (e.g.., logged MDT measurements, RLF reports, etc.,) and can be included in different RRC Complete messages (e.g., RRC Reconfiguration/Setup/Resume). It is rather straightforward to extend the same IE to indicate also the availability of logged measurements for a training of a network sided model.

Proposal 1: The UE can (opportunistically) indicate the availability of logged data using an RRC Reconfiguration Complete message (e.g., via a new indication within the UE-MeasurementsAvailable-r16 IE).

Based on the above agreements in RAN2-127 and 127bis, it naturally follows that the UE stopping the data collection/logging due to buffer limitation should be one trigger condition for availability indication. This will give an opportunity for the network to request the sending of the logged data and the UE can then reclaim the buffer. It may be beneficial for the UE to indicate the availability of the logged data immediately so that it will be able to send the logged data and reclaim the buffer space (e.g., for other data collection purposes), instead of waiting until it gets an RRC Reconfiguration message and send the availability indication in the complete message. One possibility is to use the UAI message for that purpose.

Proposal 2: If the UE stops data collection/logging due to buffer limitation, it sends an availability indication to the network (e.g., via a UAI message).

Currently, the UE is allowed to release any logged measurement (e.g., RLF report, logged MDT report, etc.,) after 48 hours. We believe it is necessary to specify a similar maximum duration of time that the UE is mandated to keep data logged for network side model training. The exact time duration can be decided later in the normative phase once we get more idea about the expected size of the logged data (e.g., if the logged data is large then the time duration needs to be small).

In relation to the time duration for keeping the logged data, it is also important to discuss whether the UE keeps the logged data during transition back and forth between CONNECTED and IDLE/INACTIVE. Considering that the inactivity timer for deciding the transition to IDLE/INACTIVE is up to network implementation, and it can be very short (e.g. in few minutes or even seconds), it is reasonable to assume that the duration for keeping the log is expected to be longer than that. Thus, we propose to keep the same approach as for legacy logged measurements such as logged MDT and RLF reports, i.e., UE keeps the logged data across state transitions, at least until the time duration for keeping the logs has expired.

Proposal 3: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/larger value) is FFS.

Proposal 4: The UE keeps the logged data upon transitioning to IDLE/INACTIVE.

Currently, the UE can indicate the availability of logged measurements when transitioning from IDLE/INACTIVE to CONNECTED states in the Setup/Resume Complete messages. If proposal 4 above is agreed, we propose to reuse the same mechanisms.

Proposal 5: The UE can indicate the availability of logged data upon transitioning from IDLE/INACTIVE to CONNECTED (i.e., in the RRC Setup/Resume Complete messages)

It was agreed in RAN2-127bis [3] that UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.  Currently, the network sends to the UE a UEInformationRequest message indicating the type of logged measurements that it wants the UE to send, and the UE includes the requested data/measurements using corresponding fields, as shown below:

UEInformationRequest-r16-IEs ::= SEQUENCE {

idleModeMeasurementReq-r16       ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16             ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16         ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                 ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

nonCriticalExtension             UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17        ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18            FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18    ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

Thus, there is a need to introduce a new IE/field to indicate the network is requesting a logged data for network side model training.

Proposal 6: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training. The name/type of the IE/field is FFS.

Similarly, a new IE/field needs to be introduced in the UEInformationResponse that will be used to include the relevant data.

Proposal 7: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data. The name/type of the IE/field is FFS.

It should be noted that we have already agreed in RAN2-127 [2] that:

The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

If the UE is not able to send all the logged data in one UEInformationResponse message, currently there is already a mechanism to indicate additional logged data is pending, as shown below (from TS 38.331):

5.7.10.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

<< skipped text>>

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

Thus, we propose similar mechanism for the sending of large logged data that do not fit into one UEInformationResponse message (i.e., UE indicating that there is more logged data in the UEInformationResponse and network requesting for the remaining ones)

Proposal 8: If the UE cannot fit all the logged data into one UEInformationResponse message, it will include an indication that more logged data is available in the UEInformationResponse message. The name/type of the IE/field is FFS.

# 4. Conclusion

In this contribution, we discussed the open issues regarding logging and reporting of data for the training of a network-sided model for the beam management use case, and the following proposals were made:

Proposal 1: The UE can (opportunistically) indicate the availability of logged data using an RRC Reconfiguration Complete message (e.g., via a new indication within the UE-MeasurementsAvailable-r16 IE).

Proposal 2: If the UE stops data collection/logging due to buffer limitation, it sends an availability indication to the network (e.g., via a UAI message).

Proposal 3: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/larger value) is FFS.

Proposal 4: The UE keeps the logged data upon transitioning to IDLE/INACTIVE.

Proposal 5: The UE can indicate the availability of logged data upon transitioning from IDLE/INACTIVE to CONNECTED (i.e., in the RRC Setup/Resume Complete messages)

Proposal 6: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training. The name/type of the IE/field is FFS.

Proposal 7: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data. The name/type of the IE/field is FFS.

Proposal 8: If the UE cannot fit all the logged data into one UEInformationResponse message, it will include an indication that more logged data is available in the UEInformationResponse message. The name/type of the IE/field is FFS.

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[3]	RAN2-127bis Chairman Notes, Hefei, China, October 2024

# 6. Annex (Agreements regarding network side data collection)

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2-127bis agreements [3]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.