3GPP TSG-RAN WG2 Meeting #128	R2-2410591

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were achieved for the Store & Forward (S&F) satellite operation [1]:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this contribution, we provide some further considerations on the Store and Forward satellite operation.

# 2	Discussion

## 2.1 Normal mode based on S&F architecture

### 2.1.1 Architecture

In the last meeting, we agreed if the S&F indication is absent, UE assumes the NTN cell is operating  in the Normal/real-time/non-S&F mode. However, here it is not crystal-clear which architecture the Normal mode refers to, since NR NTN introduced the regenerative architecture with only gNB on board and we also have transparent architecture since Rel-17. Obviously, here the Normal mode is discussed under the context of S&F and should only refer to the S&F architecture with eNB and at least partial MME on board. This needs to be clarified since the regenerative architecture similar to NR NTN (with only eNB on board) is not in the scope of Rel-19 IoT NTN. Otherwise it will lead to confusion in RAN3 since there is big impact to RAN3 if only eNB is on board.

Proposal 1: Clarify that the Normal mode agreed in the S&F context refers to the S&F architecture with eNB and partial MME/whole CN on board.

### 2.1.2	Remaining time of current operating mode

There was no consensus on whether NW needs to inform UE of the remaining time of the satellite’s current operating mode. Before determining whether to support this indication, we should first discuss why this is useful for UE to know.

Switching from normal mode to S&F mode

If the serving cell switches from normal mode to S&F mode, it means the feeder link becomes unavailable at some time point.

In the legacy mechanism, the t-service in SIB19 indicates when the current serving cell will stop the service in the area currently covering. If the cell supports both normal mode and S&F mode, it may become unavailable for the legacy UEs when switching to S&F mode. But at this time, it has not reached the t-service time yet. So it is not clear how NW and legacy UEs should behave in this case. Even if the NW releases the legacy UEs to RRC_IDLE, legacy UEs may not perform neighbouring cell measurement before t-Service. One possible NW implementation is to set the t-Service to the switching time from the normal mode to S&F mode. In this case, t-service serves as the indication of the remaining time of current operating mode. For the S&F UEs, a new t-service time would be needed to indicate the actual stop serving time for the cell.

Switching from S&F mode to normal mode

If the serving cell switches from S&F mode to normal mode, it means the feeder link becomes available at some time point. In this case, if UE knows the switch beforehand when the cell is operating in S&F mode, UE may choose not to initiate ATTACH/TAU immediately, which requires two stage under S&F mode. The overall latency of NAS procedure is shorter if UE can wait until the cell switches to normal mode. And the UEs not supporting S&F can also know when it can camp on this cell, i.e., when the cell switches to normal mode. In this case, the switching remaining time indication is useful.

Proposal 2: RAN2 discusses the usage and impact of the indication of the remaining time of current operating mode before introducing it, e.g., UE may delay some procedure until NW operates in the normal mode and legacy s-service may need to be set to this remaining time.

## 2.2	UE’s RRC state when service link is not available

Another issue that needs to be discussed is UE’s RRC state when the service link is not available. There are the following candidates:

1) UE is released to RRC_IDLE

2) UE is released to RRC_INACTIVE

3) UE stays in RRC_CONNECTED with the AS operation suspended.

For 2) and 3), since the multiple satellite scenario is supported, we need to consider whether it is always possible to transfer the UE context between the satellites. This depends on RAN3 discussion.

Proposal 3: UE can be released to RRC_IDLE when service link is not available. FFS on whether UE can be released to RRC_INACTIVE or kept in RRC_CONNECTED with the AS operation suspended.

## 2.3	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers NW to page UE. In the scenario of S&F, the CN sends MT data to satellite when feeder link is available, the satellite receives MT data and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection and then the NW sends MT data to UE. But if the satellite does not receive any MT data when feeder link is available, the satellite does not need to page UE when service link is available. And the UE doesn’t needs to monitor paging or initiate RRC connection in this case.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which saves UE power.

Proposal 4: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

## 2.4	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Considering the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure

Proposal 5: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Normal mode based on S&F architecture

Proposal 1: Clarify that the Normal mode agreed in the S&F context refers to the S&F architecture with eNB and partial MME/whole CN on board.

Remaining time of current operating mode

Proposal 2: RAN2 discusses the usage and impact of the indication of the remaining time of current operating mode before introducing it, e.g., UE may delay some procedure until NW operates in the normal mode and legacy s-service may need to be set to this remaining time

UE’s RRC state when service link is not available

Proposal 3: UE can be released to RRC_IDLE when service link is not available. FFS on whether UE can be released to RRC_INACTIVE or kept in RRC_CONNECTED with the AS operation suspended.

Paging in S&F satellite operation

Proposal 4: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

Access control

Proposal 5: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 4	References

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 127bis.