3GPP TSG RAN WG2 #128			R2-2410600

Orlando, US, November 18th – 22th, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on the first scope on on-demand SSB SCell operation.

Agreements on OD-SSB@RAN2#127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell@RAN2#127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- Discussion

2.1	Signaling for OD-SSB

RRC based indication

As captured in the FFS on reconfiguration agreed at RAN2#127 [2], whether/how RRC can reconfigure activation/deactivation state and OD-SSB parameters should be further studied.

On reconfiguration of activation/deactivation state, relevant RAN1 agreements were made as follows [3].

| Agreement@RAN1#118 For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. |  |
| --- | --- |
| Agreement@RAN1#118bis For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options |

At least for deactivation of OD-SSB, it was discussed in RAN1 to use RRC signaling to indicate deactivation of OD-SSB that is already configured and activated (which might mean RRC signaling can indicate “deactivation state” in reconfiguration of OD-SSB). Since RAN1 has already started the discussion on how to indicate OD-SSB deactivation, RAN2 should wait for RAN1’s progress for that.

Proposal 1:

- For RRC signaling to indicate reconfiguration for OD-SSB deactivation, RAN2 waits RAN1’s progress/decision.

On RRC based activation of OD-SSB, RAN1 agreed that it was supported at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. This intention should be further clarified from RAN2’s perspective. The procedures for SCell activation/modification defined in TS38.331 5.3.5.5.9, have several patterns based on the indicated RRC message and RRC parameters related to SCell configuration such as:

- SCell addition

- SCell modification

  - With or without ReconfigurationWithSync

  - In RRCResume or RRCReconfiguration

In some cases, through the above procedure, higher layer configures lower layers in a UE to consider the SCell to be in activated state (or deactivated state). We think that RAN1’s intention is OD-SSB can be activated whenever the corresponding SCell is to be activated via RRC signaling, so any cases should be supported where a UE is indicated to consider the SCell to be in activated state by the same RRC signaling.

Proposal 2:

- Support RRC signaling to indicate “activation state” of OD-SSB in a SCell in any cases where a UE is indicated to consider the SCell to be in activated state by the same RRC signaling.

On reconfiguration on some OD-SSB parameters via RRC signalling, we think there are two important factors (1) whether OD-SSB configs are cell common parameters (i.e., configured under sCellConfigCommon) or UE dedicated parameters (i.e., configured under sCellConfigDedicated) and (2) whether the reconfiguration can be applicable to activated OD-SSB or only to deactivated OD-SSB.

For the factor (1), while the legacy SSB (always-on SSB) information is configured as cell common parameters, we think that OD-SSB information can be more like a UE-dedicated signal which might be associated to different BWPs and have different properties for different UEs. Therefore, from signalling perspective, update of OD-SSB parameters can be treated same as the legacy SCell parameters configured in sCellConfigDedicated through SCell modification procedure.

On the factor (2), whether to support reconfiguration of some OD-SSB parameters (e.g., SSB burst periodicity, SSB index) for activated OD-SSB would have some additional impact/consideration in RAN1 and RAN4 such as application timeline and RRM/signalling requirements. This reconfiguration can be considered as “adaptation of OD-SSB” which would be discussed in RAN1, so RAN2 should wait for RAN1’s decision on whether/how to support adaptation of OD-SSB.

In contrast, reconfiguration of OD-SSB parameters for deactivated OD-SSB can be naturally supported, which would not cause additional impact/consideration in RAN1 and RAN4, in a same approach for other legacy SCell parameters from RAN2’s signaling perspective.

Proposal 3:

- For support of RRC based reconfiguration,

  - for deactivated OD-SSB, this can be supported at least from RAN2 perspective.

  - for activated OD-SSB, RAN2 waits RAN1’s decision on how/whether to support adaptation of OD-SSB.

MAC CE based indication

The discussion on signaling design of MAC CE based activation(/deactivation) of OD-SSB should be postponed until it is concluded in RAN1 what parameter sets can be indicated via MAC CE and whether these parameters should be configured via RRC (before MAC CE indication).

Proposal 4:

- For signaling design of MAC CE based activation of OD-SSB, RAN2 waits RAN1’s progress with the following aspects:

  - What parameter sets of OD-SSB can be indicated via MAC CE

  - What parameters sets of the above should be configured via RRC (before MAC CE indication).

Regarding MAC CE based deactivation of OD-SSB, as agreed in RAN1, the options were discussed that a UE assumes OD-SSB is deactivated based on the legacy SCell deactivation procedure (e.g., upon reception of SCell deactivation command, timer for Scell deactivation is expired). At the same time, RAN2 has agreed not to introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A [4]. In our understanding, the RAN2 agreement does not prevent RAN1 discussion from supporting the legacy SCell deactivation command as an indication of OD-SSB deactivation.

Proposal 5:

- Clarify that it is up to RAN1 decision whether UE can be indicated with OD-SSB deactivation by receiving the legacy MAC CE for SCell deactivation.

  - RAN2 agreement: Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- RAN2’s intention is not to introduce NEW MAC CE for a combination of those proposes.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SSB SCell operation for network energy saving.

Proposal 1:

- For RRC signaling to indicate reconfiguration for OD-SSB deactivation, RAN2 waits RAN1’s progress/decision.

Proposal 2:

- Support RRC signaling to indicate “activation state” of OD-SSB in a SCell in any cases where a UE is indicated to consider the SCell to be in activated state by the same RRC signaling.

Proposal 3:

- For support of RRC based reconfiguration,

  - for deactivated OD-SSB, this can be supported at least from RAN2 perspective.

  - for activated OD-SSB, RAN2 waits RAN1’s decision on how/whether to support adaptation of OD-SSB.

Proposal 4:

- For signaling design of MAC CE based activation of OD-SSB, RAN2 waits RAN1’s progress with the following aspects:

  - What parameter sets of OD-SSB can be indicated via MAC CE

  - What parameters sets of the above should be configured via RRC (before MAC CE indication).

Proposal 5:

- Clarify that it is up to RAN1 decision whether UE can be indicated with OD-SSB deactivation by receiving the legacy MAC CE for SCell deactivation.

  - RAN2 agreement: Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- RAN2’s intention is not to introduce NEW MAC CE for a combination of those proposes.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN Meeting #102, December 11th-15th, 2023.

- 3GPP, RAN2#127 meeting, Chairman’s notes

- 3GPP, RAN1#118bis meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes