3GPP TSG-RAN WG2 Meeting #128	R2-2410618

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	 8.12.3

Source:	 Huawei, HiSilicon

Title:	 Enhancements for UE-initiated/event-driven beam management

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, according to the WID as shown below [1] and recent RAN1 inputs [5], objectives relevant to UE-initiated/event-driven beam management (UEIBM) and asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present detailed analysis and provide our considerations on how RAN2 start work on the UEIBM, based on RAN1 agreements so far.

2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting is agreed by RAN1 plenary for Rel-19 MIMO WI i.e., the UE reports CSI only when the event is triggered. This includes specification of the event triggering determination procedure and of the reporting procedure after the event is triggered. In the following, we provide our considerations on the trigger event for event-driven beam management, beam reporting and UL transmission. Then, we consider how to model this in the specifications.

2.1	Event triggering

2.1.1	Application of Time window length and M

The following agreement was made for event triggering in RAN1:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

According to the RAN1 agreements, an event-2 instance occurs when the L1-RSRP of the new beam is a threshold value greater than the current beam. This threshold is configured by the network. Then, there are two cases:

- Case-1: upon occurrence of an event-2 instance, the UE initiates the reporting procedure.

- Case-2 (subject to a UE capability): when M event-2 instances for the same beam occur within a time window, the UE initiates the reporting procedure.

Based on the RAN1 agreements of Case-2, RRC should configure a threshold, time window length and M for controlling UE on triggering event determination (Event-2). It is likely that this event evaluation procedure can also be applied to other MIMO events, such as Event-1 and Event-7. RAN2 can wait for further progress from RAN1 on this.

Observation 1: The RRC procedure should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM, which is at least applicable for Event-2.

Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

UE should maintain a timer and counter to operate during the event triggering determination procedure. According the recent RAN1 agreement, the event instance(s) counting(counter) is per new beam. Therefore, the timer should also be maintained for each new beam.

Proposal 1: For the event triggering determination procedure of UEIBM, the UE should also maintain a timer per new beam.

Currently, BF and RLF are declared upon receiving a certain number of instances of an event from lower layers, which is similar to RAN1 agreement for Case-2 above. However, these procedures have some differences:

- Option-1: In the BF detection procedure, the timer is (re)started upon the reception of an instance, and the counter is incremented. Upon timer expiry, the UE resets the counter to 0. When the counter reaches the configured number of event instances, the UE consider beam failure to be detected.

- Option-2: In the RLF procedure, the timer is started upon reception of N310 consecutive “out-of-sync” instances and continues running until receiving N311 consecutive “in-sync” instances. At the point of timer expiration, RLF occurs.

Either options could be used to trigger beam reporting, however, the BF detection procedure is preferable, as it increments the counter when each event instance is received. This mechanism provides better precision compared to incrementing the counter after multiple event instance (RLF procedure). The choice between the BF detection and RLF procedure falls within the scope of RAN2. A decision should be made, after which a LS should be sent to RAN1.

Proposal 2: The timer is started upon reception of the event instance and the counter is incremented at the reception of each event instance when the timing is running. The timer should be restarted at every event instance (like for BF detection). Following this, a LS should be sent to inform RAN1.

2.1.2	RS resource set

RAN1 also made agreements on the RS resource set configuration for event triggering procedure:

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

It has been agreed that the new beams in the RS resource set used for UEIBM are configured by the NW. According to the legacy CSI measurement configuration, the maximum number of SSB/CSI-RS per resource set is 64. Given that the event instance(s) counting(counter) is performed per new beam, the potential maximum for counters and timers could reach 64, which may be too burdensome for the UE to maintain. Therefore, it is recommended to limit the number of new beams in the RS resource set to a more manageable value. Consequently, RAN2 should send a LS to RAN1 to limit the number of RS in the resource set.

Proposal 3: RAN2 should send a LS to request the RAN1 to limit the number of RS in the resource set for event triggering beam reporting.

2.3	Beam reporting and UL transmission

For MIMO, the UE sends the L1 report to the network using UCI. Two modes (Mode-A and Mode-B) have been agreed by RAN1.

During RAN1 discussions, an alternative option of transmitting the beam report via a MAC CE (Option-1) was considered but ultimately excluded. This decision was guided by a WID which explicitly assumes the reuse of the legacy CSI-RS measurement and reporting framework, where CSI-RS reports are transmitted via UCI. Given that UCI serves as a suitable container for these reports, the reuse of UCI was considered a reasonable approach.

For the UCI-based beam reporting, three options were listed during the discussions:

•	Option 2 (mode A): beam report is transmitted in a UCI in dynamically allocated UL resource.

•	Option 3 (mode B): beam report is transmitted in a UCI in pre-configured UL resources.

•	Option 4: beam report is transmitted in a UCI in pre-configured UL resources not specifically dedicated for UEI beam report.

Initially, option 3 (mode B) was accepted due to several advantages. This mode significantly reduces the complexity for the network as the network does not need to monitor whether a UEI beam report is carried in the pre-configured resource at all times. Additionally, from the network perspective, mode B offers better resource efficiency, as the pre-configured resources can be shared among UEs.

Then, option 2 (mode A) was also discussed. One key advantage of mode A is that the network can schedule flexibility to avoid collision issue on the second channels. It has very high resource utilization. However, the dynamic resource allocation of step 2 in mode A could introduce some latency. Therefore, mode A was also agreed by RAN1 as a fundamental feature for the UE.

Regarding option 4, the uplink resource to carry the UEIBM reporting is pre-configured, the absence of pre-indication requires the network to monitor all pre-configured uplink resources for potential UE beam reporting. This could result in unused resources or lead to collisions if multiple UEs share the same pre-configured resources. Therefore, from the perspective of network complexity and resource utilization, option 4 is not advantageous.

In conclusion, there are two modes used for the beam reporting procedure: mode A (option 2) and mode B (option 3).

2.3.1	Mode A reporting procedure

Mode A reporting has the following procedure:

Figure.1 Mode A for UE-initiated/event-driven beam reporting

On mode A configuration, RAN1 has the following agreement:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following. - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID Above applies at least for the single CC case.

Based on the above agreement, two options are considered for mode A configuration: dedicated SR or new UCI type. The first PUCCH in mode A provides a functionality very similar to SR, as it requests resources for PUSCH and is followed by the network response (DCI) that indicates the allocated PUSCH resources for beam reporting. Reusing the dedicated SR configuration and procedure would simplify the MAC procedure design.

Proposal 4a: For mode A, re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

2.3.2	Mode B reporting procedure

Mode B reporting has the following procedure:

Figure.2 Mode-B for UE-initiated/event-driven beam reporting

On mode B configuration, RAN1 has the following agreement:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

SR is currently used to request a DG for new transmission. However, in mode B, it is used to indicate a CG occasion that is used for beam reporting, so the purpose and the procedure are different. On the other hand, the MAC specification could be simpler if SR is used for mode B. The decision can be left to RAN1. Regardless of whether the SR or a new UCI type is selected, the procedures can still be described in the MAC specification.

Proposal 4b: For mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

2.3.3	Cross-CC beam reporting

RAN1 has agreed that cross-CC beam reporting can be applied for both mode A and mode B.

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for both Mode-A and Mode-B, the first PUCCH and the second PUSCH can be from the same or different CC(s) - FFS: whether the first PUCCH and the second PUSCH should be from the same PUCCH group - The first PUCCH and the second PUSCH should be in the same CG - FFS: Different CGs

In the case of CA, both PCell and SCell(s) share the same MAC entity. if the first PUCCH and the second PUSCH are transmitted from different CC(s) with different PUCCH group, it does not have any impact MAC layer. However, in the case of DC, the UE is configured with two MAC entities: one MAC entity for the MCG and one MAC entity for the SCG. If the first PUCCH and the second PUSCH are transmitted from different CGs, it requires extra interaction between different gNBs, which delays UL transmission of beam reporting. Therefore, this configuration should be precluded from RAN2 perspective.

Proposal 5: The scenario in which the first PUCCH and the second PUSCH transmitted from different cell groups should be explicitly excluded.

2.3.4	Second UL channel Transmission procedures

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data

It is noted that beam reporting for Mode A and B are carried in UCI on the second UL channel. From the RAN1 discussion, the second UL channel should be PUSCH, and with the possibility of using PUCCH FFS in mode-B. For the second UL channel, it needs to consider the scenario where a skipping mechanism may be applied.

Observation 2: the second UL channel could be PUSCH for both mode-A and mode-B, FFS of using PUCCH in mode-B.

In the legacy skipping mechanism with CSI reporting in Rel-15, there is an aperiodic CSI reporting to transmit on PUSCH, the UL grant cannot be skipped even there is no available data in MAC layer since the prerequisite of CSI multiplexing in physical layer is a MAC PDU generated from MAC layer, otherwise the implementation has to be changed. The semi-persistent CSI reporting on PUSCH is generated by PHY layer, which is not allowed to be multiplexed with UL data. This procedure is invisible to the MAC layer, so it does not involve UL skipping mechanism.

TS 38.214 clause 5.1.3 CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

For the periodic and semi-periodic CSI reporting on PUCCH, the UL grant is skipped if there is no available data, i.e. the CSI cannot be multiplexed on PUSCH in case PUCCH and PUSCH overlapping in time domain, and if there is some available data, the CSI is multiplexed on PUSCH without PUCCH transmission, those two cases cannot be distinguished by network. In this case network has to attempt decoding CSI on both PUSCH and PUCCH. In Rel-16, an enhancement was introduced for periodic and semi-periodic CSI reporting to reduce network complexity. Specifically, the UL grant cannot be skipped if CSI is to be multiplexed on this PUSCH transmission even there is no available data, i.e. network can always assume the CSI is always transmitted on PUSCH in case PUCCH and PUSCH overlapping.

TS 38.321 clause 5.4.3.1.3 The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Observation 3: UL grant cannot be skipped for periodic and semi-periodic CSI multiplexed on PUSCH, nor for aperiodic CSI reporting on PUSCH.

In Rel-19, a new CSI is introduced, i.e. the UE-initiated/event-driven beam reporting. How UL skipping to work with this new CSI needs to be studied regarding Mode-A and Mode-B based beam reporting.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least Option-3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

For mode-A beam reporting, it is more likely similar to the aperiodic CSI reporting, which cannot be skipped if there is no available UL data. Based on the above agreements regarding mode-B, RAN2 should send a LS to RAN1 to ask whether the UEI beam reporting is visible to MAC layer. If it is not, discussion of the UL skipping mechanism is unnecessary; however, if it is, RAN2 should consider the potential impact on the MAC layer.

Proposal 6: RAN2 should send a LS to RAN1 to ask whether the UEI beam reporting for mode-B is visible to MAC layer.

2.4	Specification aspects

RAN1 is discussing in which specification to capture the UEIBM. For the BFR procedure, two components are used for beam failure detection, BFI_COUNTER and beamFailureDetectionTimer. Once beam failure is detected, the UE can perform BFR via random access or MAC CE with beam failure and candidate beam information. These procedures are captured in MAC.

UEIBM contains the event evaluation and reporting procedure, the event evaluation is also based on a counter and a timer. The whole procedure is quite similar to beam failure detection and BFR so it makes sense to also capture it in MAC.

Proposal 7: Capture the event evaluation and reporting (using timer and counter) for UEIBM in the MAC specification.

There are two possible approaches for event evaluation modelling:

-	Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

-	Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

For Option-1, if the comparison occurs in the PHY layer, it sends the instance to the higher layers. Alternatively, for Option-2, if conducted in the higher layers, the PHY layer sends the beam quality metrics, such as L1-RSRP, to the higher layers, allowing them to perform the comparison based on beam quality. Both two methods are available and the final decision could up to RAN1 discussion.

Proposal 8: It’s up to RAN1 to discuss which option is used.

- Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

# 3	Conclusion

In this paper, we have addressed the objective regarding initial analysis on RAN2 impact for MIMO. There are the following observations and proposals, and the draft LS to RAN1 is attached in Annex.

Event triggering

Observation 1: The RRC procedure should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM, which is at least applicable for Event-2.

Proposal 1: For the event triggering determination procedure of UEIBM, the UE should also maintain a timer per new beam.

Proposal 2: The timer is started upon reception of the event instance and the counter is incremented at the reception of each event instance when the timing is running. The timer should be restarted at every event instance (like for BF detection). Following this, a LS should be sent to inform RAN1.

Proposal 3: RAN2 should send a LS to request the RAN1 to limit the number of RS in the resource set.

Beam reporting and UL transmission

Proposal 4a: For mode A, re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

Proposal 4b: For mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

Proposal 5: The scenario in which the first PUCCH and the second PUSCH transmitted from different cell groups should be explicitly excluded.

Observation 2: the second UL channel could be PUSCH for both mode-A and mode-B, FFS of using PUCCH in mode-B.

Observation 3: UL grant cannot be skipped for periodic and semi-periodic CSI multiplexed on PUSCH, nor for aperiodic CSI reporting on PUSCH.

Proposal 6: RAN2 should send a LS to RAN1 to ask whether the UEI beam reporting for mode-B is visible to MAC layer.

Specification aspects

Proposal 7: Capture the event evaluation and reporting (using timer and counter) for UEIBM in the MAC specification.

Proposal 8: It’s up to RAN1 to discuss which option is used.

- Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

# 4	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #118 chairman notes;

- RAN1 #118bis chairman notes;

- TS 3GPP 38.214 V18.3.0;

- TS 3GPP 38.321 V18.3.0.

# 5	Annex

3GPP TSG RAN WG2 #128	 		R2-24xxxxx

Orlando, USA, Nov. 18th – 22nd, 2024

Title:	(draft) LS on Rel-19 UEIBM

Response to:	-

Release:	Rel-19

Work Items:	NR_MIMO_Ph5-Core

Source:	RAN WG2, Huawei, HiSilicon

To:	RAN WG1

CC:	-

Contact Person:

Name:					TBD

E-mail Address:		TBD

Attachment:	               -

1. Overall Description:

1. RAN2 agreed the first PUCCH and the second PUSCH should be from the same PUCCH group are not supported.

Besides, RAN2 would like to ask the following questions.

2. For UEI beam reporting, RAN2 noticed the RAN1 agreement on Type-1 CG PUSCH can NOT carry UL-SCH. And RAN2 would like to ask RAN1 whether the Mode-A and Mode B grant for event driven beam report is visible to MAC layer, i.e. whether or not to generate an empty MAC PDU delivered to PHY layer.

Question 1: whether the Mode-A and Mode B grant for event driven beam report is visible to MAC layer, i.e. whether or not to generate an empty MAC PDU delivered to PHY layer?

3. RAN2 discussed two options on how to model the specification on UEIBM and would like to ask RAN1.

Question 2: which option is used for event triggering determination.

- Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to MAC layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to consider above information for Rel-19 MIMO and to provide answers to the above questions.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG1 Meeting #129				February 17 to 21, 2024		Athens, Greece

TSG-RAN WG1 Meeting #129-bis			April 07 to 11, 2024			China, CN