3GPP TSG-RAN WG2 Meeting #128                                                             R2-2410629

Orlando, USA, Nov. 18th – 22rd, 2024

Agenda Item	:	8.2.4

Title	:	Discussion on random access for ambient IoT

Source	:	Google Ireland Limited

Work Item	:	FS_Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we discuss the access timing for ambient IoT and discuss the need for a backoff function that is used to spread device’s transmission over access occasions. It is also proposed to discuss whether Msg2 can contain only one response to a Msg1 transmission or multiple responses to multiple Msg1 transmissions.

# 2	Discussion

In RFID, the reader (i.e. interrogator) transmits a Select command to select devices (i.e. tags) and then transmits a Query command to start an inventory round. The inventory round consists of at least one contention period followed by a non-contention period. Devices contend to transmit a 16-bit random ID in the contention period. If a device receives an ACK that acknowledges the16-bit random ID, the winning device transmits its data in the non-contention period.

In ambient IoT, a contention period starts with a paging message that initiates a contention-based random access. At the end of the contention period, a non-contention period starts. In the non-contention period, the reader can transmit R2D data or schedule D2R transmission, as illustrated in Fig. 1. It is proposed to agree the terminology of contention period and non-contention period for ease of discussion afterward.

Fig. 1: An example of access timing for ambient IoT.

Proposal 1:  RAN2 to agree the terminology of contention period and non-contention period in Fig.1.

There are also some discussion on slotted ALOHA in the [POST127] [033] discussion. Slotted ALOHA is a multiple access scheme based on the ALOHA multiple access. In ALOHA protocol, device can immediately transmit data when it has data in buffer. Slotted ALOHA improves the channel utilization of ALOHA by dividing time into equal-size slot. Device can only transmit data at the beginning of a time slot.

To spread a large amount of devices in time requires a backoff function. In the legacy random access procedure, gNB may indicate a BI to UE. UE uses the BI to determine which PRACH occasion to transmit a random access preamble. UE waits for the selected PRACH occasion to transmit a random access preamble. In RFID devices selects one of Q slots to transmit a 16-bits random ID.

Backoff function is also useful for ambient IoT, especially in the case that a large amount of IoT devices is near the reader. For example, an ambient IoT device can be attached to each item on a rack in a warehouse. In such a case, a reader should be able to indicate a backoff to spread devices’ transmissions in times.

The reader could indicate the backoff indicator in a paging message as in RFID, or alternatively, the reader could include the backoff indicator in Msg2 similar to legacy random access procedure.

Proposal 2:  Backoff function should be supported for Msg1 transmission.

Proposal 3:  Backoff indicator can be included in a paging message or Msg2.

In RFID, after the reader transmits a Query command, there is only one access occasion following the Query command. Devices contend to transmit a 16-bit random ID in the access occasion. At most one device will win the contention. The reader transmits a ACK to acknowledge the reception of the random ID from the winning device.

In ambient IoT, since slotted aloha is agreed to be the multiple access scheme, there may have more than one access occasion. As a result, it may happen that more than one device successfully transmit a random ID to the reader. In that case, it is unclear whether Msg2 can contain only one response for a Msg1 transmission as illustrated in Fig.1 or is allowed to contain multiple responses, each of which respond to a successful Msg1 transmissions in Fig. 2.

Fig. 2: Msg2 contains only one response for a successful Msg1 transmission.

Fig. 3: Msg2 contains multiple responses for successful Msg1 transmissions.

Proposal 4: RAN2 to discuss whether Msg2 can contain multiple responses to Msg1 transmissions.

# 3	Conclusions

In conclusion, we have the following proposals:

Proposal 1:  RAN2 to agree the access timing in Fig.1.

Proposal 2:  Backoff mechanism should be supported for Msg1 transmission.

Proposal 3:  Backoff indicator can be included in a paging message or Msg2.

Proposal 4:  RAN2 to discuss whether Msg2 can contain multiple responses to Msg1 transmissions.