3GPP TSG-RAN-WG2 Meeting #128	R2-2410705

Orlando, USA, 18th– 22nd Nov, 2024

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	discussion on C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#127 meeting, RAN2 reached the following agreements [1];

Agreements: The following connections are assumed as a baseline to be needed: - From last Relay UE perspective:  A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and  A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED. - From intermediate relay UE perspective (including first relay UE):  A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and  An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED. FFS what RRC states are supported for the intermediate relay UE. - From U2N Remote UE perspective:  A direct (non-relayed) PC5 connection with Intermediate Relay UE, and  An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED . The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18). The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

In this paper, we discuss how to introduce multi-hop relay. As needed, we’ll refer to the SA2’s conclusion [2], use case of multi-hop relaying [3] and requirement of multi-hop relaying [4].

Discussion

According to the WID, RAN2 needs to discuss (A) framework of discovery and (re)selection, (C) SRAP and QoS handling, and (D) C-plane procedure. And RAN2 needs to consider forward compatibility while RAN2 discusses one additional hop relay. Considering future compatibility, the same term (e.g. intermediate relay UE) should be used for the new relays (both for first/second relay UEs), but we will use individual UE names used in the WID where appropriate.

We also understand from Ref. 4 that, in principle, multi-hop relays are deployed to save power and extend coverage. Then, the required QoS should be satisfied.

  - Configuration for multi-hop relaying

RRC connection maintenance

RAN2 needs to discuss RRC connections between UEs and NW, and PC5 RRC connections between UEs. In the last meeting, RAN2 reached following agreements;

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

According to the agreements, we can illustrate the connections as shown in figure 1.

Figure1. RRC connections between UEs and between UEs and gNB for two-hop U2N relay

Next, RAN2 needs to discuss whether the legacy U2N relay mechanism can work on this architecture.

In this phase, we should discuss whether the current mechanism can be reused. We discuss following contents in this paper;

- Paging delivery

- SIB delivery

- RRC connection establishment

- SRAP configuration for C-plane and U-plane

- QoS handling

- Failure handling

Firstly, current paging delivery is supported by following mechanism [5];

- Method 1: The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space

- Method 2: The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).

Method 1 can be available if the U2N relay UE can receive paging message. For that purpose, the U2N relay UE should be the last relay UE in multi-hop relay, it means that PC5-RRC connection between remote UE and the last relay UE is required to forward the paging message. Furthermore, if RAN2 supports PC5-RRC connection between remote UE and the last relay UE, RAN2 implicitly support U2U multi-hop relay (i.e. violate the WID definition). If we consider that paging message for remote UE is delivered hop-by-hop, RRC-level delivery mechanism might be needed. However, this causes unnecessary processing delays and is unsuitable for the purpose of paging. Therefore, Method 1 cannot be used for multi-hop relay.

Observation 1. One of the paging delivery mechanism “The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space” cannot be reused if RAN2 assume that there is no E2E multi-hop PC5-RRC connection between the remote UE and the last relay UE.

Method 2 can be available if the first relay UE is in RRC_CONNECTED. If the first relay UE is in IDLE/INACTIVE, second/last relay UE needs to monitor first relay UE’s (and/or remote UE’s) POs or gNB sends dedicated RRC message including paging message to the second/last relay UE, and then gNB sends dedicated RRC message including paging message to the second/last relay UE. It seems wasteful from the perspective of latency and radio resource. And the last relay UE should be in RRC_CONNECTED state to receive some data for the paging for the remote UE. So, the last relay UE and the first relay UEs should always be in RRC_CONNECTED. Thus, Method 2 can be used for multi-hop relay if the last relay UE and the first relay UE is in RRC_CONNECTED.

Observation 2. One of the paging delivery mechanism “The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).” can be reused if the last relay UE and first relay UE are in RRC_CONNECTED state.

Proposal 1. Paging message for the remote UE is always delivered by dedicated RRC message to the first relay UE.

Proposal 2. For paging delivery, the last relay UE and the first relay UE is in RRC_CONNECTED.

Secondary, current SIB delivery is supported by following mechanism [5];

- The remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected relay UE and receive the SIB(s) from the relay UE via PC5-RRC message.

- The remote UE in RRC_CONNECTED receives SIB1 and other SIBs by dedicated RRC message and performs on-demand SI request.

- For SIB1 forwarding, for L2 U2N Remote UE, both request-based delivery and unsolicited forwarding are supported by L2 U2N Relay UE. If SIB1 changes, for L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE always forwards SIB1.

The last relay UE may not monitor SIB(s) directly on Uu interface. However, relay UE in RRC_CONNECTED can performs on-demand SI request for the requested SIB(s). We see no particular problems in applying these mechanisms to multi-hop relaying. So, legacy SIB delivery mechanism can be reused for multi-hop relaying.

Proposal 3. The legacy SI retrieval mechanisms (including SIB delivery) can be reused for multi-hop relaying.

If RAN2 determines that legacy SIB/paging delivery works in multi-hop system, remote UE can be in any RRC states.

Proposal 4. remote UE can be in any RRC states if legacy SIB/paging delivery works in multi-hop system.

Thirdly, remote UE needs to establish RRC connection with the gNB for communication. RAN2 discuss in the email discussion[6]. We think both approaches might work at establishment phase. But for approach 2, intermediate relay UEs derive L2 configuration by itself using SIB/pre-configuration, NW needs to reconfigure the UEs if one intermediate relay UE becomes to be in RRC_CONNECTED. This can result in unnecessary PC5-RRC signalling. For approach 1, NW needs to reconfigure all relay UEs for the remote UE upon RRC connection establishment and reconfiguration. However, after the configuration is applied, relay UEs except the first relay UE can be in any RRC state. Thus, we prefer approach 1 but can discuss in meeting. Due to the remaining TU, RAN2 should choose between approach 1 and approach 2.

Proposal 5. RAN2 choose between approach 1 and approach 2 as shown in [6].

SRAP configuration

In RAN2#127, RAN2 agreed with following protocol stack for control plane as below.

Figure 7. candidate protocol stack for C-plane in multi-hop relay

SRAP layer supports the routing function, i.e. determination of egress RLC channel/link, determination of UE/BEARER ID field and data transfer. For routing function, gNB configures relay UE(s) and remote UE with local UE ID and mapping rule. Then, the relay UE(s) and remote UE determines the route of the packet based on the SRAP header of the packet. There are two types of SRAP header, one is for U2N SRAP data PDU and another one is for U2U SRAP data PDU. The both of SRAP header has common fields, i.e. D/C field, BEARER ID field and UE ID field(s). D/C field is exactly same. So, it can be reused for multi-hop relay. The value of BEARER ID refers appropriate bearer identity. It means that, in case of U2N relay, the value refers srb-Identity or drb-Identity (minus 1) and, in case of U2U relay, the value refers SL-SRB identity or slrb-PC5-ConfigIndex. In the multi-hop case, the U2N relay format can be reused since E2E RB is not sidelink specific bearer. The discussion point is UE ID field. In case of the U2N relay, one UE ID of remote UE’s local ID and, in case of U2U relay, two UE IDs of source remote UE and destination remote UE. In our understanding, two IDs are needed for the destination remote UE that receives the SRAP PDU to determine to which PDCP the corresponding SRAP SDU should be transferred if E2E SL-DRBs from multiple source UEs are mapped to one ingress PC5 RLC channel. In case of multi-hop relay, such a situation occurs in gNB. Then, gNB understands who is source UE based on one UE ID field (i.e. remote UE’s local ID). Also, the intermediate UEs can understand whether the SRAP SDU is UL or DL based on the received RLC even if there is only one UE ID, because one RLC entity cannot be established for multiple destinations. Therefore, SRAP header for legacy U2N relay can be reused.

Proposal 6. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 7. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay, i.e. remote UE’s local ID is set to the UE ID.

And, as with legacy, to avoid the local ID collisions, gNB should allocate local ID for each remote UE for managing sources of UL data and destinations of DL data.

Proposal 8. gNB should allocate local ID for remote UE(s) for SRAP configuration.

QoS handling

The latency requirement is important when we consider the QoS handling. Especially, some use cases, e.g. public safety, wearable and V2X, needs very low E2E latency. Therefore, optimization should be done to achieve low latency.

Proposal 9. QoS related optimization should be considered since very low E2E latency is required for some use cases.

When we consider QoS related optimization, it is important to know achievable QoS at each hop. Here, gNB can guarantee at least one hop (i.e. last relay hop) QoS if gNB allocate sidelink and/or uplink grant for the last relay UE using resource allocation mode 1. Thus, we propose that the principles of resource allocation for U2N relay operation follow the legacy mechanism.

Proposal 10. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 3. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

In U2U relay, U2U relay UE split the E2E QoS for each hop, and then each UE configures each hop configuration to guarantee the split QoS. Because U2U relay UE knows quality of both hops and can configure appropriate values. In multi-hop relay, there is no special UE like a U2U relay UE. However, if gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality. Thus, we propose that the gNB sets the appropriate (split) QoS to be met for each hop and UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Observation 4. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 11. The gNB sets the appropriate (split) QoS to be met for each hop.

Proposal 12. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

For QoS parameters to be split, same as U2U relay, only PDB should be split. For example, split PER extremely decrease as the number of hops increases, making it difficult to divide it accurately. Therefore, we consider it a compromise in the multi-hop relay system to assume that gNB sets appropriate values for each hop.

Proposal 13. QoS (PDB) should be split by the gNB, and it is up to gNB how to split the QoS parameters including PDB.

- 2.2	Failure handling

For path failure handling, we can consider legacy mechanism as baseline. In legacy, there are some failure cases;

- U2N relay UE detects Uu RLF/ detects RRC failure/ receives HO command/ initiates cell-reselection

- U2U relay UE detects PC5 RLF

- U2N remote UE detects PC5 RLF/ detects RRC failure

- U2U remote UE detects PC5 RLF/ detects PC5 RRC failure

In case of multi-hop relay, intermediate relay UE is like a U2N remote UE and a U2U relay UE. Therefore, there are following failure cases from remote UE’s point of view;

- Last relay UE detects Uu RLF

- Last relay UE detects RRC failure

- Last relay UE receives HO command

- Any of the intermediate UEs detects PC5 RLF

- Any of the intermediate UEs detects RRC failure

- Any of the intermediate UEs receives path failure notification

- Remote UE detects PC5 RLF

- Remote UE detects RRC failure

- Remote UE receives path failure notification

In case of intermediate UE’s failure, legacy NotificationMessageSidelink can be reused. If the intermediate UEs detecting any failures needs to notify remote UE(s) of the failure, there should be E2E PC5-RRC connections for each UE combination. However, as mentioned above, establishment of E2E PC5-RRC connections for each UE combination will increase maintenance costs and complicate the system. Therefore, UE should use hop-by-hop PC5-RRC message in order to send a notification to remote UEs.

Proposal 14. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 15. NotificationMessageSidelink should be sent hop-by-hop if needed.

Here, we consider the case where failure information is notified for all remote UEs if last relay UE or intermediate UE detects a failure. Firstly, the UE detecting the failure notifies the failure to the nearest UE located on the branch side of the tree, and then the nearest UE notifies the failure to other nearest UE(s). Thus, all remote UEs in the outer branch of the tree to which the UE detected the failure belongs will perform RRC re-establishment. This behavior needs huge radio resource instantaneously. In addition, remote UE configured with multi-hop relay is unlikely to discover a new cell, and is also unlikely to discover any relay UEs other than the currently connected relay UE. Therefore, there are cases where it is effective, for a remote UE configured for multi-hop relay, to remain connected to the relay and wait for the relay to reconnect to the gNB. It means that the remote UE doesn’t have to perform RRC re-establishment if a connected (intermediate) relay UE initiates RRC re-establishment procedure.

Observation 5. There are cases where it is effective, for a remote UE configured for multi-hop relay, to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 16. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

We also consider the content of notification UE sends to another UE(s) when UE received the notification. One option is setting the same indicationType with the received notification. However, UE cannot understand whether the event indicated by the indicationType is occurring in the UE sent the notification, or the UE simply reflecting it. If UE doesn’t have to perform RRC re-establishment, such information is important to determine UE behavior. Then, another option is setting specific indicationType (e.g. receiving_Notification, perform_reestablish) for multi-hop.

Proposal 17. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

Conclusion

Observation 1. One of the paging delivery mechanism “The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space” cannot be reused if RAN2 assume that there is no E2E multi-hop PC5-RRC connection between the remote UE and the last relay UE.

Observation 2. One of the paging delivery mechanism “The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).” can be reused if the last relay UE and first relay UE are in RRC_CONNECTED state.

Proposal 1. Paging message for the remote UE is always delivered by dedicated RRC message to the first relay UE.

Proposal 2. For paging delivery, the last relay UE and the first relay UE is in RRC_CONNECTED.

Proposal 3. The legacy SI retrieval mechanisms (including SIB delivery) can be reused for multi-hop relaying.

Proposal 4. remote UE can be in any RRC states if legacy SIB/paging delivery works in multi-hop system.

Proposal 5. RAN2 choose between approach 1 and approach 2.

Proposal 6. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 7. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay, i.e. remote UE’s local ID is set to the UE ID.

Proposal 8. gNB should allocate local ID for remote UE(s) for SRAP configuration.

Proposal 9. QoS related optimization should be considered since very low E2E latency is required for some use cases.

Proposal 10. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 3. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

Observation 4. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 11. The gNB sets the appropriate (split) QoS to be met for each hop.

Proposal 12. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Proposal 13. QoS (PDB) should be split by the gNB, and it is up to gNB how to split the QoS parameters including PDB.

Proposal 14. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 15. NotificationMessageSidelink should be sent hop-by-hop if needed.

Observation 5. There are cases where it is effective for a remote UE configured for multi-hop relay to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 16. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

Proposal 17. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

Reference

- R2-24xxxxx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- TR23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3

- TR23.866, Study on system improvements for energy efficiency

- TS22.261, Service requirements for the 5G system

- TS38.300, NR; NR and NG-RAN Overall description; Stage-2

- R2-24xxxxx, Report of [POST127][402][Relay] Multi-hop relay control plane