3GPP TSG RAN WG2 #128                                                                                                     R2-2410725 Orlando, USA, November 18 – 22, 2024

Agenda Item:   	8.9.3
Source:             	Skylo Technologies
Title:                  	Discussion on EDT Enhancements for IOT NTN

Document for: 	Discussion and Decision

______________________________________________________________________________________________

- Introduction

The RAN Plenary has approved a work item for NB-IOT NTN to focus on uplink capacity enhancements. As part of this work item, in the last meeting, RAN2 agreed to continue working on Preamble-less EDT and Msg4 enhancements.

Skylo is a global Non-Terrestrial Network (NTN) service provider, paving the way for satellite and cellular convergence. Skylo’s NTN network uses existing Geostationary satellites and 3GPP NB-IOT NTN standard to enable IOT devices, sensors, smartphones, wearables and other devices to connect via satellite communication.

In this contribution we would like to further discuss the points from the satellite network operator points of view.

- Discussion

2.1 Proposal to Support Multicast Msg4 ACK for both Preamble-less as well as Preamble based EDT Procedure

In the last meeting RAN2 agreed to continue to work on CB-EDT along with Msg4 enhancements. In this contribution we propose to support Multicast Msg4 ACK for Preamble based EDT procedure too.

Enhanced Data Transmission (EDT) in NB-IoT provides expedited data transfer by reducing access latency and overhead.

In the context of preamble-less EDT being discussed for NB NTN, where Msg1/Msg2 are omitted, optimizing Msg4 ACK delivery is essential for minimizing resource usage while maintaining reliability.

This proposal aims to introduce the same multicast Msg4 ACK mechanism, leveraging shared responses to multiple User Equipment (UE) for efficient overhead reduction for existing EDT specification as well.

Proposal for Shared Msg4 Transmission for current EDT mechanism

Proposal 1: We propose to extend the shared response approach for Msg4 in the preamble-less EDT currently being finalized to the preamble-based EDT procedures too.

Multicast Msg4 Composition: Msg4 would contain multiple contention resolution identities, each corresponding to different UEs with successfully decoded Msg3 transmissions.

Contention Resolution Identity: Each UE’s contention resolution identity could be derived from either the UE identity present in RRCEarlyDataRequest in Msg3 as being discussed for preamble-free EDT [1][2].

Benefits of Multicast Msg4 for all EDT procedures

Resource Efficiency and overhead: By transmitting a single Msg4 to multiple UEs, the network reduces PDSCH and signaling resource usage, enhancing overall capacity and accommodating more UEs within the same resource pool.

Simplified Network Operations: With multiple UEs sharing Msg4, the network’s contention resolution process becomes more efficient, allowing simultaneous responses to several UEs, reducing the need for individual Msg4s.

2.2 Preamble-less and Preamble based EDT Support in RRC Connected state

We also propose to extend both CB-msg 3 EDT being discussed in Rel 19 and CP-EDT released as part of Release 15 to be supported for both RRC connected as well as RRC Idle mode. EDT feature helps in saving resources even when the device is in an RRC-connected state. Hence, the feature should be extended to RRC Connected mode too.

Proposal2: Support CB-msg3 EDT and CP-EDT for both RRC connected and RRC Idle mode.

- Conclusion

The observations and proposals discussed in the contribution are summarized below:

Proposal 1: We propose to extend the shared response approach for Msg4 in the preamble-less EDT currently being finalized to the preamble based EDT procedures too.

Proposal 2: Support CB-msg3 EDT and CP-EDT for both RRC connected and RRC Idle mode.

- References

- 3GPP TSG-RAN WG2 #127bis, R2-2409181 , UL capacity enhancements for IoT NTN, Ericsson.

- 3GPP TSG-RAN WG2 #127bis, R2-2408896, Discussion on EDT enhancements, Qualcomm