3GPP TSG-RAN WG2 Meeting #128	R2-2410734

Orlando, USA, 18th – 22nd November 2024

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re)selection for multi-hop U2N relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discussed. This contribution discusses the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.1 Terminologies

RAN2 discussed multi-hop relay terminologies and had the following agreements.

Agreements:

n-hop relaying involves n relay UEs by definition.

Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

FFS if we will capture the requirements according to these terms when implementing CRs.

FFS link terminology.

RAN2 intend to align with SA2 terminology when it is stable

During the discussion, there are still some mis-alignment descriptions about the relationship between the different UEs. In order to make the discussion more easier, it is proposed to use the following terminologies for the relationship between different relay UEs.

- It is proposed to use the following terminologies for the relationship between different relay UEs.

- Parent relay UE: the relay UE which can be connected by the current relay UE on the direction from Remote UE to last relay UE.

- Child relay UE: the relay UE which can be connected by the current relay UE on the direction from last relay UE to Remote UE.

## 2.3 Discovery

For discovery transmission, RAN2 had the following agreements

Agreements:

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

FFS discovery conditions for the intermediate relay UEs.

For discovery transmission in Intermediate Relay UE, the existing Rel-18 U2U discovery transmission mechanism can be taken as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

- For discovery transmission in Intermediate Relay UE, takes Rel-18 U2U mechanism as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

For Model B discovery response transmission, one issue is whether last Relay UE needs to check the PC5 link quality of the receiving link satisfies a PC5 threshold. This is also similar to U2U relay, and it is beneficial for the last Relay UE to check the receiving link.

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

For the AS container included in discovery message, it is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

## 2.4 Relay (Re)selection

In exsiting U2N or U2U relay, the Remote UE (re)selects candidate Relay UE considering the hop link quality (SD-RSRP) between the Remote UE and the Relay UE, similar mechanism can be reused for Remote UE to (re)select the candidate Intermediate Relay UEs. And the Remote UE considers an Intermediate Relay UE for which SD-RSRP exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

For Intermediate Relay UE, it can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the next Relay UE (U2N Relay UE for two-hops relay or Intermediate Relay UE for three-hops relay). The Intermediate Relay UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers an Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

Similar with existing U2N relay, the Remote UE can trigger to reselect a candidate Intermediate Relay UE when PC5 link quality between the Remote UE and the current Intermediate Relay UE is below a threshold; or the current PC5 link between the Remote UE and the Intermediate Relay is released due to e.g. RLF, released by upper layer etc.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

When the intermediate relay UE detects PC5 RLF, how to handle the failure should be discussed. One possible way is the intermediate relay UE notifies the Remote UE via PC5 and the Remote UE reselects the relay path. In this way, any PC5 link failure will trigger the Remote UE to reselect a path, which will consume signalling overhead and increase service interruption time, and if more relay UEs are added into the multi-hop relay path, there could be more signaling overhead introduced. Another way is, the intermediate relay UE can trigger relay reselection and tries to reselect candidate path towards the last relay UE. This way enables local link recovery and does not trigger the whole link recovery from the Remote UE. It is proposed that the intermediate Relay UE can trigger to reselect the next Relay UE when PC5 link quality between the Intermediate Relay UE and the next Relay UE is below a threshold; or the current PC5 link between the Intermediate Relay UE and the next Relay is released due to e.g. RLF, released by upper layer etc.

- When RLF is detected or the current link quality is below a threshold between the Intermediate Relay UE and the U2N Relay UE, the Intermediate Relay U2N triggers When the Intermediate Relay detects PC5 RLF, the Intermediate Relay UE does not need to notify the downstream UE (the Remote UE or Intermediate Relay UE)

- The Intermediate Relay UE can trigger to reselect a candidate last Relay UE when When RLF is detected or the current link quality between the Intermediate Relay UE and upstream Relay UE is below a threshold.

- Conclusion

Terminologies

- It is proposed to use the following terminologies for the relationship between different relay UEs.

- Parent relay UE: the relay UE which can be connected by the current relay UE on the direction from Remote UE to last relay UE.

- Child relay UE: the relay UE which can be connected by the current relay UE on the direction from last relay UE to Remote UE.

Discovery

- For discovery transmission in Intermediate Relay UE, takes Rel-18 U2U mechanism as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

Relay UE (re)selection

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate upstream Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers a Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

- When the Intermediate Relay detects PC5 RLF, the Intermediate Relay UE does not need to notify the downstream UE (the Remote UE or Intermediate Relay UE)

- The Intermediate Relay UE can trigger to reselect a candidate last Relay UE when When RLF is detected or the current link quality between the Intermediate Relay UE and upstream Relay UE is below a threshold.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet