3GPP TSG-RAN WG2 Meeting #128                                                      R2-2410735

Orlando, USA, 18th – 22nd November 2024

Source:	Qualcomm Incorporated

Title:	Control procedure for multi-hop L2 based U2N relay

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay was approved as Rel-19 item, and this contributions discuss the contron plane issues.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.1 Features to be supported

In Rel-17 L2 U2N relay, the basic features are supported as following. For L2 multiple U2N relay, these features should also be supported.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- Access control, location area update

- Positioning

- The following basic functions should be supported in Multi-hop U2N relay.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- Access control, location area update

- Positioning

## 2.2 Basic functionalities supported by Approach 1 and Approach 2

In the email discussion of [Post127][402][Relay] Multi-hop relay control plane (InterDigital) [3], there are two approaches are proposed to support control plane for multi-hop U2N relay. This section discusses the basic functionalities support by Approach 1 and Approach 2. In Approach 1, any intermediate Relay UE is required to be connected with the Remote UE’s serving gNB; in Approach 2, intermediate Relay can be in any RRC state.

Table 1:  Basic functionalities support

| Basic functionalities | Solutions in Approach 1 | Solutions in Approach 2 |
| --- | --- | --- |
| RRC connection establishment | Each relay UE needs to establish Uu RRC connection The child UE Uu RRC connection establishment is on top of the parent UE Uu RRC connection establishment successfully | Intermediate Relay UE does not establish Uu RRC connection |
| QoS/RLC Channel | gNB implementation to ensure QoS; gNB provides relay RLC Channel configuration to Remote UE and intermediate Relay UEs using Uu RRC message based on SUI message received from each relay UEs. | Alt1: gNB implementation to ensure QoS, and provides PC5 RLC Channel configuration to the last relay UE, and the last relay UE forwards to intermediate Relay UEs using PC5-RRC message. Alt2: E2E QoS is split into per-hop QoS, RLC Channel configuration can be derived using existing U2U relay solution. QoS splitting among PC5 hops can reuses the solution for L3 based multi-hop U2N relay defined by SA2. |
| SUI message | SUI message from each relay UE needs to be enhanced to associate with multi-hop relay path | SUI from last relay UE needs to be enhanced to report multi-hop relay path information |
| Security | E2E AS security between Remote UE and gNB | E2E AS security between Remote UE and gNB |
| Local ID assignment | gNB assigns local ID and provides to each UE using RRC message | gNB assign local ID and provide to last Relay UE, and last Relay UE forwards to intermediate Relay UE using PC5-RRC message. |
| SI/Paging forwarding | Last Relay UE sends SI/Paging to the Remote UE via per-hop PC5-RRC message | Last Relay UE sends SI/Paging to the Remote UE via per-hop PC5-RRC message |

- RAN2 condisers the solutions listed in Table 2 to support basic functionalities for multi-hop U2N relay.

## 2.3 Applicable scenarios by Approach 1 and Approach 2

During the post meeting email discisson of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG) [2], the topology scenarios were discussed and summarized in following Table 2. Some descriptions are added into the table to show the motivation of the scenarios.

Table 2: Supported scenarios by Approach 1 and Approach 2.

| Supported topology scenarios |  | Approach 1 | Approach 2 |
| --- | --- | --- | --- |
| Figure 2.5A-1 | For the case of different RSCs for intermediate Relay UE’s service and relay service | No | Yes |
| Figure 2.5A-2 | Independent paths | Yes | Yes |
| Figure 2.5A-3 | For the case of different RSCs supported in different last relay UEs which are in the same cell | No | Yes |
| Figure 2.5B-1 | Different Remote UEs have the same RSC service and share the same PC5 link. | Yes | Yes |
| Figure 2.5B-2 | Independent paths | No | Yes |
| Figure 2.5B-3 | For the case of different RSCs supported in different last relay UEs which are in the same cell. | No | Yes |
| Figure 2.5C-1 | For the case of different RSCs reqeusted by the same Remote UE, multi-path multi-hop U2N relay, out of scope | Out of scope | Out of scope |
| Figure 2.5C-2 | For the case of different RSCs reqeusted by the same Remote UE, multi-path multi-hop U2N relay, out of scope. | Out of scope | Out of scope |
| Figure 2.5D-1 | For the case of different RSCs supported in different last relay UEs which are in different cells. | No | Yes |
| Figure 2.5D-2 | For the case of different RSCs supported in different last relay UEs which are in the different cell. | No | Yes |
| Figure 2.5D-3 | For the case of different RSCs reqeusted by the same Remote UE, multi-path multi-hop U2N relay, out of scope. | Out of scope | Out of scope |

For these scenarios of Figure 2.5C-1/ Figure 2.5C-2/ Figure 2.5D-3, since one Remote UE request different indirect path, this is multi-path U2N relay, should be out of Rel-19 scope.

Observation 1: Scenarios of figure 2.5C-1/ Figure 2.5C-2/ Figure 2.5D-3, since one Remote UE request different indirect path, this is multi-path U2N relay, should be out of Rel-19 scope.

For these scenarios of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2, Approach 1 requires intermediate Relay UE to be in CONNECTED state and have Uu RRC connection towards the Remote UE’s serving gNB, then cannot support scenarios of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2 since those requires multi-path support in intermediate Relay UE. While in Approach 2, relay path does not require Uu RRC connection in intermediate Relay UE, the different paths can be treated independently. Then it is possible to support these scenarios.

Observation 2: Approach 1 cannot support the scenarios of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2 in the table of , then different RSCs connection can be supported between different Remote UEs or between intermediate Relay UE’s service and Remote UE’s service.

Observation 3: Approach 2 can support the of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2, then different RSCs connection can be supported between different Remote UEs or between intermediate Relay UE’s service and Remote UE’s service.

- RAN2 confirms the applicable scenarios supported by Approach 1 and Approach 2 respectively.

- Scenarios of figure 2.5C-1/ Figure 2.5C-2/ Figure 2.5D-3, since one Remote UE request different indirect path, this is multi-path U2N relay, should be out of Rel-19 scope.

- Approach 1 cannot support the scenarios of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2 in the table of , then different RSCs connection can be supported between different Remote UEs or between intermediate Relay UE’s service and Remote UE’s service.

- Approach 2 can support the of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2, then different RSCs connection can be supported between different Remote UEs or between intermediate Relay UE’s service and Remote UE’s service.

## 2.4 Additional issues to be addressed by Approach 1 and Approach 2

For Approach 1, there are some additional issues to be addressed.

Issue 1: How to ensure the intermediate Relay UE are in the same cell as the Remote UE

When the multi-hop relay path is established, the intermediate Relay UE may already have connection with a cell which is different from the cell of the Remote UE. RRC_CONNECTED intermediate Relay UE path switching from its serving cell to the relay cell may be needed, and new event needs to be introduced for this kind of path switching. The performance of intermediate Relay UE’s service could be degraded due to switching to multi-hop relay path from direct path. There could also be issue that the relay cell may not support intermediate Relay UE’s service, so that how to handle this is still a problem. Alternatively, a solution is needed to exclude this case, e.g. enhance discovery message to filter the candidate relay UEs which only includes those ones in the same cell, but this way will reduce the possibility of multi-hop U2N relay operation.

Issue 2: How to handle intermediate Relay’s Uu RRC connection when any hop is failure

When Uu link is failure, or any parent link is failure, it is unclear how to handle intermediate relay UE’s Uu RRC connection. This issue should be discussed.

Issue 3: How to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing

Since the intermediate Relay UE’s own service and relay service share the same Uu RRC connection, then shen the intermediate Relay UE has its own service, there could be procedure collision on how to handle RRC connection for intermediate Relay UE service and the Remote UE service. E.g,

- If HO or path switching is triggered for intermediate Relay UE’s own service, how to handle the relay connection and service.

- If path switching is triggered for relay service, how to handle intermediate Relay UE’s own service

Issue 4: How to identify and exclude the unsupported scenarios

As shown in clause 2.3, there are many scneairos Approach 1 cannot support, then solutions are needed to identify and exclude these scenarios.

- For Approach 1, the following additional issues need to be addressed.

Issue 1: How to ensure the intermediate Relay UE are in the same cell as the Remote UE.

Issue 2: How to handle intermediate Relay’s Uu RRC connection when any hop is failure.

Issue 3: How to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing

Issue 4: How to identify and exclude the unsupported topology scenarios.

For Approach 2, if it is assumed that gNB assign local ID for the Remote UE, there could potential local ID assignment collision if the Remote UEs share the same PC5 link when the Remote UEs’ serving gNB are different gNBs.

- For Approach 2, the following additional issue need to be addressed.

- Potential local ID assignment collision if the Remote UEs share the same PC5 link when the Remote UEs’ serving gNB are different gNBs.

## 2.4 Evaluation of Approach 1 and Approach 2

As discussed in clause 2.2/2.3/2.4, when evaluating Approach 1 and Approach 2, at least the follow aspect should be considered.

- Standard efforts to support basic functionality of multi-hop U2N relay operations.

- Performance, e.g. signalling overhead, access latency for multi-hop U2N relay operation.

- Standard efforts to address additional issues introduced by each approach

- Applicable topology scenarios.

- When evaluating Approach 1 and Approach 2, at least the follow aspect should be considered.

- Standard efforts to support basic functionality of multi-hop U2N relay operations.

- Performance, e.g. signalling overhead, access latency for multi-hop U2N relay operation.

- Standard efforts to address additional issues introduced by each approach

- Applicable topology scenarios.

# Conclusion

- The following basic functions should be supported in Multi-hop U2N relay.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- RAN2 condisers the solutions listed in Table 2 to support basic functionalities for multi-hop U2N relay.

- RAN2 confirms the applicable scenarios supported by Approach 1 and Approach 2 respectively.

- Scenarios of figure 2.5C-1/ Figure 2.5C-2/ Figure 2.5D-3, since one Remote UE request different indirect path, this is multi-path U2N relay, should be out of Rel-19 scope.

- Approach 1 cannot support the scenarios of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2 in the table of , then different RSCs connection can be supported between different Remote UEs or between intermediate Relay UE’s service and Remote UE’s service.

- Approach 2 can support the of Figure 2.5A-1/ Figure 2.5A-3/ Figure 2.5B-2/ Figure 2.5B-3/ Figure 2.5D-1/ Figure 2.5D-2, then different RSCs connection can be supported between different Remote

- For Approach 1, the following additional issues need to be addressed.

Issue 1: How to ensure the intermediate Relay UE are in the same cell as the Remote UE.

Issue 2: How to handle intermediate Relay’s Uu RRC connection when any hop is failure.

Issue 3: How to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing

Issue 4: How to identify and exclude the unsupported topology scenarios.

- For Approach 2, the following additional issue need to be addressed.

- Potential local ID assignment collision if the Remote UEs share the same PC5 link when the Remote UEs’ serving gNB are different gNBs.

- When evaluating Approach 1 and Approach 2, at least the follow aspect should be considered.

    - Standard efforts to support basic functionality of multi-hop U2N relay operations.

    - Performance, e.g. signalling overhead, access latency for multi-hop U2N relay operation.

    - Standard efforts to address additional issues introduced by each approach

    - Applicable topology scenarios.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet

[2]. [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

[3]. [Post127][402][Relay] Multi-hop relay control plane (InterDigital)