3GPP TSG-RAN WG2 Meeting #128	R2-2410750

Orlando, US, 18 – 22 Nov, 2024

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on SMTC

Agenda Item:	7.4.2

Document for:	Discussion and decision

Introduction

There are still some remaining open issues on SMTC in the NTN topic:

Issue 1: On PDD reporting

<From RAN2#127bis> Observation 1 UEs configured with PDD report may (simultaneously) generate and send a PDD report upon satellite switch with resynchronization. - Sequans does not necessarily agree with Obs 1 and thinks the maximum PDD is 0.3 ms in the extreme case (of 100km cell size). Sequans thinks we might still need to discuss whether there is a need for the NW to update the SMTC, e.g. due to the feeder link, but still wonders if this is a real issue - LG also thinks there is no issue, smtc4 provides enough duration to perform neighbour cell measurement after completion of soft satellite switch. Can come back in the next meeting

Issue 2: On SSB shifting for soft satellite switch

<From RAN2#126> R2-2405757 Summary of [AT116][301][NR NTN Enh] SMTC impacts for soft satellite switch Sequans Communications discussion Rel-18 NR_NTN_enh-Core Noted

Issue 3: On optionality of PCI list in smtc4 (common for R17 and R18)

<From RAN2#127bis> R2-2409090 Corrections to NR NTN (R17) Huawei, HiSilicon, CMCC CR Rel-17 38.331 17.10.0 5075 - F NR_NTN_solutions-Core - QC and Google think the first change is not needed. LG agrees with HW intention and we should specify something in the field description. MTK supports the intention and thinks we can fix the working - Regarding the second change Sequans thinks this is what we already agreed one year ago, without reflecting this in the spec so in case a reference to this should be added to the cover page - MTK would like to have more time to check this - HW thinks we can leave the first issue open or leave some room for implementation Revised in R2-2409238 to consider at least the second change

In this contribution, we provide our views on the above issues.

Discussion

## PDD reporting

In the last meeting, there was discussion on whether the UE needs to send the PDD report update the SMTC upon satellite switch [1].

In R18 discussion we only considered the case where the serving gNB applies satellite switch with resync, we never enhanced the cases where neighbour cells are using satellite switch with resync. Therefore, during satellite switch, in most of the cases the neighbour satellites are not changed and the propagation delay between the UE and neighbour cells remain the same. As a result, the PDD change upon satellite switch with re-sync is only caused by serving satellite change. The maximum PDD change upon satellite switch can be evaluated by the maximum propagation delay difference in the serving cell coverage.

In order to evaluate the PDD change, we assume that the minimum elevation angle is 30 degrees, the diameter of cell coverage is 100km and the satellite altitude is 600km.

We can see that the maximum PDD change upon satellite switch is about:

(1200-1157) km/(3*10^2 km/ms)≈0.14ms for 50km coverage

(1200-1114) km/(3*10^2 km/ms)≈0.29ms for 100km coverage

(1200-601) km/(3*10^2 km/ms)≈2ms for 1000km coverage

Figure 1: PDD change during satellite switch with resync (assuming 100km diameter for cell coverage)

PropDelayDiffReportConfig-r17 ::= SEQUENCE {

threshPropDelayDiff-r17           ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, spare5,

spare4, spare3, spare2, spare1}                OPTIONAL,   -- Need M

neighCellInfoList-r17             SEQUENCE (SIZE (1..maxCellNTN-r17)) OF NeighbourCellInfo-r17         OPTIONAL    -- Need M

}

The minimum value of threshold for one-way service link propagation delay difference is 0.5ms, which is sufficient for 50km coverage and 100km coverage, but PDD report may be triggered for large coverage like 1000km. On the other hand, the above calculation is based on the extreme case (largest coverage, largest propagation delay difference), the network implementation can configure a larger value for threshPropDelayDiff (e.g. 3ms) to avoid triggering PDD reporting even in those extreme cases.

Consequently, there is no need to mandate SMTC update during satellite switch with resync, and this conclusion applies to both hard switch and soft switch.

Observation 1: Network can configure suitable threshPropDelayDiff to avoid PDD report during satellite switch with resync.

Proposal 1: No further enhancement to PDD reporting during satellite switch with resync.

## SSB shifting for soft satellite switch

In the RAN2#126 meeting, RAN2 discussed the SMTC issue for the soft satellite switch based on contribution [2], and no agreement was made.

The issue can be briefly described as follows:

Based on RAN1 LS, one condition for soft satellite switching without PCI change is “non-colliding SSB with same PCI at UE side”. To manipulate this SSB timing difference, the SSB transmission pattern of source satellite and target satellite should be different, which is illustrated by the parameter “ssb-TimeOffset-r18”.

Three solutions were proposed:

- Shifted cell timing: the SSB transmission is on the same SFN/subframes for source and target satellite, but there is a timing difference (i.e. SFTD) between source and target satellite.

- Shifted SSB pattern: the SSB transmission is on the different SFN/subframes for source and target satellite.

- Temporarily shifted SSB pattern: the target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

For Option 1, source cell and target cell are served by the same gNB, but gNB needs to maintain different timing for them, and gNB needs to generate two SS bursts in the same SFN. gNB complexity is increased.

For Option 2, the time location (i.e. SFN/subframes) of SSB is changed, after t-Start, and continues after t-Service. In the example above, the SSB in SFN1 is shifted to SSB in SFN2. Therefore, the gNB needs to reconfigure the SMTC for intra-frequency measurements at least to cover the SSBs for the serving cell. Consider the large value range of ssb-TimeOffset-r18 (which could be at most 159subframes), this shift cannot be managed by configuring a proper SMTC window without reconfiguration. RRCReconfiguration is not expected in satellite switch with resync, otherwise the gain of satellite switch with resync is decreased.

For option 3, the time location (i.e. SFN/subframes) of SSB is not changed after t-service. Only the propagation delay is changed. UEs in RRC_IDLE/RRC_INACTIVE can adjust the SMTC based on the serving propagation delay difference, and Option 3 has no impacts on them. For the RRC_CONNECTED UEs, the SMTC configuration depends on the PDD. According to the above analysis of PDD, we think the PDD change is not large. And if the network configures the duration of SMTC to be 5ms, we think the range of PDD change is acceptable and UE can still detect the SSB of neighbour cell.

Proposal 2: The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

## PCI list in SMTC4

The last issue is on the PCI list in SMTC4, which was discussed in RAN2#127bis based on contribution [3].

In R17 NTN, smtc4list-r17 was added to MeasObjectNR and SIB2/SIB4, which contains a list of SSB-MTC4-r17, and each SSB-MTC4-r17 includes a PCI list (optional, need M) and an offset.

Combined with the legacy SSB-MTC (smtc1 in MeasObjectNR, smtc in SIB2/SIB4), the network is able to configure at most 4 SMTCs for the UE to perform measurements.

The smtc4list in MO is “OPTIONAL, -- Need R” rather than “Need M”, which means: each time the MO is modified, smtc4list should be included (otherwise it is released) for measurements.

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,    -- Need R

Considering the following guidance in 38.331 Clause 6.1.3, all the 4 SMTCs in the smtc4list should be full configuration whenever the MO is modified.

Upon reception of a list not using ToAddModList and ToReleaseList structure, the UE shall delete all entries of the list currently in the UE configuration before applying the received list and shall consider each entry as newly created. This applies also to lists whose size is extended (i.e. with a second list structure in the ASN.1 comprising additional entries), unless otherwise specified. This implies that Need M should not be used for fields in the entries of these lists; if used, UE will handle such fields equivalent to a Need R.

Therefore, the following “OPTIONAL, -- Need M” is useless, the SMTCs can never be maintained because delta configuration does not apply in this case.

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

However, during the discussion, it was raised by some companies whether PCI list can be optional for UEs RRC_IDLE/RRC_INACTIVE. In our understanding, RAN4 requirement is defined based on SMTC, it would be necessary to check whether the PCI list is assumed mandatory in RAN4 spec. The following is excepted from clause 9.2C.5.1 of 38.133. When determining “number of LEO satellites”, the UE uses the PCIs corresponding to the SMTC to find the corresponding satellite. However, there is no clear guidance from RAN4 whether PCI list must be mandatory, and whether RAN4 requirements can also apply based on blind detection of the PCIs based on the satellite ephemeris of the frequency.

if SMTCs partially overlap with each other, - Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier; - Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier; where - NLEO,i is the number of LEO satellites to be measured within i-th SMTC, - NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC, - NSMTC,overlap is the number of SMTCs that partially overlap with each other.

According to 38.331 clause 5.5.2 (which applies to both RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE), the UE shall set up the SMTC window for cells indicated in the PCI list. Consequently, if the PCI is not in the list, UE shall not set up the SMTC window. However, the existing descriptions clearly mentions “MeasObjectNR” (specific to UEs in RRC_CONNECTED), which means it is unclear whether UEs in RRC_IDLE/RRC_INACTIVE follows the same principle.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

Therefore, we think two options can be considered:

- Option 1: Similar as the changes in R2-2409090, clarify in the field description of pci-List of SSB-MTC4 that “The network shall always configure this field when SSB-MTC4-r17 is configured”

- Option 2: Clarify the UE behaviour of blind detection in 38.331 clause 5.5.2, for instance:

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition. If pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, it is up to UE implementation to detect neighbour cells associated with the SMTC, based on the ephemeris information in SIB19.

Proposal 3: RAN2 to determine whether PCI list in SMTC4 is mandatory, and choose from the following:

- Option 1: Clarify in the field description of pci-List of SSB-MTC4 that “The network shall always configure this field when SSB-MTC4-r17 is configured”;

- Option 2: Add the following description in 38.331 clause 5.5.2: “If pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, it is up to UE implementation to detect neighbour cells associated with the SMTC, based on the ephemeris information in SIB19.”

RAN2 also needs to decide whether the change is made from R17 or R18.

Conclusion

In this contribution, we discuss the remaining issues on SMTC and propose the following:

Observation 1: Network can configure suitable threshPropDelayDiff to avoid PDD report during satellite switch with resync.

Proposal 1: No further enhancement to PDD reporting during satellite switch with resync.

Proposal 2: The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

Proposal 3: RAN2 to determine whether PCI list in SMTC4 is mandatory, and choose from the following:

- Option 1: Clarify in the field description of pci-List of SSB-MTC4 that “The network shall always configure this field when SSB-MTC4-r17 is configured”;

- Option 2: Add the following description in 38.331 clause 5.5.2: “If pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, it is up to UE implementation to detect neighbour cells associated with the SMTC, based on the ephemeris information in SIB19.”

References

- R2-2409187, Remaining open issues, Ericsson

- R2-2405680, Soft satellite switch SSB time offset and SMTC impact, Sequans Communications

- R2-2409090, Corrections to NR NTN (R17), Huawei, HiSilicon, CMCC