3GPP TSG-RAN WG2 Meeting #128                                                             R2-2410766

Orlando, USA, Nov. 18th – 22rd, 2024

Agenda Item	:	8.2.3

Title	:	Discussion on paging procedure for ambient IoT

Source	:	Google Ireland Limited

Work Item	:	FS_Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we propose that RAN2 confirms what and how many device/group IDs can be included in a paging message. We also discuss the need for a paging ID to avoid duplicate response and whether it is needed to explicitly indicate random access type in a paging message.

# 2	Discussion

In RAN2 #126 meeting, RAN2 agreed to study the following cases for a paging message. We propose to confirm that, for Rel-19 ambient IoT, a paging message contains onl a single device ID, or only a group ID, or no IDs. It is FFS whether multiple device IDs are supported in a paging message. Moreover, it should be discussed whether a paging message can contain a mix of device IDs and group IDs.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Proposal 1:  RAN2 to confirm that a paging message contains only a single device ID, or only a group ID, or no IDs. FFS whether multiple device IDs or a mix of device IDs and group IDs are supported in a paging message.

In the RAN2 #127b meeting, RAN2 also has an agreement aimed to avoid duplicate response due to re-paging. A simple solution is to include a paging ID in the paging message. For example, the size of the paging ID can be 4 bits or 8 bits.

Agreements - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Proposal 2:  A reader includes a paging ID in a paging message to avoid duplicate responses. The paging ID can be 4 bits or 8 bits.

RAN2 agreed that a device determines the random access type (2step, 3step, CFRA) from the paging message.   It is FFS whether it is explicitly or implicitly.

For 2-step and 3-step CBRA, the reader could implicitly indicate the access type in Msg2 (e.g. the presence of a command). A device could also tell whether a CBRA or CFRA should be initiated as for CFRA the reader will allocate D2R resources for the device to transmit D2R data. From the observation above, there may be no need to explicitly indicate random access type in a paging message.

Proposal 3:  There is no need to explicitly indicate random access type in a paging message.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1:  RAN2 to confirm that a paging message contains only a single device ID, or only a group ID, or no IDs. FFS whether multiple device IDs or a mix of device IDs and group IDs are supported in a paging message.

Proposal 2:  A reader includes a paging ID in a paging message to avoid duplicate response. For example, the paging ID can be 4 bits or 8 bits.

Proposal 3:  There is no need to explicitly indicate random access type in a paging message.