3GPP TSG RAN WG2 Meeting #128  												                                          R2-2410776

Orlando, USA, November 18th – 22nd, 2024

Agenda item:		8.5.3

Title:						Discussion on On-demand SIB1 for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was revised in RAN#105 [1], where the specification of the studied case 2 for on-demand SIB1 was agreed:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

The agreements made so far for the WI are [2]:

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

This contribution focuses on some of the relevant aspects concerning on-demand SIB1 transmissions for UEs in idle/inactive mode, necessary for a successful feature.

# Initial Cell Search with On-Demand SIB1

During the study phase there was not enough discussion on the impact of on-demand SIB1 on Initial Cell Search. This section discusses how a suitable on-demand SIB1 design is of paramount importance to avoid performance issues on the Initial Cell Search.

The Initial Cell Search is performed when the UE has no information about the RF environment, for example, if the UE is turned on for the first time or if the UE enters a different country. The order of search is UE implementation specific and there are thousands of potential frequencies (GSCNs) to be scanned. As soon as the UE finds one suitable cell, information about other cells can be obtained from the SI. Due to the implementation specific order, there is no guarantee that the UE will find a Cell A before finding an NES cell. The more popular on-demand SIB1 gets, the more often UEs will find NES cells first. In the absence of SIB1 the UE has little information about that cell. The UE cannot even evaluate whether the cell is suitable or not, and the UE does not know if the cell is from its own operator.

- Observation 1: Often, in Initial Cell Search, UEs will find NES cells before a Cell A. In the absence of SIB1 the UE has little information about the NES cell. The UE cannot even evaluate whether the cell is suitable or not and the UE does not know if the cell is from its own operator.

In order to keep UE implementation simple and for better Initial Cell Search performance, we need to consider how to improve the UE’s knowledge about the NES cell. The first level of information is identification of NES status directly from NES cell. The UE needs to know at least whether one of the following 3 states apply: 1) the NES cell is currently broadcasting SIB1 (NES mode de-activated), 2) the NES cell is not broadcasting SIB1 at all (it cannot be obtained even on-demand, which is already possible - see ssb-SubcarrierOffset in TS 38.331 [4]) or 3) the cell is currently on NES-mode and SIB1 can be obtained on-demand (at a later stage when the UE obtains WUS configuration from Cell A). Only with this identification directly on the NES cell the UE implementation can decide whether it is important or not to keep information about this cell cached. It is important to note that the reserved values of ssb-SubcarrierOffset provide exactly the intended behaviour for OD-SIB1. Namely, legacy UEs will bar the cell and OD-SIB1 UEs will immediately identify the cell as an OD-SIB1 cell. And all of that with SSB only transmission.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

In light of the above agreement of the previous meeting [2], the signaling of identification and barring should be

- Observation 2: The reserved values of ssb-SubcarrierOffset provide exactly the intended behaviour for OD-SIB1. Namely, legacy UEs will bar the cell and OD-SIB1 UEs will immediately identify the cell as an OD-SIB1 cell. And all of that with SSB only transmission.

- Proposal 1: The reserved values of ssb-SubcarrierOffset  (K_ssb=30 on FR1, K_ssb=14 on FR2) are used to unequivocally identify the cell as operating on OD-SIB1 and being barred for legacy UEs.

In current operation with regularly broadcast SIB1 the UE can immediately evaluate if a cell is from its own PLMN or at least some allowable PLMN. Once on-demand SIB1 is broadly used, this evaluation from the UE will be hindered, unless a solution is designed to provide the UE with the respective information. For this purpose of broadcasting the PLMN, a list of PLMNs or a list of NPNs would be prohibitive and negate the energy saving benefits of omitting SIB1 on the NES cell. Instead, a compressed information can be sent for the sake of identifying (at least) the main PLMN which that cell belongs to. Namely, the cell can apply a hashing function to its PLMN ID and broadcast this PLMN hash. Then, the UE can also apply the same hashing function to its PLMN and compare the hash result with what is broadcast by the NES cell. In this way, the same Initial Cell Search performance of today can be kept: the UE can promptly evaluate whether a cell belongs to its PLMN or not. This will enable to attain similar performance on Initial Cell Search as today, while saving energy.

- Proposal 2: The NES cell broadcasts a PLMN hash, so that the UE can identify if the NES cell belongs to the desired PLMN or not.

# Reduction of UL WUS Configuration Overhead

In the case chosen for standardization (case 2), a Cell A will carry UL WUS configuration for many NES cells. Therefore, even a moderate content per NES cell (e.g. 30 octets) can quickly grow large to an extent of spending too much energy for the broadcast on Cell A.  This can make the whole feature less attractive and negate the energy saving on NES cells by adding energy expenditure on the Cell A. For this reason, it is very important to reduce the UL WUS configuration overhead.

Observation 3: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

To obtain the UL WUS configuration of NES cells the UE obtains SIB1 of Cell A. This enables the following approach for reducing the overhead drastically: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

Proposal 3: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

- Observation 1: Often, in Initial Cell Search, UEs will find NES cells before a Cell A. In the absence of SIB1 the UE has little information about the NES cell. The UE cannot even evaluate whether the cell is suitable or not and the UE does not even know if the cell is from its own operator.

- Observation 2: The reserved values of ssb-SubcarrierOffset provide exactly the intended behaviour for OD-SIB1. Namely, legacy UEs will bar the cell and OD-SIB1 UEs will immediately identify the cell as a OD-SIB1 cell. And all of that with SSB only transmission.

Observation 3: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

- Proposal 1: The reserved values of ssb-SubcarrierOffset  (K_ssb=30 on FR1, K_ssb=14 on FR2) are used to unequivocally identify the cell as operating on OD-SIB1 and being barred for legacy UEs.

- Proposal 2: The NES cell broadcasts a PLMN hash, so that the UE can identify if the NES cell belongs to the desired PLMN or not.

Proposal 3: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

# References

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #105	 Melbourne, Australia, September 2024.

- R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, 3GPP TSG RAN WG2 #127bis, Hefei, China, October 2024.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka, Japan, May 20th – 24th, 2024.

- TS 38.331, “	NR; Radio Resource Control (RRC); Protocol specification” ,  v18.2.0, July 2024.