3GPP TSG-RAN WG2 Meeting #128		                               R2-2410777

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:		8.7.5

Source:			MediaTek Inc.

Title:				RLC AM enhancements with small packet delay budget.

Document:			Discussion and decision.

### 1	Introduction

This document discusses RLC AM enhancement according to the fourth objective in R19 XR WID [1].

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

### 2	Discussion

## 2.1	Avoiding unnecessary RLC AM retransmissions

In previous several meetings, we finally reach agreements that “combined” approach is adopted to avoid unnecessary RLC AM retransmissions, 2 FFS points still need to be discussed.

- TX side stops transmissions of an outdated SDU

- RX side abandons the SDU based on a local timer

- Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

- FFS if some C-PDU handling is needed to avoid C-PDU discard

- FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

We use one example to explain why a control PDU is still needed in the “combined” approach.

- At t = 0ms, tx side send SN = 0 to rx side, and rx side successful receive SDU with SN = 0, corresponding state variable is updated as well.

- At t = 1ms, tx side send SN = 1 to rx side, but this time, rx side does not successfully receive the SDU (SN = 1), no action.

- At t = 2ms, tx side send SN = 2 to rx side, and rx side successful receive SDU with SN = 2, corresponding state variable is updated and a local timer (something similar to t-Reassembly).

- After a period of time, no other SDUs can be sent or successfully received by rx side, and those SDUs or PDUs become outdated. From rx side point of view, there is no other way to aware of that some SDUs or PDUs already become outdated unless some indication is sent from tx side.

Proposal 1: An indication from Tx to Rx is needed.

Proposal 2: The indication (e.g., RLC control PDU) at least should include information related to SN, which already being outdated.

## 2.2	Timely retransmissions for XR traffic in RLC layer

In last meeting (RAN2 #127bis), we exclude enhanced status report mechanism from the candidate list to achieve timely retransmission, two mechanisms are left for further discussion.

- Autonomous retransmission

- Enhanced polling

For autonomous retransmission (i.e., without feedback from rx side), we have concern about the capacity loss. For example: assume each RLC SDU size is 1500 bytes and SN is 18 bits. Transmitter side already send SDU 1~7 to receiver side. Assume SDU 1, 3, 5, 7 are correctly received. The STATUS PDU is at most 31bytes. With the feedback, transmitter side only need to retransmit SDU 0, 2, 4, 6. (4*1500 = 6000 bytes). However, if there is no feedback, transmitter side needs to retransmit SDU 1~7 again, which cost (7*1500 = 10500bytes).

Capacity consumption of retransmission with feedback in this example is (6000 + 31) bytes which is less than retransmission without feedback (15000 bytes). It is clearly that retransmission based on feedback has less negative impact on capacity.

| SDU SN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 (ACK_SN) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Correct received or not | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (~3bytes) |

For enhanced polling, serval parameters can be discussed, t-PollRetransmit, pollPDU, pollByte. We can just introduce smaller value to achieve timely retransmission with less spec impact and capacity loss.

| Parameters | Current minimum value |
| --- | --- |
| t-PollRetransmit | 5ms |
| pollPDU | 4PDUs |
| pollByte | 1kBytes |

Proposal 3: RAN2 to adopt enhanced polling mechanism (with smaller values in 38.331) to achieve timely retransmission.

### 3	Conclusion

This document promulgated the following observation and proposals:

Avoiding unnecessary RLC AM retransmissions

Proposal 1: An indication from Tx to Rx is needed.

Proposal 2: The indication (e.g., RLC control PDU) at least should include information related to SN, which already being outdated.

Timely retransmissions for XR traffic in RLC layer

Proposal 3: RAN2 to adopt enhanced polling mechanism (with smaller values in 38.331) to achieve timely retransmission.

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).