3GPP TSG-RAN WG2 Meeting #128	R2-2410828

Orlando, USA, Nov. 18th~22nd , 2024

Agenda item:	8.13.4

Source:	Samsung

Title:	Initial considerations on service continuity

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

According to WID, the following cases can be addressed for service continuity with different priorities:

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

In this contribution, we will share our initial thinking on this aspects.

Discussion

Scenario  A

This scenario can take the legacy path switch from indirect path to direct path as the starting point. The main difference is that after the path switch, the gNB may need to release intermediate relay UEs and last relay UEs. The question is who will trigger this release. Two options can be considered,

- Option 1: gNB triggers the release

In this option, since gNB knows (if Approach 1 for CP is applied) whether the intermediate relay UE serves other remote UEs or not, gNB can trigger RRC release procedure to the intermediate relay UEs without directly/indirectly served remote UEs.

- Option 2: intermediate relay UE determines by itself

In this option, the intermediate relay UE needs to be aware of each directly/indirectly served remote UEs. If there is no such remote UE, the intermediate relay UE can determine to release the PC5 link with its parent node.

Proposal A-1: RAN2 is kindly asked to agree that the legacy path switch from indirect to direct path is the baseline for Rel19 scenario A.

Proposal A-2: RAN2 is kindly asked to discuss how to release the intermediate relay UEs after path switch by considering the following two options:

- Option 1: gNB triggers the release

- Option 2: intermediate relay UE determines by itself

Scenario B

For scenario, two sub-scenarios can be considered:

- Sub-scenario B-1:  the last relay UE in multi-hop indirect path is different from the relay UE in the single-hop indirect path

- Sub-scenario B-2:  the last relay UE in multi-hop indirect path is same as the relay UE in the single-hop indirect path

Apparently, the sub-scenario B-1 should be supported. Whether the sub-scenario B-2 is supported or need needs RAN2’s clarification.

Proposal B-1: RAN2 is kindly asked to clarify whether to support the scenario that the last relay UE in multi-hop indirect path is same as the relay UE in the single-hop indirect path.

To support the sub-scenario B-1, the legacy path switch from indirect to indirect path can be the starting point. After the path switch, the release of intermediate relay UEs in the source path can reuse the design for scenario A.

Proposal B-2: RAN2 is kindly asked to agree to use the legacy path switch from indirect to indirect path as the starting point for scenario B.

Scenario C

This scenario can take the legacy path switch from the direct to indirect path as the starting point. One issue is that when the remote UE reports the measurement results, whether additional information is needed or not for the multi-hop path selection at the gNB side.

Proposal C-1: RAN2 is kindly asked to agree to use the legacy path switch from the direct to indirect path as the starting point for scenario C

Proposal C-2: RAN2 is kindly asked to discuss whether additional information is needed from the remote UE side to help the multi-hop path selection at gNB side.

Scenario D

Similar to scenario D, RAN2 needs clarify whether to support the scenario that the relay UE in single-hop indirect path is same as the last relay UE in multi-hop indirect path.

Proposal D-1: RAN2 is kindly asked to clarify whether to support the scenario that the relay UE in single-hop indirect path is same as the last relay UE in multi-hop indirect path.

For procedure, the legacy procedure can be used as the starting point.

Proposal D-2: RAN2 is kindly asked to agree to use the legacy path switch from indirect to indirect path as the starting point for scenario D.

Conclusion

In this contribution, we address service continuity for four different scenarios, and proposed:

Scenario  A

Proposal A-1: RAN2 is kindly asked to agree that the legacy path switch from indirect to direct path is the baseline for Rel19 scenario A.

Proposal A-2: RAN2 is kindly asked to discuss how to release the intermediate relay UEs after path switch by considering the following two options:

- Option 1: gNB triggers the release

- Option 2: intermediate relay UE determines by itself

Scenario B

Proposal B-1: RAN2 is kindly asked to clarify whether to support the scenario that the last relay UE in multi-hop indirect path is same as the relay UE in the single-hop indirect path.

Proposal B-2: RAN2 is kindly asked to agree to use the legacy path switch from indirect to indirect path as the starting point for scenario B.

Scenario C

Proposal C-1: RAN2 is kindly asked to agree to use the legacy path switch from the direct to indirect path as the starting point for scenario C

Proposal C-2: RAN2 is kindly asked to discuss whether additional information is needed from the remote UE side to help the multi-hop path selection at gNB side.

Scenario D

Proposal D-1: RAN2 is kindly asked to clarify whether to support the scenario that the relay UE in single-hop indirect path is same as the last relay UE in multi-hop indirect path.

Proposal D-2: RAN2 is kindly asked to agree to use the legacy path switch from indirect to indirect path as the starting point for scenario D.

References