3GPP TSG-RAN WG2 #128	R2-2410841

Orlando, US, 18th – 22nd, November 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on LP-WUS for IDLE/INACTIVE state

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

In this contribution, we provide our views on issues to be resolved on LP-WUS for IDLE/INACTIVE state.

- Discussion

  - Mobility of UEs in LP-WUS monitoring

RAN3 made agreements to introduce assistance information of CN assigned subgroup ID in the NG paging message and the F1 paging message.

RAN3#125-bis [1] The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups. The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

Based on above agreements, the gNB can perform CN paging towards IDLE/INACTIVE UEs monitoring LP-WUS.

On the other hand, for Xn paging, RAN3 discussion is pending on RAN2. In other words, RAN3 discussed how to realize an RAN paging from a neighboring cell when a UE monitoring LP-WUS in IDLE/INACTIVE state moves from the coverage of the serving cell to the neighboring cell, but they did not reach consensus. The reason is that when a UE monitoring LP-WUS in the IDLE/INACTIVE state leaves the serving cell and enters the coverage of a neighbor cell, there is no agreement on the UE behavior for such case. The purpose of this section is to clarify the behavior when the LP-WUS UE moves to another cell.

Figure 1: UE is monitoring LP-WUS from the serving cell.

Now we assume a situation, as shown in Figure 1, where a UE in IDLE/INACTIVE state has already fulfilled the LP-WUS entry condition and is monitoring the LP-WUS in its LR, while its MR is in deep sleep mode. Let’s assume the circle named LP-WUS coverage means the geographical area where the exit condition for LR measurement is not fulfilled.

Then the UE may leave the coverage of the serving cell and enter the proximity of the neighbor cell like Figure 2.

Figure 2: UE enters the proximity of the neighbor cell but keeps monitoring LP-WUS.

However, since the neighboring cell does not have the knowledge on the LP-WUS configuration of the serving cell for the UE, it cannot transmit LP-WUS to wake up the UE, and therefore cannot perform paging to the UE.

Figure 3: UE enters the proximity of the neighbor cell and wakes up.

Instead, the UE may fulfill the LP-WUS exit condition configured by the serving cell to wake-up the MR.

We think that RAN3 can proceed with the Xn paging discussion once RAN2 agrees on the above mobility behaviors. The UE behavior after wake-up in the proximity of the neighbor cell can be further discussed depending on demand from RAN3 discussion.

Proposal 1.	RAN2 understands that a UE monitoring LP-WUS in IDLE/INACTIVE state does not move to any other cells unless the UE wakes up either due to reception of LP-WUS or due to fulfilling the exit condition.

Proposal 2.	RAN2 can further discuss the UE behavior after wake-up in the proximity of the neighbor cell depending on demand from RAN3 discussion.

  - Subgrouping during maintaining emergency PDU session

When a UE having an emergency PDU session transits to IDLE/INACTIVE state and then satisfies the LP-WUS entry condition, if the currently agreed subgrouping is applied, it may delay the UE wake-up and this may cause a delay for emergency calls. From operator's point of view, adding delay to the emergency PDU session is not desirable, so methods to minimize delay can be discussed.

CN assigned subgrouping

In Rel-17 PEI, it is specified that a UE with an emergency PDU session shall not report a Paging Subgrouping Support Indication to the CN.

TS 23.501 [2] #### 5.4.12.2 Core Network Assistance for PEIPS (omit) When the UE has an active emergency PDU Session: - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

For LP-WUS subgrouping, it is straightforward to go with a similar solution, i.e., UEs with an emergency PDU session do not report the UE capability for the CN subgrouping ID feature. This is up to SA2 anyway.

Observation 1.	It is up to SA2 to discuss whether the UE prevents from reporting UE capability for CN subgrouping ID feature when the UE has an emergency PDU session.

UE_ID based subgrouping

In Rel-17 PEI, it was agreed not to disable UE ID-based subgrouping even when the UE has an emergency PDU session.

It is necessary to discuss whether it is reasonable not to specify anything for LP-WUS as well.

Proposal 3.	Discuss whether any solution is needed to minimize an additional delay to page a UE caused by subgrouping for LP-WUS.

- Summary and proposal

Proposal 1.	RAN2 understands that a UE monitoring LP-WUS in IDLE/INACTIVE state does not move to any other cells unless the UE wakes up either due to reception of LP-WUS or due to fulfilling the exit condition.

Proposal 2.	RAN2 can further discuss the UE behavior after wake-up in the proximity of the neighbor cell depending on demand from RAN3 discussion.

Observation 1.	It is up to SA2 to discuss whether the UE prevents from reporting UE capability for CN subgrouping ID feature when the UE has an emergency PDU session.

Proposal 3.	Discuss whether any solution is needed to minimize an additional delay to page a UE caused by subgrouping for LP-WUS.

- References

[1]	(draft) RAN3 #125-bis Meeting Report.

[2]	TS 23.501, “System architecture for the 5G System (5GS),” V19.1.0.