3GPP TSG-RAN WG2 #128	R2-2410847

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.10.5

Source:	Ericsson

Title:	On Rel.18 leftovers

Document for:	Discussion

# 1	Introduction

Rel-19 WID supports the following leftover topics in RAN2.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

- RACH optimization for SDT

- MHI Enhancement for SCG Deactivation/Activation

- MRO for MR-DC SCG failure

In this paper, we discuss these leftovers.

# 2	Discussion

## 2.1	RA report logging in case of SDT

During the last 3GPP meetings, the following was agreed for the topic of SDT:

From RAN2#127-bis

Agreements

- For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

- RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

From RAN2#127:

- UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

As captured above, in RAN2#127-bis meeting, it was agreed that the UE includes the buffered uplink data volume information and measured downlink RSRP information in the RA report for the failed SDT scenarios.

It should be noted that the UE performs the evaluation by comparing the buffered uplink data volume on radio bearers with SDT enabled and the downlink RSRP against the thresholds on data volume and RSRP respectively, as mentioned in TS-38.321-

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

However, MO-SDT can be of two types, RA-SDT and CG-SDT. Thus, for both CG-SDT and RA-SDT evaluation, the UE utilizes the same data volume threshold and RSRP threshold as reference. Hence, in case the UE fails to initiate SDT procedure and accesses the network using common RACH resources, it remains unclear if the problem was on CG-SDT or RA-SDT or both. In particular, the following scenarios can be envisaged according to the MAC spec (see TS 38.321, Section 5.27.1):

- The UE evaluated only RA-SDT (e.g. not configured with CG-SDT), but either the RSRP-based condition or the data volume-based condition was not fulfilled. The UE then performs RA on common RACH resources.

- The UE evaluated only CG-SDT (i.e. RA-SDT not configured), but either the RSRP-based condition or the data volume-based condition or the CG-based condition (e.g. TA for CG-SDT not valid, next CG occasion too far away in time, etc) was not fulfilled. The UE then performs RA on common RACH resources.

- Both the RSRP-based condition and the data volume-based condition are fulfilled. The UE first evaluated CG-SDT (according to the MAC spec procedure, the UE shall first evaluate CG-SDT), but a CG-based condition (e.g. TA for CG-SDT not valid, next CG occasion too far away in time, etc) was not fulfilled. The UE performs RA-SDT.

From the RA information, the network would not be able to distinguish the following scenarios:

  - The UE evaluates RA-SDT and/or CG-SDT, but if falls back to common RACH resources due to the RSRP-based condition or the data volume-based condition or CG-based conditions not being fulfilled.

  - The RSRP-based condition and the data volume-based condition are fulfilled, the UE evaluates CG-SDT, but it falls back to RA-SDT, due to CG-based conditions not being fulfilled.

Knowing whether the UE initially tried to perform RA-SDT or CG-SDT is useful information that the network can use to optimize RA resources. For example, if many UEs fail to initiate CG-SDT and fall back to RA-SDT, the RA-SDT resources dimensioned for the cell may become overloaded. Additionally, in this case both the RSRP-based condition and the data volume-based condition is fulfilled (otherwise the UE would not select RA-SDT resources). Hence, the network will not be able to know that the selection of the RA-SDT was because issues with the CG-SDT, and not because only RA-SDT was configured/evaluated.
Similar problem arises, if many UEs fail to initiate CG-SDT or RA-SDT and fallback to common RACH resources.

A UE may fail to initiate CG-SDT and perform RA-SDT causing overload of RA-SDT resource utilization, or it may fail to initiate CG-SDT or RA-SDT and perform RA in common RACH resources.

Hence, we propose the following:

If the UE evaluated to perform SDT, and common RACH resources were used (i.e. conditions for initiating RA-SDT or CG-SDT were not fulfilled), the UE indicates in the RA-Report whether RA-SDT and/or CG-SDT initiation was evaluated.

If the UE evaluated to perform CG-SDT, and RA-SDT resources were used (i.e. conditions for initiating CG-SDT not fulfilled), the UE indicates in the RA-Report that CG-SDT initiation was evaluated.

Related to CG-SDT, we believe that it is also beneficial to include the reason related to why the CG-SDT failed, which according to TS 38.321 are TA for CG-SDT not valid, SSB RSRP measured level smaller than the CG-SDT SSB RSRP threshold, i.e., smaller than cg-SDT-RSRP-ThresholdSSB, or the next occasion for CG-SDT is too far away in time. This would help the gNB to improve CG-SDT configuration, thereby limiting the overload issues in common RA or RA-SDT resources.

RAN2 to further discuss whether to include in the RA-Report information related to the cause of CG-SDT evaluation failure prior to the random access procedure.

Related to the SDT failure cause, RAN2 agreed in RAN2#127 to include in the RA-Report the failure causes. However, in RAN2#127-bis it was agreed to not further pursue other failure causes occurring during the SDT procedure (such as BFD/BFR during the SDT) not explicitly captured in existing specification as causes for which the UE declares SDT failure. 
According to TS 38.300, an initiated SDT can be unsuccessful in the following cases:

- Upon cell re-selection,

- Expiry of the SDT failure detection timer,

- A MAC entity reaching a configured maximum PRACH preamble transmission threshold.

- An RLC entity reaching a configured maximum retransmission threshold,

- Integrity check failure while SDT procedure is ongoing,

- Expiry of SDT-specific timing alignment timer or configuredGrantTimer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

In our view, all the above failure cases should be logged, except case f). This is because case f) is only applicable to the case of CG-SDT, i.e. no RA involvement. For the case of CG-SDT evaluation failure, followed by a fallback to RA, we believe that it is beneficial to include the failure cause (see our Proposal 3); but for the case of CG-SDT failure after CG-SDT being initiated, it is not an applicable scenario for the troubleshooting of RA issues.

UE includes failure cause of SDT for the following scenarios:

  - Unsuccessfully completed upon cell re-selection,

  - Expiry of the SDT failure detection timer,

  - A MAC entity reaching a configured maximum PRACH preamble transmission threshold

  - An RLC entity reaching a configured maximum retransmission threshold,

  - Integrity check failure while SDT procedure is ongoing.

# Conclusion

In the previous sections we made the following observations:

Observation 1	From the RA information, the network would not be able to distinguish the following scenarios:

a.	The UE evaluates RA-SDT and/or CG-SDT, but if falls back to common RACH resources due to the RSRP-based condition or the data volume-based condition or CG-based conditions not being fulfilled.

b.	The RSRP-based condition and the data volume-based condition are fulfilled, the UE evaluates CG-SDT, but it falls back to RA-SDT, due to CG-based conditions not being fulfilled.

Observation 2	A UE may fail to initiate CG-SDT and perform RA-SDT causing overload of RA-SDT resource utilization, or it may fail to initiate CG-SDT or RA-SDT and perform RA in common RACH resources.

Based on the discussion in the previous sections we propose the following:

Proposal 1	If the UE evaluated to perform SDT, and common RACH resources were used (i.e. conditions for initiating RA-SDT or CG-SDT were not fulfilled), the UE indicates in the RA-Report whether RA-SDT and/or CG-SDT initiation was evaluated.

Proposal 2	If the UE evaluated to perform CG-SDT, and RA-SDT resources were used (i.e. conditions for initiating CG-SDT not fulfilled), the UE indicates in the RA-Report that CG-SDT initiation was evaluated.

Proposal 3	RAN2 to further discuss whether to include in the RA-Report information related to the cause of CG-SDT evaluation failure prior to the random access procedure.

Proposal 4	UE includes failure cause of SDT for the following scenarios:

a.	Unsuccessfully completed upon cell re-selection,

b.	Expiry of the SDT failure detection timer,

c.	A MAC entity reaching a configured maximum PRACH preamble transmission threshold

d.	An RLC entity reaching a configured maximum retransmission threshold,

e.	Integrity check failure while SDT procedure is ongoing.