3GPP TSG-RAN WG2 Meeting #128	R2-2410863

Orlando, FL, USA, November 18th – 22nd, 2024

Agenda Item:	8.9.2

Source:	Ericsson

Title:	Support for store and forward in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusions from SA2 study report [1], in this contribution, we discuss the objectives for store and forward of the work item [2]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

# 2	Discussion

## 2.1	S&F indication signalling details

In RAN2#127bis, it was agreed that a dynamic S&F indication will be supported to inform UEs of the operation mode of the serving cell.

- The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

- RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

- An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed.

With this newly introduced indication, the UE is made aware of the current operational mode through the indication(s) provided in System Information. Consequently, the UE should be aware of the following:

Whether the serving satellite supports S&F connectivity.

The status of the serving satellite’s feeder.

After last RAN2 agreements, the UE is aware of the S&F status of the serving cell through a so-called dynamic indication. In our understanding, this means that, upon a certain time, the operation mode may change, i.e., a cell in S&F may switch to real-time (non-S&F mode) and vice versa. There potential impacts of such changes were initially discussed in RAN2#127 since the operation of RRC_IDLE/INACTIVE and RRC_CONNECTED UEs may need specification enhancement. Thus, we consider that an essential first step is that RAN2 formally agrees to consider this scenario.

RAN2 considers the transition scenario where the operation mode (S&F/non-S&F) changes during a cell serving time.

In some cases, operation changes (S&F/non-S&F) may coincide with the stop serving time of a cell (t-Service). This happens when the satellite gains or loses feeder link connectivity when it is close to the minimum elevation angle (from UE perspective). However, there are other scenarios, for instance, Earth-moving cells, where this is not possible, and it is relevant for the UE to know when a change of operation may happen. Given this could affect not only the application function but also RAN protocols, such as release or reselection, the UE should be informed of the remaining duration of the current feeder link status.

UE is informed of the remaining time of the satellite’s current S&F mode via System Information broadcast.

## 2.2	Neighbour and upcoming satellites

In their study [1], SA2 has concluded that the MME can provide, during the Attach or Tracking Area Update (TAU) procedure, a list of Satellite IDs over which the UE may later exchange signalling and data. A UE can determine a satellite’s ID from information provided by the eNB in SIB31/SIB32/SIB33. This mechanism allows the UE to identify which satellite(s) may have the necessary context to fulfill its connection requirements such as Attach or TAU. This information is beneficial for the UE implementation as (together with the information in SIB32) it helps to plan optimal network access times, thereby saving energy by avoiding unsuccessful connection attempts.

In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

In NAS, the satellite ID list is based on the values present in system information and broadcast in RAN.

Similarly, it would be beneficial for the UE in AS level to have access to the operation mode of not only its serving satellite but also neighbouring and upcoming satellites. For neighbouring satellite, this information may be included in SIB33, along with their respective satellite ID. For upcoming satellites, this information could draw from synergies with the discontinuous coverage feature and be included in SIB32. This additional information can enhance the efficiency of various AS procedures, such as cell re-selection, paging, or PSM, and facilitate power-saving mechanisms up to UE implementation. For example, the UE that is moving and is located close to the cell border may prioritize the reselection to a cell which does not support S&F architecture.

UE is informed of the S&F status of neighbour and upcoming satellites.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

Observation 2	In NAS, the satellite ID list is based on the values present in system information and broadcast in RAN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 considers the transition scenario where the operation mode (S&F/non-S&F) changes during a cell serving time.

Proposal 2	UE is informed of the remaining time of the satellite’s current S&F mode via System Information broadcast.

Proposal 3	UE is informed of the S&F status of neighbour and upcoming satellites.

# 4	References

TR 3GPP TR 23.700-29 v19.0.0

RP-241624 - WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3