3GPP TSG-RAN WG2 Meeting #128	R2-2410889

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.6.4

Source:	Samsung

Title:	Support of Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, RAN2 made following agreements to support the Conditional intra-CU LTM.

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, we will further address remaining issues to support the conditional intra-CU LTM in detail.

Discussion

Rel-18 LTM needs the explicit signalling (i.e. LTM cell switch MAC CE) to trigger the cell switch based on the measurement report by either L3 or L1 manner. In Rel-19, the event triggered measurement reporting would be introduced but it also cannot guarantee the signalling weakness of cell switch message i.e. UE might fail to receive the LTM cell switch signalling because of weak channel qualities. In order to enhance the reliability of the handover, the conditional LTM which UE autonomous triggering to execute the cell switch based on the Layer 1 measurement results is needed. From our understanding, conditional LTM could be applied for MCG and SCG but as a starting point only MCG conditional LTM is introduced in Rel-19, see the objective in [1]. So, in the following we only focus on the MCG conditional LTM for intra-CU.

According to the WID [1], support of the subsequent conditional LTM for intra-CU is the scope of Rel-19 so we want to first consider how to support subsequent conditional LTM for intra-CU. Each source cell and LTM candidate cells need to prepare the multiple execution conditions considering when each candidate cell becomes the source cell. In CHO, one target cell is associated with one or two condition(s) using the measId which provides the execution condition. RRC structure of CHO and LTM could be good reference and the CLTM structure could include the association between the L1 measurement resource configuration and the execution conditions.

Issue 1:  Conditional LTM Preparation

From our understanding, the first question to support subsequent conditional LTM is which node generates the execution condition for the subsequent conditional LTM. For the legacy CHO, the source CU generates the execution conditions based on the measurement configuration, for the subsequent CPAC, the source SN generates the initial execution condition and the candidate SN generates the following execution conditions. This is because the execution condition is L3 based measurement. However LTM is based on the beam measurement (L1). Therefore, we think the entity to generate the execution condition for the conditional LTM should be the DU (i.e. source cell and candidate cells provides its own execution condition and DU generate the execution condition for the conditional LTM) as RAN2 agreement above. In addition, each candidate cell needs to generate the execution conditions when the candidate cell becomes the source cell and provide it to the source CU in LTM candidate cell configuration to support the subsequent conditional intra-CU LTM. It means the execution conditions for the initial conditional LTM and the execution conditions for subsequent conditional LTM should be separately provided to the UE. For example, the execution conditions for the initial conditional LTM is provided in the serving cell configuration (outside of candidate cell configuration) but the execution conditions for the subsequent conditional LTM is provided in the candidate cell configuration.

Proposal 1: To support subsequent conditional LTM, the execution conditions provided to UE includes the execution conditions for initial conditional LTM and the execution conditions for subsequent conditional LTM.

Proposal 2: The execution conditions for the initial conditional LTM and the execution conditions for subsequent conditional LTM should be separately provided to the UE.

- For example, the execution conditions for the initial conditional LTM is provided in the serving cell configuration (outside of candidate cell configuration) but the execution conditions for the subsequent conditional LTM is provided in the candidate cell configuration.

Plus, we think it is network implementation to provide candidate cell configuration(s) for the same candidate cell by both the LTM and the conditional LTM. Source CU request if the both conditional LTM and the NW triggered LTM is available to the target candidate cell in the LTM preparation phase. Then each candidate cell response the LTM request including the candidate cell configuration with availability of either/both LTM and conditional LTM. In that case, we assume that the candidate cell configuration is not different between the LTM and the conditional LTM. Then, RAN2 need to determine the detail UE operation if the NW send the LTM cell switch MAC CE for the candidate cell which is also configured for the conditional LTM. We assume that one candidate cell configuration could be provided for the conditional LTM (e.g. LTM-CandidateId = 1) but NW could trigger the cell switch for this cell. Then, this candidate cell configuration also contains the parameters for the conditional LTM (e.g. execution condition for subsequent conditional LTM). How to handle this parameters for the conditional LTM is considered.

  - Option 1: UE keep the all the stored RRC configuration for the candidate cell and apply it after LTM cell switch which is triggered by the LTM cell switch MAC CE (no specification impact)

  - Option 2: UE autonomously remove the parameters for the conditional LTM after LTM cell switch which is triggered by the LTM cell switch MAC CE

The main concerned scenario is coexistence of intra-CU candidate cells for the conditional LTM and inter-CU candidate cells for LTM. If UE goes to the candidate cell in a different CU, I think this conditional LTM related configuration including the L1 measurement for this cell may not be useful. So, the simple solution is to remove the conditional LTM related configuration if inter-CU LTM is indicated by the network. We also agree that both option could work i.e. UE can keep the configuration in this case as well. RAN2 need to consider how to optimize the UE operation in case that the coexistence of intra-CU conditional LTM and inter-CU LTM is supported.

Another following question is that how RAN2 define the maximum number of candidate LTM cell configurations in case that the NW configures the LTM and the conditional LTM simultaneously. From Rel-18 to Rel-19, RAN2 stick to keep the 8 as the maximum number for the LTM candidate cell configurations so we think it is reasonable to keep the 8 as the maximum number of candidate LTM cell configurations even the NW configures the LTM and the conditional LTM simultaneously.

Proposal 3: RAN2 confirm that coexistence of intra-CU conditional LTM and inter-CU LTM is supported, and NW can provide the LTM configuration and the conditional LTM configuration for the same candidate cell.

Proposal 4: RAN2 consider how to handle the parameters for the conditional LTM (e.g. execution condition for subsequent conditional LTM) after LTM cell switch which is triggered by the LTM cell switch MAC CE.

Proposal 5: The maximum number of candidate LTM cell configurations is not increased (i.e. 8 is the maximum number) in case that the NW configures the LTM and the conditional LTM simultaneously.

We think the legacy LTM-CSI-ResourceConfig IE could be reused (or slightly updated) for the Rel-19 conditional LTM resource configuration. It means the new IE for the conditional LTM CSI measurement resource configuration is not needed. Then how to provide the execution condition and the associated L1 measurement resources are needed. We think ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration) could be used for providing this configuration (execution condition and the associated L1 measurement resources). For the explicit target cell configuration, the legacy LTM candidate cell configuration could be referred in the conditional LTM configuration which provides the associated information and execution conditions.

Proposal 6: LTM measurement resource configuration for the conditional LTM is provided in LTM-CSI-ResourceConfig-r18.

Proposal 7: Initial execution conditions with the associated L1 measurement resources and index of the candidate cell configuration to be applied are provided in ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration).

Issue 2:  Support of RACH-less CLTM

During the RAN2#127bis meeting, RAN2 already made agreement to support the RACH-less conditional LTM for intra- CU case. However, there are a lot of remaining issues to support RACH-less conditional LTM e.g. how the TA is transfer to the UE, which beam is used for the current beam, TCI state after the conditional LTM and support of DG for the first UL transmission.

For beam selection of the current beam (source cell) for event evaluation (execution condition), we think the conditional LTM is slightly different with the Rel-19 LTM principle. For the event triggered measurement report, the current beam to evaluate the pre-defined event is the NW indicated beam which is based on the L1 measurement report. Based on this indicated TCI state, UE measure the beams for the candidate cells and evaluate the event, then UE report the measurement results if any event condition is met. However, it is not convinced that the indicated TCI state is always the best beam in case of the conditional LTM because the measurement is doing after UE receiving the RRC configuration including the conditional LTM. Plus, the indicated beam from NW would be based on the measurement report from UE but it is possible that UE could not report measurement report and trigger the conditional LTM directly based on the best beam measurement if it is allowed. So, it is more reasonable that UE select the current beam to evaluate the execution condition. This current beam could be the best beam in the source cell by UE measurement or any beam indicated by the TCI state activation MAC CE).

- Option 1: Current beam of the source cell could be the indicated beam (TCI state).

- Option 2: UE could select the best beam for the source cell to evaluate the execution condition. (or use the any beam which is activated by Candidate cell TCI states activation/deactivation MAC CE)

Proposal 8: UE could select the best beam for the source cell to evaluate the execution condition i.e. use the best beam or any beam which is activated by TCI states activation/deactivation MAC CE.

Another big issue for RACH-less conditional LTM is how the calculated TA from the candidate cell is provided to the UE, Rel-18 early TA RACH preamble transmission is assumed. From our understanding, there are two options:

- Option 1: Source cell provides the TA using the new MAC CE

  - Each candidate cell (DU) provide the calculated TA to the source cell (DU)

  - Source cell (DU) send the new MAC CE including the single (or multiple) TA and candidate cell ID(s)

  - Pros:

    - No interruption to receive the TA for the candidate cell

    - Less specification impact from F1 interface (i.e. there are already F1 procedures to share the TA value between source DU and candidate DU)

- Option 2: Each candidate cell provides the TA using RAR (new RAR-like message)

  - The TA value is provided directly from the candidate cell after TA is calculated

  - Require the interruption time (e.g. timer to receive the RAR)

  - Pros:

    - Less delay to transfer the available TA to the UE

    - No need the TA transfer procedure between source DU and candidate DU

Proposal 9: RAN2 to discuss how the calculated TA from the candidate cell is provided to the UE, either by

- Option 1: Source cell provides the TA using the new MAC CE

- Option 2: Each candidate cell provides the TA using RAR (new RAR-like message)

Similar as current beam selection for the conditional LTM, there are one more beam related issue that which TCI state is used after cell switch by the conditional LTM. In Rel-18 LTM cell switch command MAC CE, the TCI state information is mandatory so UE should use this TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). However, there are no signaling to indicate the activated TCI state for data tx/rx in the target cell after cell switch for the conditional LTM. We think it is reasonable that UE uses the evaluated best beam (or any beam which met the execution condition) of the target cell used for determining the conditional LTM (based on the measurement). Reason for this is that NW has no clear information which beam triggers the conditional LTM, so the early indicated TCI state would not be the best beam.

Proposal 10: UE use the any beam (or best beam) which meet the execution condition and triggering the conditional LTM is used as TCI state at the target cell after the cell switch.

We think the CG based RACH-less conditional LTM is the baseline as RAN2 already agreed in the last meeting. In Rel-18, RACH-less LTM introduced both the CG-based first UL transmission and DG-based first UL transmission. For the DG-based approach, the NW has to know when the UE monitor the PDCCH to reduce the resource waste, otherwise, network send the PDDCH even there are no UE in the cell area. In the conditional LTM, NW has no idea on the conditional LTM i.e. which cell UE triggered conditional LTM, when UE triggers the conditional LTM.

Proposal 11: For RACH-less conditional LTM, DG-based first UL transmission on target cell is not supported.

Issue 3:  Security aspects

Since the scope of Rel-19 CLTM is limited to intra-CU, no security key change issues are foreseen. It means that the master key change is not supported and this restriction could be added in the specification similar as Rel-18 LTM.

Proposal 12: For intra-CU conditional LTM, master key change is not supported.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: To support subsequent conditional LTM, the execution conditions provided to UE includes the execution conditions for initial conditional LTM and the execution conditions for subsequent conditional LTM.

Proposal 2: The execution conditions for the initial conditional LTM and the execution conditions for subsequent conditional LTM should be separately provided to the UE.

- For example, the execution conditions for the initial conditional LTM is provided in the serving cell configuration (outside of candidate cell configuration) but the execution conditions for the subsequent conditional LTM is provided in the candidate cell configuration.

Proposal 3: RAN2 confirm that coexistence of intra-CU conditional LTM and inter-CU LTM is supported, and NW can provide the LTM configuration and the conditional LTM configuration for the same candidate cell.

Proposal 4: RAN2 consider how to handle the parameters for the conditional LTM (e.g. execution condition for subsequent conditional LTM) after LTM cell switch which is triggered by the LTM cell switch MAC CE.

Proposal 5: The maximum number of candidate LTM cell configurations is not increased (i.e. 8 is the maximum number) in case that the NW configures the LTM and the conditional LTM simultaneously.

Proposal 6: LTM measurement resource configuration for the conditional LTM is provided in LTM-CSI-ResourceConfig-r18.

Proposal 7: Initial execution conditions with the associated L1 measurement resources and index of the candidate cell configuration to be applied are provided in ltm-CSI-ReportConfig or same level of configuration (i.e. in the source cell configuration).

Proposal 8: UE could select the best beam for the source cell to evaluate the execution condition i.e. use the best beam or any beam which is activated by Candidate cell TCI states activation/deactivation MAC CE.

Proposal 9: RAN2 to discuss how the calculated TA from the candidate cell is provided to the UE, either by

- Option 1: Source cell provides the TA using the new MAC CE

- Option 2: Each candidate cell provides the TA using RAR (new RAR-like message)

Proposal 10: UE use the any beam (or best beam) which meet the execution condition and triggering the conditional LTM is used as TCI state at the target cell after the cell switch.

Proposal 11: For RACH-less conditional LTM, DG-based first UL transmission on target cell is not supported.

Proposal 12: For intra-CU conditional LTM, master key change is not supported.

References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.