3GPP TSG-RAN WG2 Meeting #128	R2-2410916

Orlando, USA, Nov.  18th – 22nd, 2024

Source: 	Session Chair (Ericsson)

Title:	Report from session on maintenance and SON/MDT

Agenda item:	9.6

[AT128][600] Organizational – Maintenance and SON/MDT (Ericsson)

Scope:

    - Share plans for the meeting and list of ongoing email discussions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs and agreed CRs for discussion

Intended outcome:

    - General information sharing about the sessions

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except V2X and Sidelink WIs and Positioning WIs, which are addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Missing reference

R2-2410559	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-15	36.331	15.23.0	5077	-	F	TEI12

R2-2410560	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-16	36.331	16.17.0	5078	-	A	TEI12

R2-2410561	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-17	36.331	17.10.0	5079	-	A	TEI12

R2-2410562	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-18	36.331	18.3.1	5080	-	A	TEI12

-	Qualcomm thinks its not really needed to add this reference. Samsung (spec rapp) think this could be considered editorial but wanted to let other companies have a chance to properly review this hence this was submitted as a stand-alone CR. Lenovo think this is not editorial since the offset would not be applied to NR, but anyway it is optional so they are OK with the change. Qualcomm is not OK with this change as it is too late now, it should have been added in Rel-15 if at all, now its too late.

-	Huawei is OK with this but from Rel-18 if so. Lenovo think its optional and it clear from the capability spec.

We do this change from Rel-18 with magic sentence. CR to be produced offline.

[AT128][604][Maint] Reference to 38.304 for Qoffsettemp (Nokia)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR in R2-2411015 (Nokia)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411015	Addition of reference to 38.304 for Qoffsettemp handling	Nokia

Agreed

### 4.1.0	In-principle agreed CRs

Revised

R2-2410891	Correction to NR Beam Reporting in LTE RRC	Apple, ZTE Corporation	CR	Rel-18	36.331	18.3.1	5069	1	F	NR_newRAT-Core	R2-2409946

Agreed unseen in R2-2411016 with adding TEI18 and add “annex G” as impacted sections. Update revision number in Annex G itself.

Old revisions

R2-2409946	Correction to NR Beam Reporting in LTE RRC	Apple, ZTE Corporation	CR	Rel-18	36.331	18.3.1	5069	-	F	NR_newRAT-Core

### 4.1.1	Other

System information acquisition

R2-2409878	Minor Corrections on TS36.331	vivo	CR	Rel-18	36.331	18.3.1	5066	-	A	LTE_eMTC5-Core

R2-2409879	Minor Corrections on TS36.331	vivo	CR	Rel-17	36.331	17.10.0	5067	-	A	LTE_eMTC5-Core

R2-2409880	Minor Corrections on TS36.331	vivo	CR	Rel-16	36.331	16.17.0	5068	-	F	LTE_eMTC5-Core

-	Qualcomm think this is agreeable

R2-2410026	Correction on SI Reception in IoT-NTN	vivo	CR	Rel-17	36.331	17.10.0	5070	-	F	LTE_NBIOT_eMTC_NTN

R2-2410027	Correction on SI Reception in IoT-NTN	vivo	CR	Rel-18	36.331	18.3.1	5071	-	A	LTE_NBIOT_eMTC_NTN

-	Qualcomm wants to add SIB33 (for R18).

CRs need to be updated and merged. This is to be handled offline.

[AT128][605][Maint] Corrections on SI Reception (vivo)

Scope:

    - Produce agreeable CRs. Discuss if we can find a more future proof way of writing this part of the spec so we don’t need to update all the time in the future.

Intended outcome:

    - Agreed CRs in R2-2411017, R2-2411018, R2-2411019 (vivo)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411017	Correction on SI Reception in RRC CONNECTED	vivo

R2-2411018	Correction on SI Reception in RRC CONNECTED	vivo

R2-2411204	Correction on SI Reception in RRC CONNECTED	vivo

-	Qualcomm want to make the wording more generic and refer to other parts of the spec, rather than referring to explicit SIBs. Samsung prefers this explicit proposal from vivo.

All three are agreed

highProrityAccess

R2-2410547	Alignment of highProrityAccess cause in RRC Establishment and Resume	Peraton Labs	CR	Rel-18	36.300	18.3.0	1413	-	F	NR_newRAT-Core

-	Peraton labs indicate that there is a type “ng-gNB”.

-	Lenovo wonders why we need to address this now (in R18)? No one complained so far, why is this critical now? Peraton indicates that there is a misalignment. Huawei think that the current first paragraph of the impacte section is clear enough since it refers to 38.300. Peraton think that 36.300 must capture ng-eNB behaviour. Samsung think that the high prio access was there since Rel-15 so why do we need this change now?

-	Lenovo think that this is stage-2 so there might be misalignment, but there is stage-2 so how come it is critical?

Agreed unseen in R2-2411035, but add impact analysis and TEI18 WI code. And “gNB” typo is fixed.

Old revisions

R2-2411019	Correction on SI Reception in RRC CONNECTED	vivo

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

Max nrof CCs

R2-2409814	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.300	15.18.0	0926	-	F	NR_newRAT-Core

R2-2409815	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.300	16.17.0	0927	-	A	NR_newRAT-Core

R2-2409816	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.10.0	0928	-	A	NR_newRAT-Core

R2-2409817	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-18	38.300	18.3.0	0929	-	A	NR_newRAT-Core

4 are agreed

BFD-BFR on PSCell

R2-2410451	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-15	38.300	15.18.0	0933	-	F	NR_newRAT-Core

R2-2410452	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-16	38.300	16.17.0	0934	-	A	NR_newRAT-Core

R2-2410453	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-17	38.300	17.10.0	0935	-	A	NR_newRAT-Core

R2-2410454	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-18	38.300	18.3.0	0936	-	A	NR_newRAT-Core

-	Huawei think there may be other places that need to be changed if we do this change. ZTE agrees that there are more than this place that would be impacted if we do this change. Ericsson think.

Postponed

#### 5.1.1.0	In-principle agreed CRs

Updated

R2-2410900	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO

R2-2410901	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO

R2-2410902	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO

All agreed

Old revisions

R2-2410419	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-16	38.300	16.17.0	0906	1	F	NR_newRAT-Core	R2-2408234

R2-2410420	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-17	38.300	17.10.0	0907	1	A	NR_newRAT-Core	R2-2408235

R2-2410421	Gap requirement for CSI-RS based measurements and inter-RAT measurements (R18)	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-18	38.300	18.3.0	0906	2	A	NR_newRAT-Core	R2-2408234	Withdrawn

R2-2410431	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-18	38.300	18.3.0	0908	1	A	NR_newRAT-Core	R2-2408236

#### 5.1.1.1	Other

Misc

R2-2409997	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-15	38.331	15.27.0	5127	-	F	NR_newRAT-Core

R2-2409998	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-16	38.331	16.18.0	5128	-	F	NR_newRAT-Core, TEI16

To be handled in a post meeting email [POST128][004]

### 5.1.3	Control Plane corrections

deprioritisationTimer

R2-2410563	Correction on deprioritisationTimer	Nokia, MediaTek Inc., LG Electronics Inc.	CR	Rel-15	38.331	15.27.0	5167	-	F	NR_newRAT-Core

R2-2410564	Correction on deprioritisationTimer	Nokia, MediaTek Inc., LG Electronics Inc.	CR	Rel-16	38.331	16.18.0	5168	-	A	NR_newRAT-Core

R2-2410565	Correction on deprioritisationTimer	Nokia	CR	Rel-17	38.331	17.10.0	5169	-	A	NR_newRAT-Core

R2-2410566	Correction on deprioritisationTimer	Nokia	CR	Rel-18	38.331	18.3.0	5170	-	A	NR_newRAT-Core

-	Lenovo think none of these changes are needed. vivo think that this is aligning to 304 and are fine with the intention but the last change (timer table) needs updating as the stopping should be in reposne to PLMN selection. Lenovo disagrees with all of vivos comments, 304 talks about deleting a timer so it doesn’t matter about stopping of the timer. Qualcomm supports all changes, in particular the change to the timer-table and want the CR from R18. Samsung agrees with the intention but think the timer table doesn’t need updating.

Try to produce agreeable CR for R18 NR offline.

[AT128][606][Maint] Example (Nokia)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2411020

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411020	Correction on deprioritisationTimer	Nokia

Agreed unseen in R2-2411039, but the RAN-tick box should be ticked.

#### 5.1.3.0	In-principle agreed CRs

R2-2409642	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, CMCC, Ericsson	CR	Rel-15	38.331	15.27.0	4893	3	F	NR_newRAT-Core	R2-2409285

R2-2409643	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-16	38.331	16.18.0	4894	3	A	NR_newRAT-Core	R2-2409286

R2-2409644	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-17	38.331	17.10.0	4895	3	A	NR_newRAT-Core	R2-2409287

R2-2409645	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-18	38.331	18.3.0	4896	3	A	NR_newRAT-Core	R2-2409288

R2-2410203	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-16	38.331	16.18.0	5089	1	F	NR_newRAT-Core, TEI16	R2-2409292

R2-2410204	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-17	38.331	17.10.0	5090	1	A	NR_newRAT-Core, TEI16	R2-2409293

R2-2410205	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-18	38.331	18.3.0	5091	1	A	NR_newRAT-Core, TEI16	R2-2409294

R2-2410829	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-15	38.306	15.26.0	1170	1	F	NR_newRAT-Core	R2-2408469

R2-2410830	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.18.0	1171	1	A	NR_newRAT-Core	R2-2408470

R2-2410831	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	17.10.0	1172	1	A	NR_newRAT-Core	R2-2408471

R2-2410832	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.306	18.3.0	1173	1	A	NR_newRAT-Core	R2-2408472

All above are agreed

Updated

R2-2411101	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	4	F	NR_UE_pow_sav-Core	R2-2410885

R2-2411102	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	4	A	NR_UE_pow_sav-Core	R2-2410886

R2-2411103	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	4	A	NR_UE_pow_sav-Core	R2-2410887

Agreed unseen in R2-2411021, R2-2411022, R2-2411023. Fix the revision number

Old revisions

R2-2410885	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	4	F	NR_UE_pow_sav-Core	R2-2410058

R2-2410886	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	4	A	NR_UE_pow_sav-Core	R2-2410059

R2-2410887	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	4	A	NR_UE_pow_sav-Core	R2-2410060

R2-2410058	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	3	F	NR_UE_pow_sav-Core	R2-2409282	Revised

R2-2410059	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	3	A	NR_UE_pow_sav-Core	R2-2409283	Revised

R2-2410060	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	3	A	NR_UE_pow_sav-Core	R2-2409284	Revised

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

SI request

R2-2409928	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-16	38.331	16.18.0	5122	-	F	TEI16

R2-2409929	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-17	38.331	17.10.0	5123	-	A	TEI16

R2-2409930	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-18	38.331	18.3.0	5124	-	A	TEI16

-	Apple thinks that even if the UE re-reads SIB1 the UE stores SIB1, “stored” is not an issue. MediaTek think the current text is clear and think there is no critical issue. Lenovo is not convinced. Samsung think the requirement is clear on SIB1 acquisition and this “stored” can be kept. Vivo agrees with the previous speakers. ZTE think this CR causes issues.

Not pursued

Protection of RRC messages

R2-2410074	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5141	-	F	NR_IAB-Core, NR_SON_MDT-Core

R2-2410075	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5142	-	A	NR_IAB-Core, NR_SON_MDT-Core

R2-2410076	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5143	-	A	NR_IAB-Core, NR_SON_MDT-Core

-	Huawei think this is informative text and can be merged to the rapp CR. Ericsson agrees and they have tried themselves earlier.

To be merged in the RRC rapporteurs CR

smtc

R2-2410356	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.27.0	5155	-	F	NR_newRAT-Core

R2-2410357	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5156	-	A	NR_newRAT-Core

R2-2410358	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5157	-	A	NR_newRAT-Core

R2-2410359	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5158	-	A	NR_newRAT-Core

-	MediaTek wonders if the purpose is to clarify or to add new requirements? ZTE think its just clarifying. Nokia think the intention is OK but think its strange to refer to an IE-name, instead they can refer to fields of a certain IE-type. ZTE points out in LTE RRC they use this IE name in procedural text. Ericsson think that if we refer to the IE name there may be an ambiguity? ZTE think that the IE-name is only used for smtc1 and smtc. Ericsson think that in this case we should spell out smtc1 and smtc and prefer to refer to field names.

Produce CRs saying “in the smtc or smtc1 configuration” instead.

[AT128][607][Maint] Corrections to smtc configuration (ZTE)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411024, R2-2411025, R2-2411026, R2-2411027

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

-	ZTE reports after offline that companies are fine to use the original proposed corrections, which is more maintainable.

R2-2411024	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.27.0	5155	1	F	NR_newRAT-Core

R2-2411025	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5156	1	A	NR_newRAT-Core

R2-2411026	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5157	1	A	NR_newRAT-Core

R2-2411027	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5158	1	A	NR_newRAT-Core

All 4 are agreed

PNI-NPN

R2-2410746	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-16	38.331	16.18.0	5182	-	F	NG_RAN_PRN-Core

R2-2410747	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-17	38.331	17.10.0	5183	-	A	NG_RAN_PRN-Core

R2-2410748	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-18	38.331	18.3.0	5184	-	A	NG_RAN_PRN-Core

-	Nokia think this is editorial and want to do it from Rel-18 only. Ericsson think the current spec is fine. China Telecom want to leave the spec as is. CATT think its important to align with SA specs. ZTE think the current definition is correct. Samsung agrees with ZTE, in RAN2 specs there is no issue to keep current definitions. Qualcomm think that the last change should be using field names instead. MediaTek think that for PLMN its difficult to refer to field names. Ericsson think that the last change causes “or.. or… or… and” issues. Ericsson think this is not critical.

Not pursued.

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

Inter-band capabilities

R2-2410052	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-16	38.306	16.18.0	1186	3	F	NR_Mob_enh-Core	R2-2409289

R2-2410053	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-17	38.306	17.10.0	1187	4	A	NR_Mob_enh-Core, NR_NTN_solutions-Core	R2-2409290

R2-2410054	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1188	4	A	NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core	R2-2409291

-	MediaTek think that the R18 CR doesnt mention the R17 fields on the cover page. Qualcomm indicates that for some Rel-17 features it was not possible to have inter-band procedures. Huawei think that conditions are incomplete or inconsistent "condHandover".

Attempt to fix the CRs offline

[AT128][608][Maint] Inter-band handover capabilities (Qualcomm)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411028, R2-2411029, R2-2411030

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411028	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-16	38.306	16.18.0	1186	4	F	NR_Mob_enh-Core	R2-2410052

R2-2411029	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-17	38.306	17.10.0	1187	5	A	NR_Mob_enh-Core, NR_NTN_solutions-Core	R2-2410053

R2-2411030	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1188	5	A	NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core	R2-2410054

The three above are agreed

Max data rate

R2-2410666	Correction to maximum data rate calculation	Nokia	CR	Rel-15	38.306	15.26.0	1213	-	F	NR_newRAT-Core

R2-2410667	Correction to maximum data rate calculation	Nokia	CR	Rel-16	38.306	16.18.0	1214	-	A	NR_newRAT-Core

R2-2410668	Correction to maximum data rate calculation	Nokia	CR	Rel-17	38.306	17.10.0	1215	-	A	NR_newRAT-Core, NR_DL1024QAM_FR1-Core

R2-2410669	Correction to maximum data rate calculation	Nokia	CR	Rel-18	38.306	18.3.0	1216	-	A	NR_newRAT-Core, NR_DL1024QAM_FR1-Core

-	Huawei think that the field descriptions for the referred fields already clarify what it means it he fields are not present, this CR on its own is not correct. MediaTek and OPPO agrees. Nokia is concerned about that the section for the data rate calculation should refer to the fields even though the field descriptions explain the fields. Ericsson agrees with Huawei and others.

Not pursued.

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 4 Tdocs

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

msg3-TransformPrecoding

R2-2409515	LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH (R1-2409301; contact: CATT)	RAN1	LS in	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core	To:RAN2

Alternatives listed in the LS:

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is

msg3-transformPrecoder included in the same AdditionalRACH-Config.

R2-2409602	Contact company’s input on RAN1 LS in R2-2409515	CATT	discussion	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

Proposal 1a: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-TRanformPrecoder to apply for “MsgA PUSCH transmission when msgA-TransformPrecoder is not configured”.

Proposal 1b: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-tranformPrecoder to apply for “PUSCH scheduled by fallbackRAR UL grant”.

Proposal 2a: Add the intended UE behavior in Proposal 1a/1b into the field descriptions of msgA-transformPrecoder/msg3-transformPrecoder.

Proposal 2b: Agree the CRs in [3][4] for TS 38.331.

Proposal 3: RAN2 approves the LS response to RAN1 with below answers provided:

	For Q1: inform RAN1 that from RAN2 perspective Alt.2 reflects the intended UE behavior on which msg3-transformPrecoder to apply, both for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured.

	For Q2: inform RAN1 that RAN2 agrees the RRC CRs to capture the intended UE behavior in the field descriptions.

-	Ericsson agrees with Alt2 but think that we should only reply this to RAN1 and RAN1 will tell us if/when we should do something. LG want to only reply that RAN2 understanding is Alt2. CATT think we need to update RRC. Vivo agrees with CATT that we should update the specs now. Qualcomm does not want to do changes to the RAN2 specs. Ericsson highlights that the RAN1 LS indicates that the discussion is still ongoing in RAN1.

Reply to RAN1 as follows:

For Q1: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-TRanformPrecoder to apply for “MsgA PUSCH transmission when msgA-TransformPrecoder is not configured” and for "PUSCH transmission scheduled by fallback RAR UL grant".

For Q2: Reply that current RAN2 specification already suggests Alt2. RAN2 does not preclude doing clarifications later if deemed necessary.

[AT128][609][Maint] Reply LS on waveform determination (CATT)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2411031

Deadline:

    - Wednesday 17:00. The intention is to approve the LS over email.

R2-2411031	LS response on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH	RAN2

Approved

R2-2409603	CR on the usage of msg3-transmitPrecoder	CATT	CR	Rel-17	38.331	17.10.0	5099	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2409604	CR on the usage of msg3-transmitPrecoder	CATT	CR	Rel-18	38.331	18.3.0	5100	-	A	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2410874	Discussion on RAN1 LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH	Ericsson	discussion	Rel-17	NR_cov_enh-Core

Moved from 6.1

R2-2410818	Discussion on RAN1 LS on waveform determination	LG Electronics Inc.	discussion	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

Moved from 6.1.1.1

#### 6.1.1.0	In-principle agreed CRs

R2-2409600	Correction on location based measurements in NR NTN	CATT, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.10.0	0909	1	F	NR_NTN_solutions-Core	R2-2408366

R2-2409601	Correction on location based measurements in NR NTN	CATT, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.300	18.3.0	0910	1	A	NR_NTN_solutions-Core	R2-2408367

R2-2409806	Clarification on MII usage	Samsung, Nokia	CR	Rel-17	38.300	17.10.0	0924	1	F	NR_MBS-Core	R2-2409276

R2-2409807	Clarification on MII usage	Samsung, Nokia	CR	Rel-18	38.300	18.3.0	0925	1	A	NR_MBS-Core	R2-2409277

All 4 agreed

#### 6.1.1.1	Other

QoE

R2-2410131	Miscellaneous corrections on stage 2 description of QMC	Lenovo	CR	Rel-17	38.300	17.10.0	0930	-	F	NR_QoE-Core

-	Ericsson has a bunch of minor comments.

To be updated offline

[AT128][610][Maint] Misc QoE corrections (Lenovo)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2411032

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411032	Miscellaneous corrections on stage 2 description of QMC	Lenovo	CR	Rel-17	38.300	17.10.0	0930	-	F	NR_QoE-Core	R2-2410131

Agreed

FeMIMO

R2-2410195	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	-	F	NR_FeMIMO-Core

R2-2410196	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	-	A	NR_FeMIMO-Core

R2-2410511	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-17	38.321	17.10.0	2005	-	F	NR_FeMIMO-Core

R2-2410512	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	2006	-	A	NR_FeMIMO-Core

The 4 above were already treated in the main session

Misc

R2-2409999	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-17	38.331	17.10.0	5082	1	F	NR_newRAT-Core, TEI17	R2-2409124

To be further checked in post meeting email disc [POST128][004]

### 6.1.3	Control Plane corrections

#### 6.1.3.0	In-principle agreed CRs

R2-2409808	Clarification on determining about to start sessions	Samsung, Nokia	CR	Rel-17	38.331	17.10.0	5092	1	F	NR_MBS-Core	R2-2409278

R2-2409809	Clarification on determining about to start sessions	Samsung, Nokia	CR	Rel-18	38.331	18.3.0	5093	1	A	NR_MBS-Core	R2-2409279

R2-2409914	Correction to unified TCI signalling	MediaTek Inc.	CR	Rel-17	38.331	17.10.0	4997	1	F	NR_FeMIMO-Core	R2-2408323

R2-2409915	Correction to unified TCI signalling	MediaTek Inc.	CR	Rel-18	38.331	18.3.0	4998	1	A	NR_FeMIMO-Core	R2-2408324

R2-2409919	Correction on UE behavior of setting failedPSCellId	NTTDOCOMO, INC.	CR	Rel-17	38.331	17.10.0	5009	1	F	NR_ENDC_SON_MDT_enh-Core	R2-2408440

R2-2409920	Correction on UE behavior of setting failedPSCellId	NTTDOCOMO, INC.	CR	Rel-18	38.331	18.3.0	5010	1	A	NR_ENDC_SON_MDT_enh-Core	R2-2408441

R2-2410417	Corrections to NR NTN (R17)	Huawei, HiSilicon, CMCC, Sequans Communications	CR	Rel-17	38.331	17.10.0	5075	2	F	NR_NTN_solutions-Core	R2-2409238

R2-2410418	Corrections to NR NTN (R18)	Huawei, HiSilicon, CMCC, Sequans Communications	CR	Rel-18	38.331	18.3.0	5076	2	A	NR_NTN_solutions-Core	R2-2409240

R2-2410533	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek	CR	Rel-17	38.331	17.10.0	4899	2	F	NR_redcap-Core, NR_slice-Core	R2-2408730

R2-2410534	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek	CR	Rel-18	38.331	18.3.0	4900	2	A	NR_redcap-Core, NR_redcap_enh-Core, NR_slice-Core	R2-2408731

R2-2410650	Correction on IE perRAInfoList for SCGFailureInformation	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5044	2	F	NR_ENDC_SON_MDT_enh-Core	R2-2409298

R2-2410651	Correction on IE perRAInfoList for SCGFailureInformation	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5045	2	A	NR_ENDC_SON_MDT_enh-Core	R2-2409299

R2-2410835	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5014	2	F	NR_FeMIMO-Core	R2-2409309

R2-2410836	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5015	2	A	NR_FeMIMO-Core	R2-2409310

R2-2410837	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-17	38.306	17.10.0	1175	2	F	NR_FeMIMO-Core	R2-2409311

R2-2410909	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1176	3	A	NR_FeMIMO-Core	R2-2410838

All above are agreed

Updated compared to the IPA

R2-2411037	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	3	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410917

R2-2411038	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5025	2	A	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410361

R2-2410893	Clarification of MeasurementTimingConfiguration use	Vodafone,Ericsson,Nokia	CR	Rel-18	38.331	18.3.0	5016	2	F	NR_BWP_wor-Core	R2-2409932

The 3 above are agreed

Old revisions

R2-2410360	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	1	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2408651

R2-2410838	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1176	2	A	NR_FeMIMO-Core	R2-2409312

R2-2409932	Clarification of MeasurementTimingConfiguration use	Vodafone,Ericsson,Nokia	CR	Rel-18	38.331	18.3.0	5016	1	F	NR_BWP_wor-Core	R2-2408506

Moved from 6.1.3

R2-2410917	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	2	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410360

R2-2410361	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5025	1	A	NR_NTN_solutions-Core, NR_redcap-Core	R2-2408652

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

Max nrof SI messages

R2-2409662	Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-17)	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-17	38.331	17.10.0	5107	-	F	TEI17

R2-2409663	Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-18)	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-18	38.331	18.3.0	5108	-	A	TEI18

-	Lenovo think the change is not complete because it only applies to Msg1-based but should apply also to Msg3-based SI request and this needs to be fixed. Docomo think that if we remove “when posSI-RequestConfig or posSI-RequestConfigRedCap or posSI-RequestConfigSUL is configured”, which is suggested by Lenovo then the whole field description may need updating, and also other FDs need updating.

We will update the CRs offline

[AT128][611][Maint] Description of si-broadcastStatus (Xiaomi)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411033 and R2-2411034

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411033	Correction of maxSI messages [SI-SCHEDULING]	Xiaomi

R2-2411034 	Correction of maxSI messages [SI-SCHEDULING]	Xiaomi

Both are agreed

R2-2410253	Maximum number of SI messages	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

Noted

IDC

R2-2409664	Clarification for IDC solutions of logged MDT (Rel-17)	Xiaomi, CMCC	CR	Rel-17	38.331	17.10.0	5109	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2409665	Clarification for IDC solutions of logged MDT (Rel-18)	Xiaomi, CMCC	CR	Rel-18	38.331	18.3.0	5110	-	A	NR_ENDC_SON_MDT_enh-Core

-	LG agrees with the intention but want to change in procedural text rather than capturing this with a NOTE. Huawei think this CR is an optimization and no change is needed. Qualcomm agrees with Huawei and think this is not testable and do not see a need for this. Samsung think this can be left to UE implementation and there is no need to overspecify here

Postponed

CPA

R2-2409775	Correction on the pre-condition for conditional reconfiguration for CPA	vivo	CR	Rel-17	38.331	17.10.0	5115	-	F	LTE_NR_DC_enh2-Core

R2-2409776	Correction on the pre-condition for conditional reconfiguration for CPA, S-CPAC and LTM configuration	vivo	CR	Rel-18	38.331	18.3.0	5116	-	F	LTE_NR_DC_enh2-Core, NR_Mob_enh2-Core

-	Apple thinks that the CR suggests that CPA can only be added if there is already an SCG,but that is not correct. Huawei disagrees with all changes. Ericsson think that there is text in the same section which clarifies that security needs to be activated (“the addition of Secondary Cell Group and SCells is performed only when AS security has been activated”). Qualcomm think the changes are not essential.

Not pursued, can be revisited if essential issues are found.

SON/MDT

R2-2410043	Correction on handling successHO-Config during full configuration	Samsung, Ericsson	CR	Rel-17	38.331	17.10.0	5136	-	F	NR_ENDC_SON_MDT_enh-Core

Agreed

R2-2410157	Correction for RA resource information in RA-report	Sharp, Ericsson, Lenovo	CR	Rel-17	38.331	17.10.0	5144	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2410162	Correction for RA resource information in RA-report	Sharp, Ericsson, Lenovo	CR	Rel-18	38.331	18.3.0	5145	-	A	NR_ENDC_SON_MDT_enh2-Core

Both above are agreed

R2-2410767	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson	CR	Rel-17	38.331	17.10.0	5186	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2410775	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.3.0	5188	-	A	NR_ENDC_SON_MDT_enh-Core

-	Samsung indicates that after offline discussion there is need to update the wording for example “current PCell” can be different than the current camped cell, so some updates are needed. LG also want to discuss more.

[AT128][601][SONMDT] Correction on PSCell mobility history information (Samsung)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411012 and R2-2411013

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411012	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-18	38.331	18.3.0	5188	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2411013	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-18	38.331	18.3.0	5188	1	A	NR_ENDC_SON_MDT_enh-Core

Both above are agreed

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

Max nrof segments

R2-2409749	Network signalling of maximum number of UL segments - RRC processing delay	Qualcomm Incorporated	discussion	Rel-17	TEI17

Proposal:	Specify 640ms as the RRC processing delay requirement for the case UECapabilityEnquiry message includes the field rrc-MaxCapaSegAllowed.

R2-2410348	Discussion on segmented UE capability	Huawei, HiSilicon	discussion	Rel-17	TEI17

Proposal 1: RAN2 confirms that the RRC processing delay for UE capability refers to the time from when the UE receives the Capability Enquiry message until it is ready to receive the grant to transmit the first segment.

Proposal 2: The mechanism on network indication of maximum number of UL segments applies to both NR and LTE.

Proposal 3: RAN2 considers to extend the processing time to 800ms for segmented UE capability transfer procedure.

Proposal 4: The proposed extended processing delay is introduced for legacy capability segmentation mechanism for NR and LTE from Release-16.

On P2:

-	MediaTek wants it both for LTE and NR. Apple wants this from Rel-17. MediaTek wants from Rel-16. Qualcomm wants Rel-17.

On P3:

-	Apple is OK with 800 ms. MediaTek wants 1280 ms since this is 16*80. ZTE is concerned by this large number and think that it is not very rare that the UE sends UE capabilities and based on feedback from their product team it seems that it happens rather frequent that the UE needs to send UE capabilities, and it happens that CN releases UE capabilities and this happens in terms of every 30 minutes. Qualcomm understand that the NW vendors want a low number but also the issue is due to NW problems and we need to find a middle ground. Nokia think that we should define a fixed value.

On P4:

-	MediaTek agrees to extend. Samsung also want to extend the existing number. Ericsson think this is NBC. Qualcomm think we can discuss the legacy part separately. Nokia agrees with Qualcomm.

Agreements - The mechanism on network indication of maximum number of UL segments applies to both NR and LTE for Rel-17. - We can discuss if the legacy processing time should be extended later, based on company contributions

[AT128][612][Maint] UE processing delay time for max nrof segments (Qualcomm)

Scope:

    - Come up with a number for the UE processing delay .

Intended outcome:

    - Decided on an agreeable UE processing delay number

Deadline:

    - In time for the Thursday session

-	Qualcomm reports from the offline that the below agreements are likely agreeable and we can have a post meeting email disc to finalize the CRs.

Agreements - RRC processing delay requirement is defined as follows: - 560 + max(0, n-7)*80 [ms], where n is the number of segments. - For both LTE and NR, CRs are from Rel-17, and made early implementable from Rel-16. - Decouple Rel-16 UL segmentation procedure and the new Rel-17 segmentation procedure, i.e. it is possible for the UE to indicate the support of Rel-17 procedure without supporting Rel-16 procedure. - Add “16” as one of the values for the network-requested number of segments.

[POST128][601][Maint] CRs for maximum number of UL segments (Qualcomm)

Scope:

    - Produce agreeable CR for NR and LTE

Intended outcome:

    - Agreed CRs

Deadline:

    - Short

R2-2409750	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.331	17.10.0	5004	2	B	TEI17	R2-2409300

R2-2409751	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.331	18.3.0	5113	-	A	TEI17

R2-2409752	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.306	17.10.0	1202	-	B	TEI17	R2-2409301

R2-2409753	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.306	18.3.0	1203	-	A	TEI17

R2-2409862	RRC processing time for NW signalled UL segments	Samsung	discussion	Rel-17	TEI17

Proposal 1: RAN2 agree to increase RRC processing delay for NW requested UL segment feature.

Proposal 2: RAN2 agree that the total RRC processing delay for UE capability enquiry with the new NW requested UL segment is 640msec.

Proposal 3: RAN2 agree to support 16 UL segments in the new NW requested UL segment feature.

Enhanced channel raster

R2-2409992	Mandatory support of Enhanced channel raster by RedCap UE	Ericsson	CR	Rel-17	38.306	17.10.0	1157	2	F	NR_redcap-Core	R2-2409295

Agreed

R2-2410905	Mandatory support of Enhanced channel raster by (e)RedCap UE	Ericsson	CR	Rel-18	38.306	18.3.0	1158	3	F	NR_redcap-Core	R2-2409993

Agreed unseen in R2-2411040, fix meeting number and clarify the “from Rel-18” change clearer on the cover page.

Old revisions

R2-2409993	Mandatory support of Enhanced channel raster by (e)RedCap UE	Ericsson	CR	Rel-18	38.306	18.3.0	1158	2	F	NR_redcap-Core	R2-2409296

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

Emergency

R2-2410772	On eDRX Operation with Emergency Services	ZTE Corporation	discussion	Rel-17	NR_redcap-Core

Proposal 1: To capture in specification that UE shall not use INACTIVE eDRX if UE has PDU session associated with emergency services.

Proposal 2: RAN2 to adopt the text proposals provided in Annex A and B for TS 38.304 Rel-17 and Rel-18 respectively.

Proposal 3: RAN2 to decide whether a standard solution is needed to get RAN aware of whether UE has PDU session associated with emergency services, or leave to network implementation to prevent RAN from using eDRX for the UE.

-	Nokia think that the NW is not aware of emergency services today but it is needed and hence SA2 would need to be contacted. Huawei also want to send an LS to SA2, SA2 needs to decide when the CN should indicate to the RAN about ongoing emergency services. Qualcomm also wants to send an LS and think that if the RAN gets the info from the CN the RAN can configure the UE properly and no autonomous UE action is needed. Nokia think that if the RAN is aware of emerngegy services it can avoid configuring eDRX. ZTE agrees to send an LS but does not want to ask SA2 if CN can indicate to the RAN about emergency services but also want to ask if CN can deconfigure IDLE eDRX and that solves everything. Samsung wants to send an LS but are not sure if IDLE eDRX should really be released by CN when emergency services is ongoing. CATT thinks that this is a NW-issue and could be left to NW implementation or if a standard solution is needed it should be handled by RAN3/SA2. Qualcomm wonders why releasing IDLE eDRX would help. ZTE wants to ask open questions to SA2 about if/how RAN needs to be aware of ongoing emergency services. Samsung are OK to ask open questions. Nokia think that we can just say that based on RAN2 undersatnding the gNB is not aware of emergencv and then highlight the issue.

-	Ericsson think that ARP values can be used by the CN to indicate to the RAN that emergency is ongoing. Ericsson are unsure what is the intention of the LS would be, instead Ericsson prefers to leave this whole problem to NW implementation.

Send an LS to SA2 and CT1, cc RAN3, explaining the RAN2 understanding of the issue and see if and how SA2 think this should be addressed.

R2-2410557	Correction on emergency call back when eDRX is configured in RRC_INACTIVE	Nokia	CR	Rel-17	38.304	17.9.0	0423	-	F	NR_redcap-Core

R2-2410558	Correction on emergency call back when eDRX is configured in RRC_INACTIVE	Nokia	CR	Rel-18	38.304	18.3.0	0424	-	A	NR_redcap-Core

Both are noted

[POST128][602][Maint] LS on emergency services and eDRX (ZTE)

Scope:

    - Produce approvable LS to SA2 and CT1, cc RAN3 as per the agreement

Intended outcome:

    - Approved LS in R2-2411041 (ZTE)

Deadline:

    - Short

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

R2-2409647	Corrections on SPR configuration release and logging PSCell identity	CATT	CR	Rel-18	38.331	18.3.0	5105	-	F	NR_ENDC_SON_MDT_enh2-Core

On first change:

-	Samsung thinks this reflects an earlier agreement and sees no issue with current spec. Ericsson think the spec is not broken but are OK to do the change to make the spec clearer. Huawei think this change is not needed, it is OK that there is a duplication. LG agrees with the other companies that nothing is broken and want to leave the spec as is. Nokia agrees with LG.

On the second change:

-	Samsung think this is a new scenario and it is too late now. CATT think that the field description (shown on the cover page) dictates that we should do this change. Ericsson agrees with CATT and are OK with this change. Huawei is OK on this.

-	LG wonders if the current spec text is not intentional? Nokia think that this is a functional change and are not OK with it. Samsung think it is correct but its too late and this is not an essential correction. CATT think that a previous RAN3 LS suggests that the behaviour is as described by this CR. ZTE agrees with CATT that we should do this based on what RAN3 suggested earlier. Samsung need more time to check this.

The first change (release of successPSCell-Config) is not pursued as RAN2 understands that this duplicating of release is not causing any issues.

Discuss the second change (setting of the pSCellId) offline.

[AT128][602][SONMDT] Corrections on SPR configuration (CATT)

Scope:

    - Produce agreeable CRs if possible

Intended outcome:

    - Agreed CR R2-2411036 (Company)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

    - In time for the Thursday session

R2-2411036	Correction on logging PSCell identity CATT CR Rel-18 38.331 18.3.0 5105 1 F NR_ENDC_SON_MDT_enh2-Core

Agreed

R2-2409648	Corrections on RA Report retrieval	CATT	CR	Rel-18	37.340	18.3.0	0405	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung and ZTE think this text comes from RAN3 and should be discussed there if needed. Huawei does not agree with the changes and agree that RAN3 should do changes to this part of the spec if needed.

Not pursued by RAN2, RAN3 may discuss if they want

R2-2409986	NR-U related RA information in SCGFailureInformation	Nokia	CR	Rel-18	38.331	18.3.0	5126	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung are OK with the intention but the ASN.1 is wrong, it should be “v18” instead of “r18xx” and impacted arch options are missing.

-	Ericsson are OK to add perRAInfoList but not numberOfLBT-Failures since that is new behaviour.

-	ZTE think we intentionally agreed to not support this, but if we do something then only adding the perRAInfoList is enough. Nokia think the numberOfLBT-Failures is also useful and if we don’t add it we need clarifying the procedural text. ZTE are open to clarify in the procedural text.

We aim to add perRAInfoList to ASN.1 and update the procedural text to clarify the behaviour for numberOfLBT-Failures. Also add impacted arch options and fix ASN.1 (v18). CRs to be produced offline

[AT128][603][SONMDT] NR-U related RA information in SCGFailureInformation (Nokia)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed in principle CR in R2-2411014

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411014	NR-U related RA information in SCGFailureInformation	Nokia	CR	Rel-18	38.331	18.3.0	5126	1	F	NR_ENDC_SON_MDT_enh2-Core

R2-2411165	Correction in the optionality of NR-U related information	Nokia	CR	Rel-18	38.306	18.3.0	1221	-	F	NR_ENDC_SON_MDT_enh2-Core

Both are agreed

R2-2410045	Correction on Inter-RAT SHR	Samsung	CR	Rel-18	38.331	18.3.0	5137	-	F	NR_ENDC_SON_MDT_enh2-Core

-	CATT, Ericsson and Nokia think that the last bullet 1> in 5.7.10.6 can be removed instead. Nokia think that the proposed change in this CR is a functional enhancement. Samsung wants to postpone. Ericsson also want to postpone.

Postponed

R2-2410057	Correction on logging RSSI measurements in RLF report and SHR	Samsung	CR	Rel-18	38.331	18.3.0	5138	-	F	NR_ENDC_SON_MDT_enh2-Core

NE-DC should be added as impacted architecture options. With that the CR is agreed unseen in R2-2411011.

# 8	Rel-19

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

LTM

R2-2410656	Discussion on MRO enhancements for Rel-18 mobility	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P7

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

Proposal 2: RAN2 include in the RLF report whether the UE receives the TA from the cell switch command.

Proposal 3: RAN2 reuse the existing ra-InformationCommon for the RA-based LTM failure

Proposal 4: RAN2 discuss whether to include early sync info to existing RACH information. The detailed early sync info is FFS.

Proposal 5: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 6: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

Proposal 7: Introduce a flag of whether it is RACH-less for both SHR report and RLF report.

On P1:

-	vivo think P1 is not needed since the NW will know this even without an explicit indication. Ericsson that the existing RLF report indicates if the RA is due to HOF, and RA info common and existing flags the NW knows this. Huawei asks if the last HO type is what Ericsson have in mind? Ericsson confirms. Huawei seem to agree now.

On P2:

-	CMCC think this is not needed since the UE will report if it got the TA from the NW or if it was determined by the UE. Huawei think its easier to intropduce this rather than wait for RAN3. Qualcomm think the NW-based solution should work. Samsung think we need to wait.

On P4:

-	vivo think that the previous indication is sufficient which says whether the UE or the NW gave the TA. Ericsson understands that the proposal is that the UE shall report the preambles sent due to early sync and this is not really a full RA and it is not clear what the gNB should do with this info.

-	ZTE think that the point of SONMDT is to help the NW to give more info so they are supportive in general but not sure exactly which info Huawei has in mind. Huawei are more thinking on a high level about the scenario and want to discuss further if there is anything useful. CATT think that the early sync procedure is the core part of LTM and think that the NW can use this info to optimize early sync resources (e.g. RA attempts). Ericsson does not understand what can be optimized since this early UL sync for LTM is just a one-shot preamble transmission.

On P5:

-	Samsung think that L1 measurements has already been agreed to include in the RLF report and that should be enough as they carry beam info. Nokia agrees. ZTE think that the L1 measurements are not necessarily carrying sufficient info. Vivo agrees with Samsung that L1 measurements is sufficient.

-	CATT understand that the TCI state indicated in the MAC CE should be logged. Qualcomm think that the NW should know which TCI state is indicated in the LTM MAC CE. Ericsson think that this is for the RLF report and the NW cannot know afterwards. Ericsson think that it is useful to get the TCI state. Qualcomm think that even for RLF the NW can know this. CMCC think that the NW cannot know all configurations, e.g. the TCI state after a failure. Huawei agrees with CMCC that the LTM MAC CE is not a configuration and the NW cannot remember this.

On P6:

-	Ericsson think that CHO we did a list in the RLF report where the candidates were indicated and the point was that some measurements were perhaps not available. Ericsson prefer to align with CHO. CMCC agrees with Ericsson.

On P7:

-	Ericsson think that P1 addresses this.

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

R2-2409754	Further considerations on MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2-P3

Proposal 2	RAN2 introduces a new near failure case where BFR happens soon after a successful LTM cell switch, and UE re-selects to another beam in the same target cell.

Proposal 3	Introduce a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed.

On P2:

-	vivo think that there is not need for new triggers. ZTE think that T304 can reused, but that does not exclude other failure causes. Samsung think this causes a lot of additional complexity and need more time to look in to this if we should consider it. LG also want more time.

-	Nokia think that MRO is per cell not per beam and are not fine with this proposal.

On P3:

-	LG think that the issue is different since if BFR happens then its not really a failure since the UE recovered. Qualcomm think it is a very rare case that the LTM succeeded but the beam failed. Ericsson want to keep this open and want to look more in to this.

Postponed

R2-2410065	MRO for Rel-18 mobility	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1 and P2

Proposal 1: Include the following information to be included in RLF-report and SHR:

	TA value measured by the UE

	Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

Proposal 2: Include the following information to be included in RLF-report and SHR:

	TCI state information included in the LTM cell switch command

	Beam with the TCI state information in the LTM cell switch command

R2-2409984	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P4

Proposal 1: RAN2 to log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment cell.

Proposal 3: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Proposal 4:  RAN2 to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as new SHR trigger.

CHO with candidate SCGs

R2-2409635	MRO enhancements for Rel-18 mobility features	Samsung	discussion

Focus on 2-1, 2-2, 2-3

Proposal 2-1: UE reports the time gap between the first met condition and the second met condition as follows:

a.	When at least one PCell condition is fulfilled before all the conditions for PSCell, UE logs the time gap between the first triggered event for PCell and the first triggered event for PSCell.

b.	When at least one PSCell condition is fulfilled before all the conditions for PCell, UE logs the time gap between the first triggered event for PSCell and the first triggered event for PCell.

Proposal 2-2:	Elapsed time between the first fulfilled condition and RLF is included in RLF report for all the PCells.

Proposal 2-3:	Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also.

On P2-1:

-	vivo think that we should consider the last triggered event for the PCell (or PSCell) to the last triggered event for the PSCell (or PCell). Ericsson think we have already agreed what vivo suggests. Ericsson want, in addition to this, have the first triggered PSCell condition to the last triggered PSCell condition. LG does not agree with with the Ericsson assumption.

-	Nokia think we don’t need more granularity. Ericsson is OK with not adding more granularity but points out that current spec already makes the UE log, for the CHO, the time from the first trigger for the PCell and the second trigger for the PCell and does not want to remove this possibility.

On P2-2

-	 vivo wonders if “first fulfilled condition” refers to either CHO or CPAC. Samsung wonder what is the intention of logging this, hence not supportive.

On P2-3:

-	ZTE has questions about stage-3 details. The session chair think they can be disucssed later.

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

R2-2409650	Discussion on MRO Enhancements for Rel-18 Mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P8 – P12

Proposal 8: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

Proposal 11: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

Proposal 12: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

-	Option 1: one report to include both SHR and SPR information;

-	Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

R2-2409779	MRO for MOB DCCA	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	R2-2409041

R2-2409780	MRO for MOB LTM	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2409970	SON for LTM	Apple	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2409933	Discussion on MRO enhancement for LTM	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2409934	Discussion on MRO enhancement for CHO with candidate SCGs	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2409983	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410182	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410274	Discussion on MRO for R18 mobility	Lenovo	discussion	Rel-19

R2-2410328	MRO enhancements for CHO with candidate SCGs	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410329	MRO enhancements for LTM	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410473	SON support for MRO	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2410635	SON enhancement for CHO with candidate SCG	SHARP Corporation	discussion

R2-2410733	MRO enhancement for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2410815	MRO for Rel-18 mobility features	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.3	SON/MDT for Slicing

No contributions are expected and this AI will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

### 8.10.4	SON/MDT for NTN

No contributions are expected and this AI will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

SDT

R2-2409649	Considerations on R18 leftovers for SDT	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: In the case of UE falling back from RA-SDT, UE logs the DL RSRP and UL data volume for the failed triggering of RA-SDT in RA report.

Proposal 2: The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

⁻	unsuccessfully completed upon cell re-selection.

Proposal 3: To introduce an indication to differentiate whether it’s CG-SDT failure or RA-SDT failure in RA report, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters need to be adjusted.

On P1:

-	Ericsson agrees and think this is implicit from earlier agreements. Nokia does not understand why RSRP is needed to be logged. Ericsson think that the NW can adjust the threshold based on the UE reported measured RSRP. Nokia are not convinced that it is useful since the RSRP threshold cannot be adjusted based on the UE reported values.

On P2:

-	vivo are not sure about “unsuccessfully completed upon cell re-selection.” and wonders whats the use to the NW? Samsung think this cause is not useful and want to have it as FFS. Ericsson think that the spec and work becomes more complex if we cherry-pick, its better that the UE simply logs the reason for the failure.

On P3:

-	Qualcomm think that the NW should know this since it is configured by the NW. Ericsson think that the NW cannot know that the UE tried by failed to initiate CG-SDT, or if it fell back from CG-SDT to RA-SDT. Qualcomm think that CG-SDT is not in scope. Nokia agrees with Qualcomm

Agreements - The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. - The following failure causes can be logged for failed SDT in RA report: ⁻ upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; ⁻ upon random access problem indication is received from MCG MAC while SDT procedure is ongoing; ⁻ upon T319a expires; ⁻ upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; ⁻ FFS: unsuccessfully completed upon cell re-selection.

R2-2409655	Reporting failure cause for SDT	Samsung	discussion

R2-2409755	Consideration on RAN3 agreed RA report content	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410275	Discussion on RACH optimization for SDT	Lenovo	discussion	Rel-19

R2-2410657	Discussion on support of the Rel-18 leftovers	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410816	RACH optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410847	On Rel.18 leftovers	Ericsson	discussion

# Summary

NBC CRs:

None

Post meeting email discussions:

[POST128][601][Maint] CRs for maximum number of UL segments (Qualcomm)

Scope:

    - Produce agreeable CR for NR and LTE

Intended outcome:

    - Agreed CRs

Deadline:

    - Short

[POST128][602][Maint] LS on emergency services and eDRX (ZTE)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2411041 (ZTE)

Deadline:

    - Short